HEX       HEX   DEC               DEC

0000      0C95   3221 PROMIDA      $0C95        EPROM I.D. NUMBER        DATE CODE

0002      FFFF  65535 DATECODE     $FFFF        EPROM DATE CODE

0004      FFFF  65535 SEQNUMB      $FFFF        PRODUCTION SEQUENCE NUMBER

0006      4A74  19060 KKSUM        $4A74        CHECK SUM $8008 THROUGH $FFFF

0008        A1    161 KKPGMID        $A1        PROGRAM ID WORD - $AA = BYPASS CHECKSUM  VALID ID FOR 1991

2.8/3.1L IS   $A1

0009        C0    192 KNUMCYL          6 CYLS         NUMBER OF CYLINDERS IN THE ENGINE

000A      0889   2185 KRPMUP         600 RPM      # OF 65 KHZ COUNTS = STARTUP RPM-SPARK

000C      0B61   2913 KFUELUP        450 RPM      # OF 65 KHZ COUNTS = STARTUP RPM-FUEL

000E        3C     60 KREDLINE           6000 RPM    5500 = 55dec = $37

000F      0110    272 KSHDNTM        3.4 SEC         ECM SHUT DOWN TIME  (Sec * 80 = dec)

0011      0FFF   4095 KSAFILT2    1999.5 %/SEC  RATE OF CRANK TO RUN SPK. ADV. FILTERING

ALSO RATE OF LO RPM/HI MAP SPK  BYPASS EXIT FILTERING

0013        1A     26 KINITSPK         9 DEG              INITIAL SPARK ADVANCE

0014      0100    256 KSPKBLND       125 %/SEC  RATE OF SPK FILTERING WHEN DFCO SPARK EXIT

0016        1E     30 KDFCOSPK      10.5 DEG       DEFAULT SPARK ADVANCE FOR DFCO SPARK

0017        28     40 KESTRPME      1000 RPM       RPM THRESH TO ENABLE LO RPM/HI MAP EST BYPASS

0018        10     16 KESTRPMD       400 RPM        LOW RPM THRESH TO DISABLE EST (WITH HI MAP)

0019        A2    162 KMAPSTAL        70 KPA         HIGH MAP THRESH TO DISABLE EST (WITH LO RPM)

001A        40     64 KSFILRPM       800 RPM         RPM THRESHOLD TO STOP SPK. ADV. FILTERING

001B        AA    170 KREFANGL      59.8 DEG      SPARK REFERENCE ANGLE

001C        FF    255 KRPMXHI       6375 RPM       F1 TABLE EXTENSION HI RPM BREAKPOINT

001D        00      0 KADVSLHI         0 DEG/K         (DEG/THOUSAND RPM) HI RPM ADVANCE SLOPE

001E        F0    240 KFILTMPH     0.938 COEF     LAG FILTER COEFFICIENT FOR NMPH, N.D. (0- 1)

001F        A0    160 KFILTRPM     0.625 COEF     RPM FILTER TIME CONSTANT, N.D. (0-1)

0020        32     50 KFTES125     0.195 COEF       RPM FILTER TIME CONSTANT, N.D. (0-1)

0021      000A     10 KTIMELAG       153 USEC      TIME DOMAIN CORRECTION TO SPARK

0023      FFE4  65508 KMAXADV2       -10 DEG    MAX ADVANCE RELATIVE TO REFERENCE (2'S COMP)

0025      FF39  65337 KMAXRTD2       -70 DEG    MAX RETARD RELATIVE TO REFERENCE (2'S COMPL)

0027        E0    224 KTIMOUT        2.8 SEC           RE-CRANK NO START TIME VALUE

0028        04      4 KERUNCTR         4                    # OF SUCC. LOW REFPERS TO ALLOW ENG. RUN- SPARK

0029        04      4 KRUNFCTR         4                    # OF SUCC. LOW REFPERS TO ALLOW ENG. RUN-FUEL

002A        06      6 KIDLTPS1       2.3 %                  LOW TPS THRESHOLD, EGR OFF

002B        04      4 KIDLTPS2       1.6 %           LOW TPS THRESHOLD, EGR ON

002C        10     16 KF4CNTR        0.2 SEC     F4 FLAG THROTTLE OPENING DELAY

002D        00      0 KF4TCTH        -40 DEG C  TEMP ABOVE WHICH EGR TIP-IN DELAY USED

002E        FF    255 KPSTEMP        151 DEG C  COOLANT TEMP. THRESHOLD FOR P.S. SPARK

# BARO UPDATE

002F        E8    232 KADBARO         96 KPA    * 243 = 100 KPA - - DEFAULT BARO A/DCOUNTS

0030        A0    160 KBARSPDA      4000 RPM    BARO UPDATE RPM THRESHHOLD

0031        02      2 KBAROTPS       0.8 %           BARO CAN BE UPDATED IF POS. DELTA < THIS

0032        08      8 KBAROTM        0.2 SEC       BARO UPDATE RATE

0033        04      4 KIDLTPSL       1.6 % TPS  THROT POS BELOW WHICH CAUSES IDLE SPARK

FORMS HYSTERISIS PAIR WITH KIDLTPSH

0034        03      3 KIDLMPHL         3 MPH    MPH BELOW WHICH CAUSES IDLE SPARK

FORMS HYSTERISIS PAIR WITH KIDLMPHH

0035        05      5 KIDLTPSH         2 % TPS  THROT POS ABOVE WHICH DISABLES IDLE SPARK

FORMS HYSTERISIS PAIR  KIDLTPSL

0036        03      3 KIDLMPHH         3 MPH    MPH ABOVE WHICH DISABLES IDLE SPARK

FORMS HYSTERISIS PAIR WITH KIDLMPHL

0037        00      0 KDCLRPML         0 RPM    ENG SPEED BELOW WHICH CAN BE DECLUTCHED

LOW HYSTERISIS VALUE, WITH  KDCLRPMH

0038        04      4 KDCLTPSL       1.6 % TPS  THROT POS BELOW WHICH CAN BE DECLUTCHED

LOW HYSTERISIS VALUE, WITH KDCLTPSH

0039        FF    255 KDCLMPHH       255 MPH    MPH ABOVE WHICH CAN BE DECLUTCHED

HIGH HYSTERISIS VALUE, WITH KDCLMPHL

003A        FF    255 KDCLMAPH     104.4 KPA    MAP ABOVE WHICH CAN BE DECLUTCHED

HIGH HYSTERISIS VALUE, WITH KDCLMAPL

003B        00      0 KDCLMPH       10.3 KPA    MAP BELOW WHICH CAN BE DECLUTCHED

HIGH HYSTERISIS VALUE, WITH KDCLMPL

003C        00      0 KDCLRPMH         0 RPM    ENG SPEED BELOW WHICH CAN BE DECLUTCHED

HIGH HYSTERISIS VALUE, WITH KDCLRPML

003D        08      8 KDCLTPSH         3 % TPS  THROT POS BELOW WHICH CAN BE DECLUTCHED

HIGH HYSTERISIS VALUE, WITH KDCLTPSL

003E        FF    255 KDCLMPHL       255 MPH    MPH ABOVE WHICH CAN BE DECLUTCHED

LOW HYSTERISIS VALUE, WITH KDCLMPHH

003F        FF    255 KDCLMAPL     104.4 KPA    MAP ABOVE WHICH CAN BE DECLUTCHED

LOW HYSTERISIS VALUE, WITH KDCLMAPH

0040        00      0 KDCLMPL       10.3 KPA    MAP BELOW WHICH CAN BE DECLUTCHED

LOW HYSTERISIS VALUE, WITH KDCLMPH

0041        5A     90 KWOTACTM         9 SEC    WIDE OPEN THROTTLE A/ C DISABLE TIME

0042        28     40 KNVACTM          4 SEC       WIDE OPEN THROTTLE A/ C DISABLE TIME WITH HI N/V

0043        80    128 KNVACDIS       128 N/V     WIDE OPEN THROTTLE A/ C DISABLE N/V THR.

0044        96    150 KNVDLTAC       150 N/V     DELTA TPS A/C DISABLE N/V DISABLE THR.

0045        F6    246 KPIDACDT        96 % TPS  IF TPS > THIS DISABLE A/C; (TPS MUST BE <= KPIDACDT - 16

TO RE-ENABLE)

0046        18     24 KACDISTH       9.4 % TPS  IF POS. DELTA TPS >THIS, DISABLE A/C

0047        20     32 KACDISDL       3.2 SEC    DISABLE A/C CLUTCH FOR THIS TIME AFTER HIGH POSITIVE DELTA

TPS

0048        01      1 KACONDLY       0.1 SEC    A/C DELAY TIME BEFORE ENGAGING THE CLUTCH

0049        00      0 KACOFDLY         0 SEC     A/C DELAY TIME BEFORE DISENGAGING CLUTCH

004A        D7    215 KCLTACMX       121 DEG C  IF COOLANT > THIS DISENGAGE CLUTCH

004B        00      0 KDSACOFT         0 SEC    DOWN-SHIFT A/C OFF TIME

004C        B6    182 KV5RPM        4550 RPM    V5 COMPRESSOR DISABLE RPM THRESHOLD

004D        14     20 KV5TIME          2 SEC    V5 RE-ENABLE TIME THRESHOLD

004E        30     48 KACDELAY       4.8 SEC    INITIAL (ONE TIME) A/ C DELAY

004F        1D     29 KACLPDL         29 CTS    A/C LO PRESSURE CYCLING THRESH, LO HYST

0050        22     34 KACLPDH         34 CTS    A/C LO PRESSURE CYCLING THRESH, HI HYST

0051        F3    243 KACHPDH        243 CTS    A/C HI PRESSURE CYCLING THRESH, HI HYST

0052        79    121 KACHPDL        121 CTS    A/C HI PRESSURE CYCLING THRESH, LO HYST

AC Fan ON/OFF

0053        74    116 KACFANPH       116 CTS    HI A/C PRESSURE FAN1 ENABLE THRESH, HI HYST

0054        33     51 KACFANPL        51 CTS    HI A/C PRESSURE FAN1 ENABLE THRESH, LO HYST

0055        74    116 KACFNPH2       116 CTS    HI A/C PRESSURE FAN2 ENABLE THRESH, HI HYST

0056        33     51 KACFNPL2        51 CTS    HI A/C PRESSURE FAN2 ENABLE THRESH, LO HYST

0057        80    128 KSPDDIV          6         'DIVISOR'    IP PULSE DIVISOR: THE FREQUENCY OF THE SIGNAL FROM

THE MAGNETIC SENSOR DIVIDED BY THIS VALUE AND THE RESULTING

SIGNAL IS SENT OUT OF THE ECM FOR USE BY  THE INSTRUMENT PANEL.

0058        08      8 KVEHMOVE       0.8 SEC    SECONDS BEFORE VEHICLE NOT MOVING BIT SET

0059      0133    307 KSPDSEN      24016        PU/MI  ROAD SPEED SENSOR CONSTANT

005B        00      0 KEGRBIAS         0 DEG       BIAS FOR EGR ADV CORR (TABLE F4)

005C        64    100 KCTBIAS         35 DEG      BIAS FOR BASE COOL ADV CORR (TABLE F2)

005D        00      0 KTCCBIAS         0 DEG       BIAS FOR TCC LOCKED SPARK RETARD (F8)

005E        64    100 KSTIBIAS        35 DEG      BIAS FOR INITIAL SPARK TIMEOUT (F81)

ESC Enable2

005F        24     36 KRPMKNOB       450 RPM    IF FILTERED RPM >= THIS ENABLE ESC

0060        01      1 KMPHKNOB         1 MPH       IF FILTERED MPH >= THIS ENABLE ESC

0061        6E    110 KVOLKNOB        11 VOLTS  DISAB ESC IF ADBAT < THIS & AT IDLE

0062        8E    142 KESCOOLA      66.5 DEG C   IF COOLANT < THIS DISABLE ESC

0063        40     64 KESCDCOL        48 DEG C     IF DELTA COOLANT SINCE STARTUP > THIS, ENABLE ESC

## BARO UPDATE TABLE

#************************************************

#  F59   OFFSET TO MAP A/D READING FOR BARO     *

#        ADJUSTMENT VS. RPM AND TPS             *

#        TABLE VALUE = KPA * 2.71               *

#************************************************

* -------------------------------------------------------------------

0064        10     16 F59A            16        R MIN:  R = RPM (NTRPMX)

0065        30     48                 48        Q MIN:  Q = NTPSLD

0066        06      6                  6        R NUM   (table length)

* --------------------------------------------- SPEED       1200 RPM

0067        08      8                  3 KPA     37.5      %-THROTPOS

0068        03      3                  1 KPA     50.0

0069        00      0                  0 KPA     62.5                             6X table

006A        00      0                  0 KPA     75.0

006B        00      0                  0 KPA     87.5

006C        00      0                  0 KPA    100.0

* --------------------------------------------- SPEED       2400 RPM

006D        19     25                9.2 KPA     37.5      %-THROTPOS

006E        0B     11                  4 KPA     50.0

006F        06      6                2.2 KPA     62.5

0070        04      4                1.5 KPA     75.0

0071        03      3                  1 KPA     87.5

0072        02      2                0.7 KPA    100.0

* --------------------------------------------- SPEED       3600 RPM

0073        38     56               20.7 KPA     37.5      %-THROTPOS

0074        1F     31               11.4 KPA     50.0

0075        16     22                  8 KPA     62.5

0076        0F     15                5.5 KPA     75.0

0077        0B     11                  4 KPA     87.5

0078        09      9                3.3 KPA    100.0

* --------------------------------------------- SPEED       4800 RPM

0079        4A     74               27.3 KPA     37.5      %-THROTPOS

007A        2D     45               16.6 KPA     50.0

007B        1C     28               10.3 KPA     62.5

007C        18     24                  9 KPA     75.0

007D        10     16                  6 KPA     87.5

007E        0E     14                  5 KPA    100.0

007F        50     80 KMAXOFF       29.5 KPA    MAXIMUM MAP OFFSET FOR BARO ADJUSTMENT

#************************************************

# F11Q   ALTITUDE COMPENSATION FACTOR VS NBARO  *

#        USED FOR MAP OFFSET TO COMPUTE BARO    *

#        TABLE VALUE = FACTOR * 128             *

#************************************************

* -------------------------------------------------------------------

0080        5C     92 F11Q          0.72 FACTR     65    KPA-BARO

0081        60     96               0.75 FACTR     75

0082        6D    109               0.85 FACTR     85

0083        7A    122               0.95 FACTR     95

0084        86    134               1.05 FACTR    105

#************************************************

# F12    ALTITUDE COMPENSATION FACTOR VS NBARO  *

#        USED FOR ALTITUDE COMP TO NMAPLD       *

#        NMAPLDAL IS THE RESULTANT VARIABLE     *

#        TABLE VALUE = FACTOR * 128             *

#************************************************

* -------------------------------------------------------------------

0085        C5    197 F12B          1.54 FACTR     65    KPA-BARO

0086        AB    171              1.336 FACTR     75

0087        96    150               1.17 FACTR     85

0088        80    128                  1 FACTR     95

0089        80    128                  1 FACTR    105

#************************************************

# F13    ALTITUDE COMPENSATION FACTOR VS NBARO  *

#        USED FOR ALTITUDE COMP TO NMAPLD       *

#        NMAPLD13 IS THE RESULTANT VARIABLE     *

#        TABLE VALUE = FACTOR * 128             *

#************************************************

* -------------------------------------------------------------------

008A        C5    197 F13A          1.54 FACTR     65    KPA-BARO

008B        B2    178               1.39 FACTR     75

008C        A0    160               1.25 FACTR     85

008D        8C    140               1.09 FACTR     95

008E        7A    122               0.95 FACTR    105

008F        80    128 KMPGMULT         1 FACT   EMPIRICALLY DERIVED MULTIPLIER TO BE USED TO MULTIPLY

ACCUMULATED FUEL  SO THAT MPG CALC IS OK

0090        E0    224 KLDHM           90 KPA    IF MAP > KLDHM, DISABLE CONV OVERTEMP SPARK

0091        8C    140 KCLTHM         65 DEG C    IF COOLANT <= KCLTHM, DISABLE CONV OVERTEMP SPARK

0092        40     64 KRPMHM         1600 RPM    IF RPM < THIS DISABLE CONV OVERTEMPSPARK

0093        06      6 KTIMEHMA         6 SEC    CONV OVERTMP IS ENABLED IF CONDITIONS ARE

PRESENT FOR TIME > THIS

0094        00      0 KHMCONTM         0 SEC    CONV OVERTMP IS ENABLED IF CONDITIONS ARE

PRESENT FOR TIME > THIS (IF

CATALYTIC CONVERTER

OVERTEMP CONDITION PRESENT)

#################################################

#  F5D TABLE    SAHM                            #

# CONVERTER OVERTEMP SPARK ADVANCE VS. MAP LOAD #

#################################################

* -------------------------------------------------------------------

0095        08      8 F5D              8        USE 9 VALUE TABLE

0096        06      6                  2 DEG       20    KPA-MAP

0097        06      6                  2 DEG       30

0098        0B     11                  4 DEG       40

0099        0B     11                  4 DEG       50

009A        09      9                  3 DEG       60

009B        06      6                  2 DEG       70

009C        03      3                  1 DEG       80

009D        00      0                  0 DEG       90

009E        00      0                  0 DEG      100

009F        CA    202 KMAPSTL         85 KPA    MAP THRESH FOR ST TORQ MANAGE

00A0      0000    $9999   0 KVATSHI          0 HERTZ  FAIL VATS CHECK IF INPUT FREQ > THIS

89 F-Car conversion =   N=65536/E

LOW CAL  =  30Hz Signal =  20 HZ = 13107 = $3333       40Hz = 26214  = $6666

HI CAL  =  50Hz Signal =  40 Hz =  26214  = $6666        60Hz =  39321 = $9999

00A2      0000   $6666    0 KVATSLO          0 HERTZ  FAIL VATS CHECK IF INPUT FREQ < THIS

00A4        30     48 K4THNVMN        48         'RPM/MPH' 4TH GEAR RATHRESHOLD (CONV OT SP AD)

00A5        0C     12 KMATOFST        12 CTS    A/D COUNTS OFFSET FOR MAT SENSOR

00A6        50     80 KELTIME          8 SEC         MAXIMUM SHIFT LITE ON TIME

00A7        14     20 KELNDTPS         8 %TPS    TURN OFF E-LITE IF NEGATIVE DELTA  TPS > THIS

00A8        4B     75 KELCOOL         16 DEG C   COOLANT THRESHOLD FOR E-LITE ENABLE

00A9        05      5 KTPSHYSM         2 %            TPS HYSTERESIS FOR MINIMUM TPS FOR SHFT LITE ON

00AA        14     20 KRPMHYSM       500 RPM    RPM HYSTERESIS FOR MIN RPM FOR SHIFT LITE ON

00AB        E0     224 KRPMAX          5600 RPM    RPM ABOVE WHICH LIGHT IS ALWAYS TURNED ON    RPM/25

00AC      0003      3 KSHFMPHL       0.01 MPH     MPH BELOW WHICH LIGHT IS ALWAYS TURNED OFF

* ------------------------------------------- ------------------------

00AE        33     51 F47G1ST         20 %       1000    RPM-SPEED

00AF        38     56                 22 %       1400

00B0        3D     61                 24 %       1800

00B1        42     66               25.8 %       2200

00B2        4D     77                 30 %       2600

00B3        5A     90                 35 %       3000

00B4        66    102                 40 %       3400

00B5        80    128                 50 %       3800

00B6        9A    154                 60 %       4200

00B7        C0    192                 75 %       4600

00B8        E6    230                 90 %       5000

00B9        BE    190 KNVRAT1H       190         'RPM/MPH' UPPER N/V WINDOW VALUE, 1ST GEAR

00BA        A5    165 KNVRAT1L       165         'RPM/MPH' LOWER N/V WINDOW VALUE, 1ST GEAR

00BB        5E     94 KRPMIN1       2350 RPM    MINIMUM RPM FOR LIGHT ON, 1ST GEAR

00BC        0D     13 KTPSNLT1         5 %         MINIMUM TPS FOR LIGHT ON, 1ST GEAR

00BD        05      5 KGRDLY1        0.5 SEC      LIGHT ON DELAY TIME, 1ST GEAR

00BE        05      5 KTPSHYS1         2 %         TPS HYSTERESIS FOR LIGHT ON, 1ST GEAR

00BF        0A     10 KLITDLY1         1 SEC       LIGHT ON DELAY TIME, 1ST GEAR

* -------------------------------------------------------------------

00C0        28     40 F47G2ND       15.6 %       1000    RPM-SPEED

00C1        2E     46                 18 %       1400

00C2        33     51                 20 %       1800

00C3        38     56                 22 %       2200

00C4        40     64                 25 %       2600

00C5        4C     76               29.7 %       3000

00C6        5A     90                 35 %       3400

00C7        73    115                 45 %       3800

00C8        8D    141                 55 %       4200

00C9        B3    179                 70 %       4600

00CA        DA    218                 85 %       5000

00CB        73    115 KNVRAT2H       115         'RPM/MPH' UPPER N/V WINDOW VALUE, 2ND GEAR

00CC        5A     90 KNVRAT2L        90         'RPM/MPH' LOWER N/V WINDOW VALUE, 2ND GEAR

00CD        5A     90 KRPMIN2       2250 RPM    MINIMUM RPM FOR LIGHT ON, 2ND GEAR

00CE        0D     13 KTPSNLT2         5 %      MINIMUM TPS FOR LIGHT ON, 2ND GEAR

00CF        0A     10 KGRDLY2          1 SEC    LIGHT ON DELAY TIME, 2ND GEAR

00D0        05      5 KTPSHYS2         2 %      TPS HYSTERESIS FOR LIGHT ON, 2ND GEAR

00D1        0A     10 KLITDLY2         1 SEC    LIGHT ON DELAY TIME, 2ND GEAR

* -------------------------------------------------------------------

00D2        1E     30 F47G3RD       11.7 %       1000    RPM-SPEED

00D3        23     35               13.7 %       1400

00D4        28     40               15.6 %       1800

----      ---- ------ -------- --------- ------ -------------------------------

00D5        33     51                 20 %       2200

00D6        40     64                 25 %       2600

00D7        4D     77                 30 %       3000

00D8        5A     90                 35 %       3400

00D9        66    102                 40 %       3800

00DA        80    128                 50 %       4200

00DB        9A    154                 60 %       4600

00DC        CD    205                 80 %       5000

00DD        50     80 KNVRAT3H        80         'RPM/MPH' UPPER N/V WINDOW VALUE, 3RD GEAR

00DE        41     65 KNVRAT3L        65         'RPM/MPH' LOWER N/V WINDOW VALUE, 3RD GEAR

00DF        4C     76 KRPMIN3       1900 RPM    MINIMUM RPM FOR LIGHT ON, 3RD GEAR

00E0        0A     10 KTPSNLT3         4 %      MINIMUM TPS FOR LIGHT ON, 3RD GEAR

00E1        12     18 KGRDLY3        1.8 SEC    LIGHT ON DELAY TIME, 3RD GEAR

00E2        05      5 KTPSHYS3         2 %      TPS HYSTERESIS FOR LIGHT ON, 3RD GEAR

00E3        12     18 KLITDLY3       1.8 SEC    LIGHT ON DELAY TIME, 3RD GEAR

* -------------------------------------------------------------------

00E4        14     20 F47G4TH          8 %       1000    RPM-SPEED

00E5        17     23                  9 %       1400

00E6        1B     27               10.5 %       1800

00E7        1F     31                 12 %       2200

00E8        33     51                 20 %       2600

00E9        40     64                 25 %       3000

00EA        4D     77                 30 %       3400

00EB        5A     90                 35 %       3800

00EC        73    115                 45 %       4200

00ED        8D    141                 55 %       4600

00EE        B3    179                 70 %       5000

00EF        37     55 KNVRAT4H        55         'RPM/MPH' UPPER N/V WINDOW VALUE, 4TH GEAR

00F0        28     40 KNVRAT4L        40         'RPM/MPH' LOWER N/V WINDOW VALUE, 4TH GEAR

00F1        46     70 KRPMIN4       1750 RPM    MINIMUM RPM FOR LIGHT ON, 4TH GEAR

00F2        0A     10 KTPSNLT4         4 %      MINIMUM TPS FOR LIGHT ON, 4TH GEAR

00F3        12     18 KGRDLY4        1.8 SEC    LIGHT ON DELAY TIME, 4TH GEAR

00F4        05      5 KTPSHYS4         2 %      TPS HYSTERESIS FOR LIGHT ON, 4TH GEAR

00F5        0A     10 KLITDLY4         1 SEC    LIGHT ON DELAY TIME, 4TH GEAR

00F6        29     41 KNVRAT5H        41         'RPM/MPH' UPPER N/V WINDOW VALUE, 5TH GEAR

00F7        23     35 KNVRAT5L        35         'RPM/MPH' LOWER N/V WINDOW VALUE, 5TH GEAR

*************************************************************

* F48  BARO CORRECTION FACTOR FOR SHIFTLIGHT TPS THRESHOLD  *

*        TABLE VALUE = FACTOR * 128                         *

*************************************************************

* -------------------------------------------------------------------

00F8        80    128 F48              1 FACTR     65    KPA-BARO

00F9        80    128                  1 FACTR     75

00FA        80    128                  1 FACTR     85

00FB        80    128                  1 FACTR     95

MAIN SPARK F1 TABLE

#################################################

#     F1 TABLE      SAMAIN                      #

#     MAIN SPARK ADVANCE VS. MAP AND RPM        #

#     TABLE VALUE = DEG * 256/90                #

#################################################

* -------------------------------------------------------------------

00FC        00      0 F1D              0        R MIN; R = NTRPM

00FD        00      0                  0        Q MIN; Q = NMAPLD

00FE        11     17                 17        R NUM

* --------------------------------------------- SPEED        400 RPM

00FF        30     48                 17 DEG       20    KPA-MAP

0100        33     51                 18 DEG       25

0101        3A     58               20.4 DEG       30

0102        3C     60                 21 DEG       35

0103        3F     63                 22 DEG       40

0104        41     65                 23 DEG       45

0105        44     68                 24 DEG       50

0106        44     68                 24 DEG       55

0107        33     51                 18 DEG       60

0108        22     34                 12 DEG       65

0109        1C     28                 10 DEG       70

010A        1C     28                 10 DEG       75

010B        17     23                  8 DEG       80

010C        11     17                  6 DEG       85

010D        11     17                  6 DEG       90

010E        0B     11                  4 DEG       95

010F        06      6                  2 DEG      100

* --------------------------------------------- SPEED        600 RPM

0110        30     48                 17 DEG       20    KPA-MAP

0111        33     51                 18 DEG       25

0112        3A     58                 20.4 DEG       30

0113        3C     60                 21 DEG       35

0114        3F     63                 22 DEG       40

0115        41     65                 23 DEG       45

0116        44     68                 24 DEG       50

0117        44     68                 24 DEG       55

0118        33     51                 18 DEG       60

0119        33     51                 18 DEG       65

011A        2E     46                 16 DEG       70

011B        28     40                 14 DEG       75

011C        28     40                 14 DEG       80

011D        28     40                 14 DEG       85

011E        28     40                 14 DEG       90

011F        25     37                 13 DEG       95

0120        25     37                 13 DEG      100

* --------------------------------------------- SPEED        800 RPM

0121        3C     60                 21 DEG       20    KPA-MAP

0122        3C     60                 21 DEG       25

0123        3C     60                 21 DEG       30

0124        3C     60                 21 DEG       35

0125        3F     63                 22 DEG       40

0126        41     65                 23 DEG       45

0127        44     68                 24 DEG       50

0128        44     68                 24 DEG       55

0129        33     51                 18 DEG       60

012A        33     51                 18 DEG       65

012B        2E     46                 16 DEG       70

012C        28     40                 14 DEG       75

012D        28     40                 14 DEG       80

012E        28     40                 14 DEG       85

012F        28     40                 14 DEG       90

0130        25     37                 13 DEG       95

0131        25     37                 13 DEG      100

* --------------------------------------------- SPEED       1000 RPM

0132        44     68                 24 DEG       20    KPA-MAP

0133        44     68                 24 DEG       25

0134        44     68                 24 DEG       30

0135        3C     60                 21 DEG       35

0136        3F     63                 22 DEG       40

0137        41     65                 23 DEG       45

0138        44     68                 24 DEG       50

0139        44     68                 24 DEG       55

013A        41     65                 23 DEG       60

013B        41     65                 23 DEG       65

013C        3F     63                 22 DEG       70

013D        3F     63                 22 DEG       75

013E        3F     63                 22 DEG       80

013F        3C     60                 21 DEG       85

0140        3C     60                 21 DEG       90

0141        30     48                 17 DEG       95

0142        25     37                 13 DEG      100

* --------------------------------------------- SPEED       1200 RPM

0143        44     68                 24 DEG       20    KPA-MAP

0144        44     68                 24 DEG       25

0145        44     68                 24 DEG       30

0146        3C     60                 21 DEG       35

0147        3F     63                 22 DEG       40

0148        41     65                 23 DEG       45

0149        44     68                 24 DEG       50

014A        44     68                 24 DEG       55

014B        41     65                 23 DEG       60

014C        41     65                 23 DEG       65

014D        3F     63                 22 DEG       70

014E        3F     63                 22 DEG       75

014F        3F     63                 22 DEG       80

0150        3C     60                 21 DEG       85

0151        3C     60                 21 DEG       90

0152        30     48                 17 DEG       95

0153        25     37                 13 DEG      100

* --------------------------------------------- SPEED       1400 RPM

0154        44     68                 24 DEG       20    KPA-MAP

0155        44     68                 24 DEG       25

0156        44     68                 24 DEG       30

0157        3C     60                 21 DEG       35

0158        3C     60                 21 DEG       40

0159        41     65                 23 DEG       45

015A        44     68                 24 DEG       50

015B        44     68                 24 DEG       55

015C        44     68                 24 DEG       60

015D        44     68                 24 DEG       65

015E        41     65                 23 DEG       70

015F        41     65                 23 DEG       75

0160        41     65                 23 DEG       80

0161        36     54                 19 DEG       85

0162        2E     46                 16 DEG       90

0163        2E     46                 16 DEG       95

0164        2B     43                 15 DEG      100

* --------------------------------------------- SPEED       1600 RPM

0165        4A     74                 26 DEG       20    KPA-MAP

0166        4A     74                 26 DEG       25

0167        4A     74                 26 DEG       30

0168        3C     60                 21 DEG       35

0169        3C     60                 21 DEG       40

016A        3F     63                 22 DEG       45

016B        47     71                 25 DEG       50

016C        4A     74                 26 DEG       55

016D        4A     74                 26 DEG       60

016E        4A     74                 26 DEG       65

016F        4A     74                 26 DEG       70

0170        47     71                 25 DEG       75

0171        44     68                 24 DEG       80

0172        4A     74                 26 DEG       85

0173        3F     63                 22 DEG       90

0174        39     57                 20 DEG       95

0175        30     48                 17 DEG      100

* --------------------------------------------- SPEED       1800 RPM

0176        4A     74                 26 DEG       20    KPA-MAP

0177        4A     74                 26 DEG       25

0178        4A     74                 26 DEG       30

0179        4A     74                 26 DEG       35

017A        44     68                 24 DEG       40

017B        44     68                 24 DEG       45

017C        44     68                 24 DEG       50

017D        4A     74                 26 DEG       55

017E        4A     74                 26 DEG       60

017F        4A     74                 26 DEG       65

0180        4A     74                 26 DEG       70

0181        47     71                 25 DEG       75

0182        44     68                 24 DEG       80

0183        3C     60                 21 DEG       85

0184        3C     60                 21 DEG       90

0185        33     51                 18 DEG       95

0186        33     51                 18 DEG      100

* --------------------------------------------- SPEED       2000 RPM

0187        4A     74                 26 DEG       20    KPA-MAP

0188        4A     74                 26 DEG       25

0189        4A     74                 26 DEG       30

018A        44     68                 24 DEG       35

018B        44     68                 24 DEG       40

018C        44     68                 24 DEG       45

018D        44     68                 24 DEG       50

018E        47     71                 25 DEG       55

018F        47     71                 25 DEG       60

0190        47     71                 25 DEG       65

0191        47     71                 25 DEG       70

0192        47     71                 25 DEG       75

0193        44     68                 24 DEG       80

0194        3C     60                 21 DEG       85

0195        39     57                 20 DEG       90

0196        36     54                 19 DEG       95

0197        36     54                 19 DEG      100

* --------------------------------------------- SPEED       2200 RPM

0198        50     80                 28 DEG       20    KPA-MAP

0199        50     80                 28 DEG       25

019A        50     80                 28 DEG       30

019B        4D     77                 27 DEG       35

019C        4D     77                 27 DEG       40

019D        50     80                 28 DEG       45

019E        50     80                 28 DEG       50

019F        50     80                 28 DEG       55

01A0        4D     77                 27 DEG       60

01A1        4D     77                 27 DEG       65

01A2        4A     74                 26 DEG       70

01A3        44     68                 24 DEG       75

01A4        41     65                 23 DEG       80

01A5        36     54                 19 DEG       85

01A6        36     54                 19 DEG       90

01A7        36     54                 19 DEG       95

01A8        36     54                 19 DEG      100

* --------------------------------------------- SPEED       2400 RPM

01A9        55     85                 30 DEG       20    KPA-MAP

01AA        55     85                 30 DEG       25

01AB        55     85                 30 DEG       30

01AC        55     85                 30 DEG       35

01AD        55     85                 30 DEG       40

01AE        55     85                 30 DEG       45

01AF        55     85                 30 DEG       50

01B0        52     82                 29 DEG       55

01B1        50     80                 28 DEG       60

01B2        50     80                 28 DEG       65

01B3        4D     77                 27 DEG       70

01B4        4D     77                 27 DEG       75

01B5        4D     77                 27 DEG       80

01B6        4A     74                 26 DEG       85

01B7        47     71                 25 DEG       90

01B8        41     65                 23 DEG       95

01B9        3F     63                 22 DEG      100

* --------------------------------------------- SPEED       2800 RPM

01BA        55     85                 30 DEG       20    KPA-MAP

01BB        55     85                 30 DEG       25

01BC        55     85                 30 DEG       30

01BD        50     80                 28 DEG       35

01BE        58     88                 31 DEG       40

01BF        58     88                 31 DEG       45

01C0        55     85                 30 DEG       50

01C1        52     82                 29 DEG       55

01C2        50     80                 28 DEG       60

01C3        50     80                 28 DEG       65

01C4        4D     77                 27 DEG       70

01C5        4A     74                 26 DEG       75

01C6        47     71                 25 DEG       80

01C7        47     71                 25 DEG       85

01C8        44     68                 24 DEG       90

01C9        3F     63                 22 DEG       95

01CA        3C     60                 21 DEG      100

* --------------------------------------------- SPEED       3200 RPM

01CB        4D     77                 27 DEG       20    KPA-MAP

01CC        4D     77                 27 DEG       25

01CD        4D     77                 27 DEG       30

01CE        50     80                 28 DEG       35

01CF        50     80                 28 DEG       40

01D0        50     80                 28 DEG       45

01D1        4D     77                 27 DEG       50

01D2        4D     77                 27 DEG       55

01D3        4D     77                 27 DEG       60

01D4        4A     74                 26 DEG       65

01D5        47     71                 25 DEG       70

01D6        47     71                 25 DEG       75

01D7        44     68                 24 DEG       80

01D8        41     65                 23 DEG       85

01D9        41     65                 23 DEG       90

01DA        3C     60                 21 DEG       95

01DB        39     57                 20 DEG      100

* --------------------------------------------- SPEED       3600 RPM

01DC        4D     77                 27 DEG       20    KPA-MAP

01DD        4D     77                 27 DEG       25

01DE        4D     77                 27 DEG       30

01DF        4A     74                 26 DEG       35

01E0        4A     74                 26 DEG       40

01E1        4D     77                 27 DEG       45

01E2        4D     77                 27 DEG       50

01E3        4A     74                 26 DEG       55

01E4        4A     74                 26 DEG       60

01E5        47     71                 25 DEG       65

01E6        47     71                 25 DEG       70

01E7        44     68                 24 DEG       75

01E8        44     68                 24 DEG       80

01E9        41     65                 23 DEG       85

01EA        3F     63                 22 DEG       90

01EB        39     57                 20 DEG       95

01EC        36     54                 19 DEG      100

* --------------------------------------------- SPEED       4000 RPM

01ED        52     82                 29 DEG       20    KPA-MAP

01EE        52     82                 29 DEG       25

01EF        52     82                 29 DEG       30

01F0        55     85                 30 DEG       35

01F1        5B     91                 32 DEG       40

01F2        58     88                 31 DEG       45

01F3        52     82                 29 DEG       50

01F4        50     80                 28 DEG       55

01F5        4D     77                 27 DEG       60

01F6        4A     74                 26 DEG       65

01F7        4A     74                 26 DEG       70

01F8        4A     74                 26 DEG       75

01F9        4A     74                 26 DEG       80

01FA        44     68                 24 DEG       85

01FB        41     65                 23 DEG       90

01FC        39     57                 20 DEG       95

01FD        39     57                 20 DEG      100

* --------------------------------------------- SPEED       4400 RPM

01FE        55     85                 30 DEG       20    KPA-MAP

01FF        55     85                 30 DEG       25

0200        55     85                 30 DEG       30

0201        55     85                 30 DEG       35

0202        52     82                 29 DEG       40

0203        55     85                 30 DEG       45

0204        55     85                 30 DEG       50

0205        50     80                 28 DEG       55

0206        50     80                 28 DEG       60

0207        4D     77                 27 DEG       65

0208        4A     74                 26 DEG       70

0209        47     71                 25 DEG       75

020A        44     68                 24 DEG       80

020B        41     65                 23 DEG       85

020C        41     65                 23 DEG       90

020D        41     65                 23 DEG       95

020E        41     65                 23 DEG      100

* --------------------------------------------- SPEED       4800 RPM

020F        4D     77                 27 DEG       20    KPA-MAP

0210        4D     77                 27 DEG       25

0211        4D     77                 27 DEG       30

0212        4A     74                 26 DEG       35

0213        4D     77                 27 DEG       40

0214        4D     77                 27 DEG       45

0215        4D     77                 27 DEG       50

0216        47     71                 25 DEG       55

0217        47     71                 25 DEG       60

0218        44     68                 24 DEG       65

0219        41     65                 23 DEG       70

021A        41     65                 23 DEG       75

021B        41     65                 23 DEG       80

021C        41     65                 23 DEG       85

021D        41     65                 23 DEG       90

021E        41     65                 23 DEG       95

021F        41     65                 23 DEG      100

PE SPARK

#############################################

#  F1BIAS       PE SPARK ADVANCE CORRECTION #

# PE SPARK ADVANCE CORRECTION VS. DELTA A/F #

#############################################

* -------------------------------------------------------------------

0220        00      0 F1BIAS           0 DEG    0.0   DELTA A/F FROM 14.7

0221        00      0                  0 DEG    1.6

0222        00      0                  0 DEG    3.2

0223        00      0                  0 DEG    4.8

#############################################

#  F1SPRTD  IDLE SPARK RETARD               #

#  SPARK RETARD VS. IDLE RPM ERROR          #

#############################################

* -------------------------------------------------------------------

0224        00      0 F1SPRTD1         0 DEG        0    RPM-SPEED

0225        06      6                  2 DEG     12.5

0226        0B     11                  4 DEG     25.0

0227        11     17                  6 DEG     37.5

0228        17     23                  8 DEG     50.0

0229        1C     28                 10 DEG     62.5

022A        22     34                 12 DEG     75.0

022B        1C     28                 10 DEG     87.5

022C        17     23                  8 DEG    100.0

022D        11     17                  6 DEG    112.5

022E        11     17                  6 DEG    125.0

022F        11     17                  6 DEG    137.5

0230        11     17                  6 DEG    150.0

0231        11     17                  6 DEG    162.5

0232        11     17                  6 DEG    175.0

0233        11     17                  6 DEG    187.5

0234        11     17                  6 DEG    200.0

IDLE SPARK RETARD

#############################################

#  F1SPADV  IDLE SPARK RETARD               #

#  SPARK ADVANCE VS. IDLE RPM ERROR         #

#############################################

* -------------------------------------------------------------------

0235        00      0 F1SPADV          0 DEG        0    RPM-SPEED

0236        03      3                  1 DEG     25.0

0237        06      6                  2 DEG     50.0

0238        09      9                  3 DEG     75.0

0239        09      9                  3 DEG    100.0

023A        09      9                  3 DEG    125.0

023B        09      9                  3 DEG    150.0

023C        09      9                  3 DEG    175.0

023D        09      9                  3 DEG    200.0

023E        09      9                  3 DEG    225.0

COOLANT SPARK

##############################################################

#  F2 TABLE      SATCP              CF.KCTBIAS               #

#  BASE COOLANT ADVANCE CORRECTION VS. LOAD AND COOLANT      #

#  EQU N=(E+KCTBIAS)*256/90                                  #

##############################################################

* -------------------------------------------------------------------

023F        00      0 F2F              0        LOAD SELECTOR (0=NVACLD,1=NMAPLD)

0240        20     32                 32        R MIN; R = COOLDEG

0241        10     16                 16        Q MIN; Q = LOAD

0242        08      8                  8        R NUM

* --------------------------------------------- COOLTEMP     -16 DEG C

0243        6F    111                  4 DEG       70    KPA-VAC

0244        6F    111                  4 DEG       60

0245        6F    111                  4 DEG       50

0246        6F    111                  4 DEG       40

0247        6F    111                  4 DEG       30

0248        6F    111                  4 DEG       20

0249        72    114                  5 DEG       10

024A        75    117                  6 DEG        0

* --------------------------------------------- COOLTEMP      -4 DEG C

024B        6F    111                  4 DEG       70    KPA-VAC

024C        6F    111                  4 DEG       60

024D        6F    111                  4 DEG       50

024E        6F    111                  4 DEG       40

024F        6F    111                  4 DEG       30

0250        6F    111                  4 DEG       20

0251        72    114                  5 DEG       10

0252        75    117                  6 DEG        0

* --------------------------------------------- COOLTEMP       8 DEG C

0253        64    100                  0 DEG       70    KPA-VAC

0254        64    100                  0 DEG       60

0255        64    100                  0 DEG       50

0256        64    100                  0 DEG       40

0257        69    105                  2 DEG       30

0258        6F    111                  4 DEG       20

0259        72    114                  5 DEG       10

025A        75    117                  6 DEG        0

* --------------------------------------------- COOLTEMP      20 DEG C

025B        55     85                 -5 DEG       70    KPA-VAC

025C        58     88                 -4 DEG       60

025D        5B     91                 -3 DEG       50

025E        64    100                  0 DEG       40

025F        6F    111                  4 DEG       30

0260        75    117                  6 DEG       20

0261        7A    122                  8 DEG       10

0262        80    128                 10 DEG        0

* --------------------------------------------- COOLTEMP      32 DEG C

0263        52     82                 -6 DEG       70    KPA-VAC

0264        55     85                 -5 DEG       60

0265        5B     91                 -3 DEG       50

0266        64    100                  0 DEG       40

0267        6F    111                  4 DEG       30

0268        75    117                  6 DEG       20

0269        7A    122                  8 DEG       10

026A        80    128                 10 DEG        0

* --------------------------------------------- COOLTEMP      44 DEG C

026B        5E     94                 -2 DEG       70    KPA-VAC

026C        58     88                 -4 DEG       60

026D        58     88                 -4 DEG       50

026E        58     88                 -4 DEG       40

026F        5B     91                 -3 DEG       30

0270        64    100                  0 DEG       20

0271        66    102                  1 DEG       10

0272        6C    108                  3 DEG        0

* --------------------------------------------- COOLTEMP      56 DEG C

0273        64    100                  0 DEG       70    KPA-VAC

0274        58     88                 -4 DEG       60

0275        58     88                 -4 DEG       50

0276        58     88                 -4 DEG       40

0277        5B     91                 -3 DEG       30

0278        64    100                  0 DEG       20

0279        66    102                  1 DEG       10

027A        69    105                  2 DEG        0

* --------------------------------------------- COOLTEMP      68 DEG C

027B        64    100                  0 DEG       70    KPA-VAC

027C        5B     91                 -3 DEG       60

027D        5B     91                 -3 DEG       50

027E        5B     91                 -3 DEG       40

027F        5E     94                 -2 DEG       30

0280        64    100                  0 DEG       20

0281        64    100                  0 DEG       10

0282        64    100                  0 DEG        0

* --------------------------------------------- COOLTEMP      80 DEG C

0283        64    100                  0 DEG       70    KPA-VAC

0284        5E     94                 -2 DEG       60

0285        61     97                 -1 DEG       50

0286        61     97                 -1 DEG       40

0287        64    100                  0 DEG       30

0288        64    100                  0 DEG       20

0289        64    100                  0 DEG       10

028A        64    100                  0 DEG        0

* --------------------------------------------- COOLTEMP      92 DEG C

028B        64    100                  0 DEG       70    KPA-VAC

028C        58     88                 -4 DEG       60

028D        55     85                 -5 DEG       50

028E        55     85                 -5 DEG       40

028F        55     85                 -5 DEG       30

0290        58     88                 -4 DEG       20

0291        5E     94                 -2 DEG       10

0292        64    100                  0 DEG        0

* --------------------------------------------- COOLTEMP     104 DEG C

0293        64    100                  0 DEG       70    KPA-VAC

0294        58     88                 -4 DEG       60

0295        55     85                 -5 DEG       50

0296        55     85                 -5 DEG       40

0297        55     85                 -5 DEG       30

0298        5E     94                 -2 DEG       20

0299        5E     94                 -2 DEG       10

029A        5E     94                 -2 DEG        0

* --------------------------------------------- COOLTEMP     116 DEG C

029B        64    100                  0 DEG       70    KPA-VAC

029C        64    100                  0 DEG       60

029D        64    100                  0 DEG       50

029E        64    100                  0 DEG       40

029F        61     97                 -1 DEG       30

02A0        5E     94                 -2 DEG       20

02A1        5B     91                 -3 DEG       10

02A2        5B     91                 -3 DEG        0

* --------------------------------------------- COOLTEMP     128 DEG C

02A3        64    100                  0 DEG       70    KPA-VAC

02A4        64    100                  0 DEG       60

02A5        64    100                  0 DEG       50

02A6        64    100                  0 DEG       40

02A7        64    100                  0 DEG       30

02A8        5E     94                 -2 DEG       20

02A9        5B     91                 -3 DEG       10

02AA        5B     91                 -3 DEG        0

* --------------------------------------------- COOLTEMP     140 DEG C

02AB        64    100                  0 DEG       70    KPA-VAC

02AC        64    100                  0 DEG       60

02AD        64    100                  0 DEG       50

02AE        64    100                  0 DEG       40

02AF        64    100                  0 DEG       30

02B0        5E     94                 -2 DEG       20

02B1        5B     91                 -3 DEG       10

02B2        5B     91                 -3 DEG        0

* --------------------------------------------- COOLTEMP     152 DEG C

02B3        64    100                  0 DEG       70    KPA-VAC

02B4        64    100                  0 DEG       60

02B5        64    100                  0 DEG       50

02B6        64    100                  0 DEG       40

02B7        64    100                  0 DEG       30

02B8        5E     94                 -2 DEG       20

02B9        5B     91                 -3 DEG       10

02BA        5B     91                 -3 DEG        0

EGR SPARK ADVANCE

######################################################

#  F4  TABLE     SAEGR           CF.KEGRBIAS         #

#  EGR SPARK ADVANCE VS. %EGR                        #

#  N=(E+KEGRBIAS)*256/90                             #

######################################################

* -------------------------------------------------------------------

02BB        00      0 F4D              0 DEG        0      %-EGR

02BC        00      0                  0 DEG      1.6

02BD        05      5                1.8 DEG      3.2

02BE        09      9                  3 DEG      4.8

02BF        0B     11                  4 DEG      6.4

02C0        0E     14                  5 DEG      8.0

02C1        11     17                  6 DEG      9.6

02C2        11     17                  6 DEG     11.2

02C3        11     17                  6 DEG     12.8

02C4        11     17                  6 DEG     14.4

### ESC SPARK

#####################################################

# F6                                                #

# ESC ATTACK RATE VS. RPM                           #

TABLE VALUE = ATTACK RATE IN (DEGREES/MSEC) / .0225 #

#####################################################

* -------------------------------------------------------------------

02C5        06      6 F6            0.13 DEG/MS   400    RPM-SPEED

02C6        08      8               0.18 DEG/MS  1200

02C7        13     19               0.43 DEG/MS  2000

02C8        13     19               0.43 DEG/MS  3200

02C9        13     19               0.43 DEG/MS  4800

#####################################################

# F6PE                                              #

# ESC ATTACK RATE VS. RPM  (IN POWER ENRICHMENT)    #

TABLE VALUE = ATTACK RATE IN (DEGREES/MSEC) / .0225 #

#####################################################

* -------------------------------------------------------------------

02CA        04      4 F6PE           0.1 DEG/MS   400    RPM-SPEED

02CB        04      4                0.1 DEG/MS  1200

02CC        08      8               0.18 DEG/MS  2000

02CD        0B     11               0.25 DEG/MS  3200

02CE        0E     14               0.31 DEG/MS  4800

################################################

# F7                                           #

# ESC % RECOVERY RATE VS RPM                   #

# TABLE VALUE: % RECOVERY PER SECOND * 256/500 #

################################################

* -------------------------------------------------------------------

02CF        0D     13 F7              25 %/SEC    400    RPM-SPEED

02D0        10     16                 31 %/SEC   1200

02D1        10     16                 31 %/SEC   2000

02D2        0C     12                 23 %/SEC   3200

02D3        0C     12                 23 %/SEC   4800

################################################

# F7PE TABLE                                   #

# ESC % RECOVERY RATE VS RPM                   #

#    (IN POWER ENRICHMENT)                     #

# TABLE VALUE: % RECOVERY PER SECOND * 256/500 #

################################################

* -------------------------------------------------------------------

02D4        18     24 F7PE            47 %/SEC    400    RPM-SPEED

02D5        18     24                 47 %/SEC   1200

02D6        20     32                 63 %/SEC   2000

02D7        20     32                 63 %/SEC   3200

02D8        24     36                 70 %/SEC   4800

ESC RETARD

############################

#  F6MAX1 TABLE            #

# MAX ESC RETARD VS RPM    #

# TABLE VALUE = DEG*256/45 #

############################

* -------------------------------------------------------------------

02D9        39     57                 10 DEG      800    RPM-SPEED

02DA        39     57                 10 DEG     1600

02DB        39     57                 10 DEG     2400

02DC        55     85                 15 DEG     3200

02DD        55     85                 15 DEG     4000

###########################

#  F6PEMAX TABLE          #

# MAX ESC RETARD VS RPM   #

# (IN POWER ENRICHMENT)   #

#TABLE VALUE = DEG*256/45 #

###########################

* -------------------------------------------------------------------

02DE        22     34 F6PEMAX          6 DEG      800    RPM-SPEED

02DF        22     34                  6 DEG     1600

02E0        2E     46                  8 DEG     2400

02E1        55     85                 15 DEG     3200

02E2        55     85                 15 DEG     4000

#################################################

# F8E TABLE     SRTCC               CF.KTCCBIAS #

# TCC LOCKED SPARK RETARD VS RPM AND MAP        #

#    N=E*256/90                                 #

#################################################

* -------------------------------------------------------------------

02E3        60     96 F8E             96        RMIN; R=NTRPM125 (1200 RPM)

02E4        60     96                 96        QMIN; Q = 96

02E5        08      8                  8        RNUM

* --------------------------------------------- SPEED       1200 RPM

02E6        00      0                  0 DEG       50    KPA-MAP

02E7        00      0                  0 DEG       55

02E8        00      0                  0 DEG       60

02E9        03      3                  1 DEG       65

02EA        03      3                  1 DEG       70

02EB        06      6                  2 DEG       75

02EC        06      6                  2 DEG       80

02ED        06      6                  2 DEG       85

* --------------------------------------------- SPEED       1400 RPM

02EE        00      0                  0 DEG       50    KPA-MAP

02EF        00      0                  0 DEG       55

02F0        00      0                  0 DEG       60

02F1        00      0                  0 DEG       65

02F2        03      3                  1 DEG       70

02F3        03      3                  1 DEG       75

02F4        06      6                  2 DEG       80

02F5        06      6                  2 DEG       85

* --------------------------------------------- SPEED       1600 RPM

02F6        00      0                  0 DEG       50    KPA-MAP

02F7        00      0                  0 DEG       55

02F8        00      0                  0 DEG       60

02F9        00      0                  0 DEG       65

02FA        03      3                  1 DEG       70

02FB        03      3                  1 DEG       75

02FC        06      6                  2 DEG       80

02FD        06      6                  2 DEG       85

* --------------------------------------------- SPEED       1800 RPM

02FE        00      0                  0 DEG       50    KPA-MAP

02FF        00      0                  0 DEG       55

0300        00      0                  0 DEG       60

0301        00      0                  0 DEG       65

0302        00      0                  0 DEG       70

0303        00      0                  0 DEG       75

0304        03      3                  1 DEG       80

0305        03      3                  1 DEG       85

* --------------------------------------------- SPEED       2000 RPM

0306        00      0                  0 DEG       50    KPA-MAP

0307        00      0                  0 DEG       55

0308        00      0                  0 DEG       60

0309        00      0                  0 DEG       65

030A        00      0                  0 DEG       70

030B        00      0                  0 DEG       75

030C        00      0                  0 DEG       80

030D        00      0                  0 DEG       85

###########################################################

#     F180 TABLE  IDLE SPARK MULTIPLIER                   #

#     MULTIPLIER (0-1) VS COOLANT TEMP. (COOLDEGB)        #

#   TABLE VALUE = MULT. * 256                             #

###########################################################

* -------------------------------------------------------------------

030E        60     96             0.375 MULT     -28  DEG C-COOLTEMP

030F        70    112              0.438 MULT      -4

0310        80    128                0.5 MULT      20

0311        A0    160              0.625 MULT      44

0312        C0    192               0.75 MULT      68

0313        F0    240              0.938 MULT      92

0314        E0    224              0.875 MULT     116

#########################################################

#  F81   TABLE                                                                                        #

# INITIAL TIME OUT SPARK VALUE VS COOLANT TEMPERATURE #

# TABLE VALUE = (DEGREES+KSTIBIAS)*256/90                              #

#########################################################

* -------------------------------------------------------------------

0315        6F    111 F81              4 DEG      -40  DEG C-COOLTEMP

0316        6F    111                  4 DEG      -28

0317        6F    111                  4 DEG      -16

0318        6F    111                  4 DEG       -4

0319        6F    111                  4 DEG        8

031A        6F    111                  4 DEG       20

031B        6C    108                  3 DEG       32

031C        64    100                  0 DEG       44

031D        64    100                  0 DEG       56

031E        64    100                  0 DEG       68

031F        64    100                  0 DEG       80

0320        5B     91                 -3 DEG       92

0321        5B     91                 -3 DEG      104

0322        5B     91                 -3 DEG      116

0323        64    100                  0 DEG      128

0324        64    100                  0 DEG      140

0325        64    100                  0 DEG      152

#########################################################

#  F83  TABLE                                           #

# SPARK TIME OUT DECAY VALUE VS STARTUP COOLANT         #

# TABLE VALUE = DEGREES * 256/90                        #

#                                                       #

* -------------------------------------------------------------------

0326        01      1 F83            0.3 DEG      -40  DEG C-COOLTEMP

0327        01      1                0.3 DEG      -28

0328        01      1                0.3 DEG      -16

0329        01      1                0.3 DEG       -4

032A        01      1                0.3 DEG        8

032B        01      1                0.3 DEG       20

032C        01      1                0.3 DEG       32

032D        01      1                0.3 DEG       44

032E        01      1                0.3 DEG       56

032F        01      1                0.3 DEG       68

0330        01      1                0.3 DEG       80

0331        01      1                0.3 DEG       92

0332        01      1                0.3 DEG      104

0333        01      1                0.3 DEG      116

0334        01      1                0.3 DEG      128

0335        01      1                0.3 DEG      140

0336        01      1                0.3 DEG      152

###########################################################

#     F181 TABLE  IDLE SPARK MULTIPLIER                   #

#     MULTIPLIER (0-1) VS UNFILTERED MAP                  #

#   TABLE VALUE = MULT. * 256                             #

###########################################################

* -------------------------------------------------------------------

0337        A0    160 F181         0.625 MULT    19.9    KPA-MAP

0338        A0    160              0.625 MULT    25.8

0339        9E    158              0.617 MULT    31.7

033A        9A    154                0.6 MULT    37.6

033B        80    128                0.5 MULT    43.5

033C        80    128                0.5 MULT    49.4

033D        80    128                0.5 MULT    55.3

033E        60     96              0.375 MULT    61.2

033F        00      0 KCCPLOAD        80 KPA    LOAD THRESH TO MOD DELTADC IN CL CCP

0340        00      0 KCCPTIME         0 SEC    TIME INTERVAL TO MOD DELTADC IN CL CCP

0341        00      0 KCPINTHA         0 CTS    INT THRESH ABOVE WHICH DELTADC IS DECREASED

0342        00      0 KCPINTLA         0 CTS    INT THRESH BELOW WHICH DELTADC IS INCREASED

0343        00      0 KCCPDCDN         0 %      AMOUNT BY WHICH DELTADC IS DECREASED

0344        00      0 KCCPDCUP         0 %      AMOUNT BY WHICH DELTADC IS INCREASED

0345        00      0 KCCPTPDC         0 %      DELTADC THRESH BELOW WHICH DELTADC IS CLEARED

0346        00      0 KCCPTPMP         0 MPH    MPH THRESH BELOW WHICH DELTADC IS CLEARED

0347        00      0 KCCPTPBL         0 CTS    BLM THRESH BELOW WHICH DELTADC IS CLEARED

0348        FE    254 KCCPTM         254 SEC    IF ENGINE RUN TIME <= THIS DISABLE CCP

0349        A0    160 KCCPTMP         80 DEG C  DISABLE CCP IF COOLANT < THIS

034A        01      1 KCPVST1        0.3 MPH    ENABLE CCP IF MPH >= THIS (CCP OFF)

034B        0F     15 KCCPTPSU         6 %      ENABLE CCP IF TPS >= THIS (CCP OFF)

034C        0C     12 KCCPFLWH         3 GM/SEC ENABLE CCP IF FLOW > THIS (CCP OFF)

034D        00      0 KCPVST2          0 MPH    DISABLE CCP IF MPH < THIS (CCP ON)

034E        05      5 KCCPTPSL         2 %      DISABLE CCP IF TPS < THIS (CCP ON)

034F        04      4 KCCPFLWL         1 GM/SEC DON'T DISABLE CCP IF FLOW > THIS

0350        0B     11 KCCPCOEF        11 COEFF  FILTER COEFFICIENT FOR CCP DC

0351        04      4 KCPMULT        1.6 %      INC CPMULT BY THIS EVERY SECOND IF PURGE ON

0352        01      1 KCPMULTA       0.4 %      DEC CPMULT BY THIS EVERY SECOND IF PURGE OFF

AND CPMULT > KCPMULTL

0353        80    128 KCPMULTL        50 %      LOWER LIMIT FOR CPMULT DECREMENT (SEE KCPMULTA)

* -------------------------------------------------------------------

0354        00      0 F173             0 %          0 AIRFLO-GM/SEC

0355        0E     14                5.5 %          4

0356        1B     27               10.5 %          8

0357        25     37               14.4 %         12

0358        2F     47               18.4 %         16

0359        39     57               22.3 %         20

035A        4C     76               29.7 %         24

035B        64    100                 39 %         28

035C        73    115                 45 %         32

035D        82    130               50.8 %         36

035E        91    145               56.6 %         40

035F        9F    159                 62 %         44

0360        AD    173               67.6 %         48

0361        C7    199               77.7 %         52

0362        D9    217               84.8 %         56

0363        E8    232               90.6 %         60

0364        FF    255               99.6 %         64

* -------------------------------------------------------------------

0365        80    128 F174             1 MULT       8  DEG C-TEMP

0366        80    128                  1 MULT      20

0367        80    128                  1 MULT      32

0368        80    128                  1 MULT      44

0369        80    128                  1 MULT      56

036A        80    128                  1 MULT      68

036B        78    120               0.94 MULT      80

036C        60     96               0.75 MULT      92

036D        40     64                0.5 MULT     104

036E        40     64                0.5 MULT     116

036F        40     64                0.5 MULT     128

0370        30     48              0.375 MULT     140

0371        20     32               0.25 MULT     152

* -------------------------------------------------------------------

0372        66    102 F175           0.8 MULT      60    KPA-VAC

0373        6C    108               0.84 MULT      50

0374        85    133               1.04 MULT      40

0375        9D    157               1.23 MULT      30

0376        CD    205                1.6 MULT      20

0377        F7    247               1.93 MULT      10

0378        FF    255               1.99 MULT       0

----      ---- ------ -------- --------- ------ -------------------------------

0379        A6    166 KTPSCON         65 %      TPS THRESH IN CONV MELT DOWN FOR TCC LOCK

037A        FF    255 KREL1         99.6 %      IF NEG DELTA TPS IN 100 MSEC > THIS, UNLK TCC

037B        08      8 KLCKDLYT       0.8 SEC    LOCK TCC IF ALL CONDITIONS MET FOR TIME >= THIS

037C        32     50 KPDLY            5 SEC    POSITIVE DELTA TPS UNLOCK TIME

037D        32     50 KDNSHFTM         5 SEC    HIGH-MID GEAR DOWNSHIFT UNLOCK TIME

037E        54     84 KMPHXULL        84 MPH    DO NOT FORCE TCC LOCK IF MPH < THIS CAL

037F        58     88 KMPHXULH        88 MPH    UNCONDITIONALLY LOCK TCC IF MPH > THIS CAL

0380        FF    255 KRPMXUL       6375 RPM    IF LOCKED AND RPM > THIS, STAY LOCKED

0381        05      5 KTPSULKM         2 %TPS   LEAVE TCC UNLOCKED UNTIL 100 MSEC DTPS

IS LESS THAN THIS VALUE (MID

GEAR,UNLOCK)

0382        05      5 KTPSULKH         2 %TPS   LEAVE TCC UNLOCKED UNTIL 100 MSEC DTPS

IS LESS THAN THIS VALUE (HI

GEAR,UNLOCK)

0383        05      5 KCOASTUM         2 % TPS  IF MID GEAR & TPS < THIS UNLOCK TCC

0384        04      4 KRELUNMD       1.6  %     IF POSITIVE DELTA TPS IN 12.5 MSEC > THIS

UNLOCK TCC FOR KPDLY TIME

0385        20     32 KMNLCKM         32 MPH    ENABLE LOCK IF MPH > THIS

(IF IN MID GEAR AND UNLOCKED)

0386        23     35 KMLCKMAC        35 MPH    ENABLE LOCK IF MPH > THIS

(IF IN MID GEAR, UNLOCKED AND

A/C ON)

##################################

# F43L2 TABLE  (MID GEAR,LOWER)  #

# LOAD LIMIT VS. FILTMPH         #

# TABLE VALUE = % FULL LOAD*2.56 #

##################################

* -------------------------------------------------------------------

0387        13     19                 7.4 %      20   MPH

0388        20     32               12.5 %      25

0389        2C     44                 17 %      30

038A        39     57               22.3 %      35

038B        47     71               27.7 %      40

038C        57     87                 34 %      45

038D        66    102                 40 %      50

038E        74    116               45.3 %      55

038F        7F    127               49.6 %      60

0390        85    133                 52 %      68

0391        85    133                 52 %      76

0392        85    133                 52 %      84

0393        85    133                 52 %      92

0394        05      5 KCOASTUH         2 % TPS  IF HIGH GEAR & TPS < THIS UNLOCK TCC

0395        09      9 KRELUNHI       3.5  %     IF POSITIVE DELTA TPS IN 12.5 MSEC > THIS

UNLOCK TCC FOR KPDLY TIME

0396        20     32 KMNLCKH         32 MPH    ENABLE LOCK IF MPH > THIS

(IF IN HIGH GEAR AND UNLOCKED)

0397        23     35 KMLCKHAC        35 MPH    ENABLE LOCK IF MPH > THIS

(IF IN HIGH GEAR,UNLOCKED AND

A/C ON)

##################################

# F44L2 TABLE (HIGH GEAR,LOWER)  #

# LOAD LIMIT VS. FILTMPH         #

# TABLE VALUE = % FULL LOAD*2.56 #

##################################

* -------------------------------------------------------------------

0398        24     36                 14 %      20   MPH

0399        24     36                 14 %      25

039A        24     36                 14 %      30

039B        24     36                 14 %      35

039C        24     36                 14 %      40

039D        2A     42               16.4 %      45

039E        36     54                 21 %      50

039F        43     67                 26 %      55

03A0        4F     79                 31 %      60

03A1        5F     95                 37 %      68

03A2        5F     95                 37 %      76

03A3        5F     95                 37 %      84

03A4        5F     95                 37 %      92

03A5        03      3 KCOASTLM         1 % TPS  UNLOCK TCC IF TPS < THIS (MID GEAR)

03A6        04      4 KRELLKMD       1.6  %     IF POSITIVE DELTA TPS IN 12.5 MSEC > THIS

UNLOCK TCC FOR KPDLY TIME

03A7        1E     30 KMNULCKM        30 MPH    REMAIN LOCKED IF MPH > THIS, MID GEAR

03A8        21     33 KMULKMAC        33 MPH    REMAIN LOCKED IF MPH > THIS, MID GEAR

WITH A/C ON

##################################

# F43U2 TABLE  (MID GEAR,UPPER)  #

# LOAD LIMIT VS. FILTMPH         #

# TABLE VALUE = % FULL LOAD*2.56 #

##################################

* -------------------------------------------------------------------

03A9        1F     31                12 %      20   MPH

03AA        2D     45               17.6 %      25

03AB        3D     61                 24 %      30

03AC        4E     78               30.5 %      35

03AD        5F     95                 37 %      40

03AE        71    113                 44 %      45

03AF        82    130               50.8 %      50

03B0        8D    141                 55 %      55

03B1        94    148                 58 %      60

03B2        94    148                 58 %      68

03B3        94    148                 58 %      76

03B4        94    148                 58 %      84

03B5        94    148                 58 %      92

03B6        03      3 KCOASTLH         1 % TPS  UNLOCK TCC IF TPS < THIS (HIGH GEAR)

03B7        09      9 KRELLKHI       3.5  %     IF POSITIVE DELTA TPS IN 12.5 MSEC > THIS

UNLOCK TCC FOR KPDLY TIME

03B8        1E     30 KMNULCKH        30 MPH    REMAIN LOCKED IF MPH > THIS, HIGH GEAR

03B9        21     33 KMULKHAC        33 MPH    REMAIN LOCKED IF MPH > THIS, HIGH GEAR

WITH A/C ON

##################################

# F44U2 TABLE (HIGH GEAR,UPPER)  #

# LOAD LIMIT VS. FILTMPH         #

# TABLE VALUE = % FULL LOAD*2.56 #

##################################

* -------------------------------------------------------------------

03BA        2E     46                 18 %      20   MPH

03BB        2E     46                 18 %      25

03BC        2E     46                 18 %      30

03BD        2E     46                 18 %      35

03BE        3A     58               22.7 %      40

03BF        45     69                 27 %      45

03C0        4D     77                 30 %      50

03C1        5A     90                 35 %      55

03C2        61     97                 38 %      60

03C3        7A    122               47.7 %      68

03C4        7A    122               47.7 %      76

03C5        7A    122               47.7 %      84

03C6        7A    122               47.7 %      92

####################################

#  F123 TABLE                      #

# TCC LOAD LIMIT ALT. COMP. MULT.  #

#     VS. BARO    (UNLOCKED)       #

####################################

* -------------------------------------------------------------------

03C7        70    112               0.875 MULT      65    KPA-BARO

03C8        73    115                0.9 MULT      70

03C9        77    119               0.93 MULT      75

03CA        7C    124               0.97 MULT      80

03CB        80    128                  1 MULT      85

03CC        80    128                  1 MULT      90

03CD        80    128                  1 MULT      95

03CE        80    128                  1 MULT     100

03CF        80    128                  1 MULT     105

####################################

#  F124 TABLE                      #

# TCC LOAD LIMIT ALT. COMP. MULT.  #

#     VS. BARO    (LOCKED)         #

####################################

* -------------------------------------------------------------------

03D0        70    112               0.875 MULT      65    KPA-BARO

03D1        73    115                0.9 MULT      70

03D2        77    119               0.93 MULT      75

03D3        7C    124               0.97 MULT      80

03D4        80    128                  1 MULT      85

03D5        80    128                  1 MULT      90

03D6        80    128                  1 MULT      95

03D7        80    128                  1 MULT     100

03D8        80    128                  1 MULT     105

03D9        8C    140 KTCCTMP         65 DEG C  UNLOCK TCC IF COOLANT < THIS

03DA        FF    255 KTCCTMPH       151 DEG C  UNLOCK TCC IF COOLANT >= THIS

03DB        E3    227 KTCCTMPT       130 DEG C  LOCK TCC IF COOLANT > THIS

03DC        19     25 KMPHOTL         25 MPH    IF COOLANT > KTCCTMPT , UNLOCK IF MPH < THIS

LOW HYSTERESIS VALUE, WITH

KMPHOTH

03DD        1B     27 KMPHOTH         27 MPH    IF COOLANT > KTCCTMPT , LOCK IF MPH > THIS

HIGH HYSTERESIS VALUE, WITH

KMPHOTL

----      ---- ------ -------- --------- ------ -------------------------------

Fan 1 and  Fan 2 Cals

FAN # 1

03DE        C5    197 KFANTMPH       108 DEGC   IF COOL > THIS & A/C OFF, TURN ON FAN

03DF        C0    192 KFANTMPL       104 DEGC   IF COOL < THIS & A/C OFF, TURN OFF FAN

03E0        C3    195 KFNTMPHA       106 DEGC   IF A/C ON & COOL >= THIS, TURN ON FAN

03E1        BD    189 KFNTMPLA       102 DEGC   IF A/C ON & COOL <= THIS, TURN OFF FAN

03E2        FE    254 KFANTMR       25.4 SEC    MINIMUM FAN ON TIME

03E3        46     70 KFANMPH         70 MPH    IF VEHICLE SPEED > THIS IGNORE FAN REQ.

FAN # 2

03E4        D0    208 KFN2TMPH       116 DEG C  IF COOL >= THIS & A/C OFF, TURN ON FAN 2

03E5        CB    203 KFN2TMPL       112 DEG C  IF COOL <= THIS & A/C OFF,TURN OFF FAN 2

03E6        D0    208 KFN2THAC       116 DEG C  IF COOL >= THIS & A/C ON, TURN ON FAN 2

03E7        CB    203 KFN2TLAC       112 DEG C  IF COOL <= THIS & A/C ON,TURN OFF FAN 2

03E8        46     70 KFAN2MPH        70 MPH    IF MPH > THIS IGNORE 2ND FAN REQUEST

03E9        FE    254 KFAN2TMR      25.4 SEC    SECOND FAN MINIMUM 'ON' TIME

03EA        01      1 KFANDLY1       0.1 SEC    FAN #1 TURN ON DELAY

03EB        02      2 KFANDLY2       0.2 SEC    FAN #2 TURN ON DELAY

03EC        02      2 KFANSTP1         2 STEPS  # OF IAC MOT STEPS TO ADD FOR FAN #1

03ED        02      2 KFANSTP2         2 STEPS  # OF IAC MOT STEPS TO ADD FOR FAN #2

03EE        14     20 KFANAC           2 SEC    FAN ON DELAY IF FAN REQUESTED DURING A/C LEARN

CAT OVERTEMP

03EF        28     40 KCONRPML      1000 RPM    LO HYST VALUE TO DISABLE OVERTEMP AND MELTDOWN

MODES.  IF RPM < THIS, DISABLE.

03F0        30     48 KCONRPMH      1200 RPM    HI HYST VALUE TO ENABLE OVERTEMP AND MELTDOWN

MODES.  IF RPM > THIS, ENABLE.

03F1        87    135 KCONVHI        705 DEG C  CONVERTER OVER-TEMP THRESHOLD

WHEN THE CONVERTER TEMPERATURE  EXCEEDS THIS VALUE,

THE CONVERTER   PROTECTION MODE IS ENABLED.

03F2        82    130 KCONVLO        690 DEG C  CONVERTER COOL-DOWN THRESHOLD  WHEN THE CONVERTER

TEMPERATURE  FALLS BELOW THIS  VALUE, THE CONVERTER

PROTECTION MODE IS DISABLED.

03F3        A7    167 KCONVHIA       801 DEG C  CONVERTER OVER-TEMP THRESHOLD (HIGH)  WHEN THE

CONVERTER TEMPERATURE RISE EXCEEDS THIS VALUE ALL

CONVERTER PROTECTION REMEDIAL ACTION WILL TAKE PLACE.

03F4        94    148 KCONVLOA       744 DEG C  CONVERTER COOL-DOWN THRESHOLD (HIGH) WHEN

CONVERTER TEMPERATURE RISE FALLS BELOW  THIS VALUE,

LOW TEMP OVERRISE REMEDIAL ACTION WILL STILL TAKE PLACE.

03F5        A7    167 KCONVCLT        85 DEG C  CONVERTER INITIALIZATION THRESHOLD WHEN COOLANT

TEMPERATURE >  THIS VALUE DURING INITIALIZATION, INITIALIZE

THE  CONVERTER TEMPERATURE TO KCONVTMP.

03F6        21     33 KCONVTMP       399 DEG C  INITIAL CONVERTER TEMP WHEN COOLANT HOT INITIALIZE

CONVERTER TEMPERATURE TO THIS VALUE  WHEN THE

COOLANT TEMPERATURE  IS ABOVE KCONVCLT.

*******************************************************************

* TABLE F140A                                                     *

*      ESTIMATED EXHAUST GAS TEMPERATURE                          *

*                 VS.  AIR FLOW                                   *

*******************************************************************

* -------------------------------------------------------------------

03F7        02      2                   2 CTS        0 AIRFLO-GM/SEC

03F8        4A     74                 74 CTS       16

----      ---- ------ -------- --------- ------ -------------------------------

03F9        6C    108                108 CTS       32

03FA        8F    143                143 CTS       48

03FB        99    153                153 CTS       64

03FC        A7    167                167 CTS       80

03FD        B7    183                183 CTS       96

03FE        C8    200                200 CTS      112

03FF        D9    217                217 CTS      128

*******************************************************************

* TABLE F141                                                      *

*      CONVERTER  TEMPERATURE FILTER COEFFICIENT                  *

*                 VS.  AIR FLOW                                   *

*******************************************************************

* -------------------------------------------------------------------

0400        02      2              0.008 COEF       0 AIRFLO-GM/SEC

0401        02      2              0.008 COEF      16

0402        03      3               0.01 COEF      32

0403        05      5               0.02 COEF      48

0404        08      8               0.03 COEF      64

0405        10     16              0.063 COEF      80

0406        13     19              0.074 COEF      96

0407        14     20               0.08 COEF     112

0408        14     20               0.08 COEF     128

*******************************************************************

* TABLE F142A                                                      *

*      ENDO/EXOTHERMIC REACTION TEMPERATURE COMPONENT             *

*                 VS.  AIR/FUEL RATIO                             *

*******************************************************************

* -------------------------------------------------------------------

0409        57     87                  87 CTS      7.0    A/F-RATIO

040A        5A     90                 90 CTS      7.8

040B        5A     90                 90 CTS      8.6

040C        5A     90                 90 CTS      9.4

040D        5F     95                 95 CTS     10.2

040E        5F     95                 95 CTS     11.0

040F        56     86                 86 CTS     11.8

0410        55     85                 85 CTS     12.6

0411        5A     90                 90 CTS     13.4

0412        7B    123                123 CTS     14.2

0413        83    131                131 CTS     15.0

AIR FLOW TABLE

*******************************************************************

* TABLE F143A                                                                   *

*      ENGINE SPEED REACTION TEMPERATURE          *

*                    VS.                                                                 *

*             RPM AND AIR FLOW                                           *

*******************************************************************

* -------------------------------------------------------------------

0414        00      0                  0        RMIN; R = NTRPMX

0415        00      0                  0        QMIN; Q = DISPFLOW

* -------------------------------------------------------------------

0416        09      9                  9        RNUM

* --------------------------------------------- SPEED          0 RPM

0417        00      0                  0 CTS        0 AIRFLO-GM/SEC

0418        00      0                  0 CTS        8

0419        00      0                  0 CTS       16

041A        00      0                  0 CTS       24

041B        00      0                  0 CTS       32

041C        00      0                  0 CTS       40

041D        00      0                  0 CTS       48

041E        00      0                  0 CTS       56

041F        00      0                  0 CTS       64

* --------------------------------------------- SPEED        800 RPM

0420        00      0                  0 CTS        0 AIRFLO-GM/SEC

0421        00      0                  0 CTS        8

0422        00      0                  0 CTS       16

0423        00      0                  0 CTS       24

0424        00      0                  0 CTS       32

0425        03      3                  3 CTS       40

0426        00      0                  0 CTS       48

0427        00      0                  0 CTS       56

0428        00      0                  0 CTS       64

* --------------------------------------------- SPEED       1600 RPM

0429        03      3                  3 CTS        0 AIRFLO-GM/SEC

042A        2F     47                 47 CTS        8

042B        14     20                 20 CTS       16

042C        1A     26                 26 CTS       24

042D        24     36                 36 CTS       32

042E        2A     42                 42 CTS       40

042F        24     36                 36 CTS       48

0430        1A     26                 26 CTS       56

0431        0E     14                 14 CTS       64

* --------------------------------------------- SPEED       2400 RPM

0432        6D    109                109 CTS        0 AIRFLO-GM/SEC

0433        44     68                 68 CTS        8

0434        25     37                 37 CTS       16

0435        1D     29                 29 CTS       24

0436        23     35                 35 CTS       32

0437        15     21                 21 CTS       40

0438        1F     31                 31 CTS       48

0439        0E     14                 14 CTS       56

043A        0A     10                 10 CTS       64

* --------------------------------------------- SPEED       3200 RPM

043B        73    115                115 CTS        0 AIRFLO-GM/SEC

043C        18     24                 24 CTS        8

043D        34     52                 52 CTS       16

043E        42     66                 66 CTS       24

043F        32     50                 50 CTS       32

0440        1E     30                 30 CTS       40

0441        39     57                 57 CTS       48

----      ---- ------ -------- --------- ------ -------------------------------

0442        3B     59                 59 CTS       56

0443        3D     61                 61 CTS       64

* --------------------------------------------- SPEED       4000 RPM

0444        CF    207                207 CTS        0 AIRFLO-GM/SEC

0445        C3    195                195 CTS        8

0446        39     57                 57 CTS       16

0447        45     69                 69 CTS       24

0448        3B     59                 59 CTS       32

0449        37     55                 55 CTS       40

044A        45     69                 69 CTS       48

044B        46     70                 70 CTS       56

044C        28     40                 40 CTS       64

* --------------------------------------------- SPEED       4800 RPM

044D        AD    173                173 CTS        0 AIRFLO-GM/SEC

044E        B7    183                183 CTS        8

044F        59     89                 89 CTS       16

0450        3A     58                 58 CTS       24

0451        38     56                 56 CTS       32

0452        22     34                 34 CTS       40

0453        1F     31                 31 CTS       48

0454        18     24                 24 CTS       56

0455        14     20                 20 CTS       64

* --------------------------------------------- SPEED       5600 RPM

0456        B7    183                183 CTS        0 AIRFLO-GM/SEC

0457        B3    179                179 CTS        8

0458        29     41                 41 CTS       16

0459        29     41                 41 CTS       24

045A        3C     60                 60 CTS       32

045B        3E     62                 62 CTS       40

045C        39     57                 57 CTS       48

045D        0C     12                 12 CTS       56

045E        18     24                 24 CTS       64

----      ---- ------ -------- --------- ------ -------------------------------

045F        64    100 KPORTIM         10 SEC    IF IN PE FOR TIME >= THIS DIVERT AIR, MAXIMUM AIR TO PORTS

TIME WHEN  IN PE

0460        37     55 KAETIM         5.5 SEC    IF RPM CONTINUOUSLY > KADRPMA FOR  TIME > THIS DIVERT AIR

0461        A1    161 KA0          0.712 VOLTS  DIVERT IF FILTERED O2 > KA0 FOR  KARLTM MORE TIME THAN

IT IT <=  TO KA0

0462        45     69 KB0          0.305 VOLTS  DIVERT IF FILTERED O2 < KB0 AND IN  CLOSED LOOP BOTH FOR A

TIME >   KARLTM

0463      012C    300 KARLTM          30 SEC    RICH OR LEAN O2 DIVERT TIMER

0465        C8    200 KADRPMA       5000 RPM    IF RPM CONTINUOUSLY > THIS FOR TIME  GREATER THAN KAETIM

DIVERT AIR

0466        00      0 KDLTMR           0 SEC    ENABLE AIR TO CONVERTER IF TIME SINCE  NEGATIVE DELTA MAP

LAST  EXCEEDED KDLAIR IS < KDLTMR WHILE IN OPEN LOOP

0467        15     21 KACMAP          18 KPA    DIVERT AIR IF MAP < THIS

0468        E0    224 KACMAPH         93 KPA    DIVERT AIR IF MAP > THIS AND  MPH > KMPHDVT

0469        3E     62 KMPHDVT         62 MPH    DIVERT AIR IF FILTMPH > THIS AND  MAP > KACMAPH

046A        00      0 KDLAIR           0 KPA    ENABLE AIR TO CONVERTER IF IN OPEN LOOP AND 100 MSEC DROP IN

MAP IS < THIS

046B        4B     75 KDVTCLD         16 DEG C  DIVERT AIR IF COOLANT < THIS

046C        96    150 KOLDVTM         15 SEC    DIVERT IF IN OPEN LOOP FOR TIME >= THIS AFTER HAVING BEEN

CLOSED LOOP

MALFS

046D        FF    255 KKMASK1   %11111111        MASK FOR MALFFLG1

046E        BF    191 KKMASK2   %10111111        MASK FOR MALFFLG2

046F        F6    246 KKMASK3    %11110110        MASK FOR MALFFLG3

0470        42     66 KKMASK4     %01000010        MASK FOR MALFFLG4

With VATS Enabled     0470        62     66 KKMASK4     %01100010        MASK FOR MALFFLG4

0471        00      0 KKMASK5       %00000000        MASK FOR MALFFLG5

0472        12     18 F9MALFN        $12        TABLE OUTPUT = 2 DIGIT BCD NUMBERS

MALF WORD 1

0473        12     18                $12        B7 - CODE 12 - NO REFERENCE PULSES

0474        13     19                $13        B6 - CODE 13 - OXYGEN SENSOR

0475        14     20                $14        B5 - CODE 14 - COOLANT SENSOR HIGH TEMP

0476        15     21                $15        B4 - CODE 15 - COOLANT SENSOR LOW TEMP

0477        21     33                $21        B3 - CODE 21 - THROTTLE POSITION SENSOR HI

0478        22     34                $22        B2 - CODE 22 - THROTTLE POSITION SENSOR LOW

0479        23     35                $23        B1 - CODE 23 - LOW MANIFOLD AIR TEMPERATURE

047A        24     36                $24        B0 - CODE 24 - VEHICLE SPEED SENSOR LOW

MALF WORD 2

047B        25     37                $25        B7 - CODE 25   HI MANIFOLD AIR TEMPERATURE

047C        31     49                $31        B6 - CODE 31   FUEL INJECTOR MALF

047D        32     50                $32        B5 - CODE 32   EVRV/ EGR

047E        33     51                $33        B4 - CODE 33   MAP SENSOR HIGH

047F        34     52                $34        B3 - CODE 34   MAP SENSOR LOW

0480        35     53                $35        B2 - CODE 35   IAC ERROR

0481        41     65                $41        B1 - CODE 41 CYLINDER SELECT ERROR

0482        42     66                $42        B0 - CODE 42   EST MONITOR ERROR

MALF WORD 3

0483        43     67                $43        B7 - CODE 43   ESC FAILURE

0484        44     68                $44        B6 - CODE 44   OXYGEN SENSOR LEAN

0485        45     69                $45        B5 - CODE 45   OXYGEN SENSOR RICH

0486        51     81                $51        B4 - CODE 51   PROM ERROR

0487        26     38                $26        B3 - CODE 26

0488        53     83                $53        B2 - CODE 53   HIGH BATTERY VOLTAGE

0489        54     84                $54        B1 - CODE 54   FUEL PUMP VOLTAGE LOW

048A        55     85                $55        B0 - CODE 55   ADU ERROR

MALF WORD 4

048B        56     86                $56        B7 - CODE 56

048C        61     97                $61        B6 - CODE 61 DEGRADED O2 SENSOR

048D        46     70                $46        B5 - CODE 46   VATS FAILED

048E        62     98                $62        B4 - CODE 62 TRANSMISSION GEAR SWITCH FAIL

048F        64    100                $64        B3 - CODE 64

0490        65    101                $65        B2 - CODE 65

0491        66    102                $66        B1 - CODE 66   A/C PRESSURE SENSOR FAIL

0492        67    103                $67        B0 - CODE 67

MALF WORD 5

0493        68    104                $68        B7 - CODE 68

0494        69    105                $69        B6 - CODE 69

0495        71    113                $71        B5 - CODE 71

0496        72    114                $72        B4 - CODE 72

0497        73    115                $73        B3 - CODE 73

0498        74    116                $74        B2 - CODE 74

0499        75    117                $75        B1 - CODE 75

049A        76    118                $76        B0 - CODE 76

049B        32     50 KKNOMALF        50        NO. OF SUCCESSIVE POWER UPS WITH NO MALFS,N.D.

049C        0A     10 KMCNT1           1 SEC      MALFUNCTION LOGGING FILTER CONSTANT 1

049D        14     20 KMCNT2            2 SEC     MALFUNCTION LOGGING FILTER CONSTANT 2

049E        64    100 KMCNT3           10 SEC    MALFUNCTION LOGGING FILTER CONSTANT 3

049F        78    120 KMCNT4           12 SEC    MALFUNCTION LOGGING FILTER CONSTANT 4

04A0        3C     60 KKO2ENBL        120 SEC   TIME SINCE RUN ENABLE

04A1        4F     79 KKO2LOW          0.35 VOLTS       O2 SENSOR LOW LIMIT

04A2        7C    124 KKO2HIGH        0.55 VOLTS      O2 SENSOR HIGH LIMIT

04A3        0A     10 KKO2LOD          4 %           THROTTLE POSITION LIMIT

04A4        FE    254 KKO2OLTM      25.4 SEC    TIME LIMIT

04A5        14     20 KK13DMN1         2 SEC      MALF 13 DFCO INHIBIT TIME (MINIMUM)

04A6        1E     30 KK13DMX1         3 SEC      MALF 13 DFCO INHIBIT TIME (MAXIMUM)

04A7        A7    167 KKDIAGWM       85 DEG C   COOLANT THRESHOLD

04A8      0014     20 KKETMPTH       20 SEC       TIME SINCE RUN ENABLE

04AA        E9    233 KKCTMPHI       135 DEG C   COOLANT HIGH LIMIT

04AB      0001      1 KKETMPTM       1 SEC           TIME SINCE RUN ENABLE

04AD        02      2 KKCTMPLO       -38.5 DEG C   COOLANT LOW LIMIT

04AE        A7    167 KKTCDF          85 DEG C        LONG TERM DEFAULT COOLANT READING

04AF        6B    107 KKTCDF1        40 DEG C       INITIAL DEFAULT COOLANT READING

04B0        44     68 KKFLOM21       17 GM/SEC     IF AIR FLOW >= THIS DISABLE M21A

04B1        BF    191 KKTA21           191                  'AD CNTS' THROTTLE POLIMIT

04B2        64    100 KK21TIM         10 SEC            TIME LIMIT

04B3        0A     10 KKTA22            10                    'AD CNTS' THROTTLE POLIMIT

04B4        03      3 KKVSPDK          3 MPH             VEHICLE SPEED LIMIT

04B5        58     88 KKVRPMLA       2200 RPM      ENGINE SPEED LOWER LIMIT

04B6        B0    176 KKVRPMHA      4400 RPM     ENGINE SPEED UPPER LIMIT

04B7        35     53 KK24MAP           30 KPA          MAP LOAD LIMIT

04B8        03      3 KKVST                 3 SEC           TIME LIMIT

04B9        FE    254 KK23TMPA       254 CNTS      IF MAT A/D CNTS > THIS ENABLE MALF 23

04BA        64    100 KK23TIME        10 SEC           TIME THRESHOLD FOR MALF 23

04BB        06      6 KKM23SUC        -35.5 DEG C    IF START UP COOLANT <= THIS, AND S.U. COOLANT TEST

ENABLED, DISABLE MALF 23

04BC      0100    256 KKTM2325       256 SEC         IF ENGINE RUN TIME <= THIS DISABLE MALF 23 AND MALF 25

04BE        01      1 KKMATMPH         1 MPH            IF MPH > THIS DO M25, ELSE M23

04BF        57     87 KKMATDEF        25 DEGC        DEFAULT MAT IF MALF 23 OR 25

04C0        02      2 KK25TMPA         2 CNTS           IF MAT A/D CNTS  < THIS ENABLE MALF 25

04C1        02      2 KK25TIME          0.2 SEC           TIME THRESHOLD FOR MALF 25

04C2        60     96 KM32BARO        75 KPA           BARO THRESHOLD TO ENABLE TEST

04C3        0A     10 KKEGRTIM        10 SEC           1 SEC RESOLUTION, DIAG. CYCLE TIME

04C4        38     56 KKEGRLLR        1400 RPM       LOW RPM DISABLE (NTRPMX)

04C5        48     72 KKEGRHLR        1800 RPM      HIGH RPM DISABLE (NTRPMX)

04C6        38     56 KKEGRLLV         62.5 KPA        LOW LOAD DISABLE (NVACLD)

04C7        B0    176 KKEGRHLV       25 KPA           HIGH LOAD DISABLE (NVACLD)

04C8        18     24 KKMAPHI           27.5 KPA         HI MAP DISABLE (NMAPLDAL)

04C9        02      2 KKMAPCH           0.7 KPA          DELTA MAP DISABLE (ADMAP)

04CA        00      0 KKEGRLLT         0 %                  LOW TPS DISABLE

04CB        04      4 KKEGRHLT        1.6 %               HIGH TPS DISABLE

04CC        04      4 KKRPM32           100 RPM        DELTA RPM DISABLE (NTRPMX)

04CD        0A     10 KKEGRDLT        1 SEC           EGR M32 DELAY TIMER

04CE        02      2 KKEGRDFA         2 N                IS FAIL COUNTER GREATER THAN THIS VALUE

#***************************************

# FV1                                  *

# TABLE OF OF DELTA MAP VALUES FOR M32 *

# VALVE 1 TEST AS A FUNCTION OF BARO   *

#***************************************

* -------------------------------------------------------------------

04CF        01      1                 0.4 KPA       65    KPA-BARO

04D0        03      3                  1 KPA       85

04D1        05      5                  2 KPA      105

#***************************************

# FV2                                  *

# TABLE OF OF DELTA MAP VALUES FOR M32 *

# VALVE 2 TEST AS A FUNCTION OF BARO   *

#***************************************

* -------------------------------------------------------------------

04D2        07      7                 2.6 KPA       65    KPA-BARO

04D3        0A     10                3.7 KPA       85

04D4        0D     13                4.8 KPA      105

#***************************************

# FV3                                  *

# TABLE OF OF DELTA MAP VALUES FOR M32 *

# VALVE 3 TEST AS A FUNCTION OF BARO   *

#***************************************

* -------------------------------------------------------------------

04D5        0C     12 FV3            4.4 KPA       65    KPA-BARO

04D6        0F     15                5.5 KPA       85

04D7        14     20                7.4 KPA      105

04D8        0A     10 KK32TIME         1 SEC    EGR M32 TIME ADMAP TEST IS RUN

04D9        26     38 KKEGRMPH        38 MPH    VEHICLE SPEED THRESHOLD TO ENABLE TEST

04DA        02      2 KKEGRDEC         2 N      FAIL COUNTER DECREMENT RATE

04DB        00      0 KKEGRSPK         0 DEG    EGR DIAGNOSTIC SPARK RETARD

04DC        20     32 KKBPWFRZ     $0.66 SEC    DONT UPDATE BPW IF M32 DONE FOR LESS THAN THIS

04DD        08      8 KKTPSHI          3 %      HI TPS VALUE TO DISABLE M32 FUEL COMPENSATION

04DE        24     36 KKRPMHI        900 RPM    LO RPM VALUE TO DISABLE M32 FUEL COMPENSATION

04DF        04      4 KKTA33         1.6 %      THROTTLE POSITION LIMIT

04E0        0F     15 KKTA33A          6 %      THROTTLE POSITION LIMIT

04E1        B4    180 KK33NV         180         'RPM/MPH' N/V RATIO INDICATING 1ST GEAR

04E2        AC    172 KKPM33          74 KPA    MAP LIMIT

04E3        C6    198 KKPMAC33      83.4 KPA    MAP LIMIT WITH A/C ON

04E4        30     48 KK33TIM        4.8 SEC    TIME LIMIT

04E5        07      7 KKPM34          13 KPA    MAP LIMIT

04E6        18     24 KKES34A        600 RPM    ENGINE SPEED LIMIT

04E7        12     18 KK34TIM       0.22 SEC    TIME LIMIT

04E8        33     51 KKTA34          20 %      THROTTLE POSITION LIMIT

04E9        62     98 KKPMDF        46.5 KPA    DEFAULT MAP READING FOR ENGINE NOT RUNNING

04EA        3D     61 KKA1          0.95 COEF   MAP DEFAULT COEF IF TPS <=F69

04EB        1D     29 KKA2          0.45 COEF   MAP DEFAULT COEF IF TPS >F69

04EC        10     16 KKPMACON         6 KPA    A/C LOAD ADJUSTMENT TO MAP DEFAULT COMPUTATION

04ED        10     16 KKDRMAP          6 KPA    P/N OFF DELTA MAP DEFAULT VALUE

04EE        40     64 KDMPINTR         4 FACTOR FACTOR FOR DECREASING C/L INT DELAY

DURING DEFAULT MAP OPERATION

MALF 35 PARAMETERS (IAC ERROR TOO HIGH)

04EF        14     20 KKIADIAG       250 RPM    IF |DESIRED RPM - ACTUAL RPM| IS > THIS

FOR KK35TIM THEN SET MALF 35

04F0        FE    254 KK35TIM       50.8 SEC    MALF 35 TIME THRESHOLD

MALF 41 PARAMETERS (CYLINDER SELECT)

04F1        08      8 KK41MODE         8 MODE   FUEL DELIVERY MODE

*     $18=TBI

*     $10=4 CYL PFI

*     $08=6 CYL PFI

*     $00=8 CYL PFI

#***************************************

# F69                                  *

# TABLE OF DEFAULT MAP COMPONENTS      *

#***************************************

* -------------------------------------------------------------------

04F2        08      8                   8        USE 9 VALUE TABLE

04F3        71    113                113 COUNTS     0    RPM-SPEED

04F4        42     66                 66 COUNTS   800

04F5        28     40                 40 COUNTS  1600

04F6        12     18                 18 COUNTS  2400

04F7        04      4                  4 COUNTS  3200

04F8        00      0                  0 COUNTS  4000

04F9        00      0                  0 COUNTS  4800

04FA        00      0                  0 COUNTS  5600

04FB        00      0                  0 COUNTS  6400

#***************************************

# F69TH                                *

# TABLE OF NTPSLD THRESHOLDS FOR SLOPE *

# CHANGE IN CALCULATION OF DEFAULT MAP *

#***************************************

* -------------------------------------------------------------------

04FC        08      8                 8        USE 9 VALUE TABLE

04FD        FF    255               99.6 % TPS      0    RPM-SPEED

04FE        98    152               59.4 % TPS    800

04FF        68    104               40.6 % TPS   1600

0500        5F     95                 37 % TPS   2400

0501        5A     90                 35 % TPS   3200

0502        5A     90                 35 % TPS   4000

0503        64    100                 39 % TPS   4800

0504        6E    110                 43 % TPS   5600

0505        78    120                 47 % TPS   6400

#***************************************

# F70                                  *

# TABLE OF DEFAULT MAP COMPONENTS      *

# AS A FUNCTION OF IAC MOTOR POSITION  *

#***************************************

* -------------------------------------------------------------------

0506        00      0                   0 COUNTS     0    IAC-STEPS

0507        14     20                 20 COUNTS    32

0508        28     40                 40 COUNTS    64

0509        34     52                 52 COUNTS    96

050A        40     64                 64 COUNTS   128

#***************************************

# F71                                  *

# TABLE OF FACTORS (0-1) USED TO       *

# MODIFY THE RESULT OF THE F70 TABLE   *

#***************************************

* -------------------------------------------------------------------

050B        08      8                 8        USE 9 VALUE TABLE

050C        FF    255              0.996 % TPS      0    RPM-SPEED

050D        FF    255              0.996 % TPS    800

050E        E0    224              0.875 % TPS   1600

050F        95    149              0.582 % TPS   2400

0510        64    100               0.39 % TPS   3200

0511        37     55              0.215 % TPS   4000

0512        00      0                  0 % TPS   4800

0513        00      0                  0 % TPS   5600

0514        00      0                  0 % TPS   6400

#*******************************************

# F78    TABLE OF NTPSLD TERMS TO GET      *

#        TPS DEFAULT VALUE AS A F(RPM)     *

#   TABLE VALUE = % FULL THROTTLE * 2.56   *

#*******************************************

* -------------------------------------------------------------------

0515        00      0                   0 %       1000    RPM-SPEED

0516        30     48               18.8 %       1800

0517        47     71               27.7 %       2600

0518        54     84                 33 %       3400

0519        5C     92                 36 %       4200

051A        61     97                 38 %       5000

051B        08      8 KK42RMA        200 RPM    ENGINE SPEED LIMIT

051C        08      8 KK42ACT          8 CNTS   NUMBER OF ESTFBCTR COUNTS FOR A MALF 42A

A SPARK COUNTER WILL BE  INCREMENTED ONCE PER REF.  PULSE

IF THERE IS ??????    N THE EST FEEDBACK LINE.  BEFORE EST IS

ENABLED AT START-UP  THERE SHOULD BE NO PULSES ON THE  EST

MONITOR .  IF MORE THAN KK42ACT  COUNTS OCCUR BETWEEN

INITIALIZATION AND THE MALF 42A LOGIC THEN MALF 42 WILL BE

FLAGGED AND AND EST WON'T BE ENABLED.

051D        10     16 KK42PLWD     0.001 SEC    SPARK PULSE WIDTH THRESHOLD

THE EST FEEDBACK SIGNAL GOES

INTO A PULSE INTEGRATOR

IN THE ECU.  THIS MEANS THAT

AS LONG AS THE SIGNAL

IS HIGH, THE PA2CTR REGISTER

INCREMENTS AT A 16KHZ

RATE.  WHEN THE SIGNAL GOES

LOW, THE COUNTER STOPS.

TO DISCRIMINATE BETWEEN NOISE

AND REAL EST PULSE THE

PA2CTR IS READ AFTER EVERY

REFERENCE PULSE AND THE

DIFFERENCE IS CALCULATED

BETWEEN THE CURRENT AND OLD

VALUE.  THIS DIFFERENCE MUST

BE GREATER THAN KK42PLWD

IN ORDER FOR THE PULSE COUNTER

(ESTFBCTR) TO INCREMENT.

051E        F9    249 KKESCP        3.89 SEC    ENABLE M43 IF ESC INPUT LO TIME OVER

3.9 SEC INTERVAL > THIS TIME

051F        55     85 KKRTDF          15 DEG    DEFAULT ESC RETARD WHEN M43 IS PRESENT

0520        64    100 KKM43ATM        10 SEC    IF ESC LINE OUT OF RANGE THIS LONG...

...THEN SET A MALF 43A

0521        E6    230 KKM43ATH       230 CTS    IF ESC A/D COUNTS > THIS FOR KKM43ATM

TIME THEN SET A MALF 43A

0522        20     32 KKM43ATL        32 CTS    IF ESC A/D COUNTS < THIS FOR KKM43ATM

TIME THEN SET A MALF 43A

0523        73    115 KKM43VOL      11.5 VOLTS  M43A NOT ENABLED UNLESS BAT VOLT

EXCEEDS THIS CAL

0524        2D     45 KKO2MIN        0.2 VOLTS  O2 SENSOR LOW LIMIT

0525        01      1 KFLTO2DG     0.004 COEF   100 MS O2 SENSOR FILTER COEF

(INPUT TO 100 MSEC O2 FILTER

IS 6.25

MSEC FILTERED VALUE)

0526        42     66 KK44TIMS        66 SEC    MALF 44 SLOW INTEGRATOR RATE TIME THRESHOLD

0527        40     64 KK44TIMF        64 SEC    MALF 44 FAST INTEGRATOR RATE TIME THRESHOLD

0528        9E    158 KKO2MAX        0.7 VOLTS  IF O2 VOLTAGE >= THIS DISABLE M45

0529        A9    169 KKO2MAXI     0.748 VOLTS  IF O2 VOLTAGE >= THIS DISABLE M45

(WHEN IDLING)

052A        32     50 KK45TIM         50 SEC    IF M45 CRITERIA MET FOR TIME > THIS

STORE CODE

052B        08      8 KK45TPSH         3 %      ENABLE MALF 45 IF TPS > THIS

052C        73    115 KK45TPSL        45 %      ENABLE MALF 45 IF TPS < THIS

052D        05      5 KKNOVATS       0.5 SEC    TIME ALLOWED TO PASS VATS TEST

MALF 54 PARAMETERS (PPSW LOW)

052E        04      4 KKFULPTM       0.4 SECS   IF TIME> THIS,VOLTAGE NEVER HI, SETM54

DO NOT SET TO 255 (ELSE MALF

CAN'T BE SET)

052F        28     40 KKPPSWV          4 VOLTS  IF PPSW < THIS, SET M54

0530        28     40 KK54BVLT         4 VOLTS  IF PPSW < THIS AND ENG RUNNING, SET M54

0531        5A     90 KK54IGNV         9 VOLTS  IF IGN < THIS SKIP M54 CHECK

0532        64    100 KK53TIM         10 SEC    TIME FOR MALF 53 CONDITION BEFORE FLAGGING

MALF 61 PARAMETERS (OXYGEN SENSOR DEGRADED)

0533        A7    167 KKM61CLT        85 DEG C  BYPASS MALF 61 IF COOLANT <= THIS

0534        88    136 KKVLT61H       0.6 VOLTS  IF VOLTAGE > THIS AND IN DFCO, BEGIN M61 TEST

0535        44     68 KKVLT61L       0.3 VOLTS  IF VOLTAGE < THIS M61 TEST PASSED

0536        02      2 KKM61CT1         2 CTS    NUMBER OF TESTS PASSED BEFORE SETTING PASSED TEST FLAG

0537        02      2 KKM61CT2         2 CTS    NUMBER OF TESTS FAILED BEFORE SETTING FAILED TEST FLAG

0538        78    120 KKM61TMR       1.5 SEC    M61 INDIVIDUAL TEST TIMER THRESHOLD

0539        26     38 KK61MAT      -11.5 DEGC   BYPASS MALF 61 IF MAT <= THIS MALF 62 PARAMETERS

(TRANSMISSION GEAR SWITCH FAILURE)

053A        00      0 KK62MPH1         0 MPH    SWITCHES SHOULD BE CLOSED IF MPH < THIS (LO GEAR)

053B        FF    255 KK62MPH2       255 MPH    MPH MUST BE > THIS FOR MALF 62 LOGGING

053C        FF    255 KK62TPS       99.6 %      TPS MUST BE > THIS FOR MALF 62 LOGGING

053D        FF    255 KK62RPM       6375 RPM    RPM MUST BE > THIS FOR MALF 62 LOGGING

053E        FF    255 KK62TIME      25.5 SEC    MALF 62 DELAY BEFORE LOGGING AFTER CONDITIONS MET

MALF 66 PARAMETERS (A/C PRESSURE SENSOR FAILURE)

053F        FC    252 KKACMAXV       252 CTS    A/C PRESSURE SENSOR FAILURE UPPER THRESHOLD

0540        05      5 KKACMINV         5 CTS        A/C PRESSURE SENSOR FAILURE LOWER THRESHOLD

0541        32     50 KKACTIM          5 SEC      MALF 66 DELAY BEFORE LOGGING AFTER CONDITIONS MET

0542        14     20 KLCHBIAS         7 DEG       LAUNCH SPARK BIAS

0543        D0    208 KLCHAR       0.813 MULT   LAUNCH MODE ESC ATTACK RATE MULTIPLIER

0544        0A     10 KLCHVS           3 MPH    LAUNCH MODE VEHICLE SPEED THRESHOLD

0545        03      3 KDTPLCH          1 %TPS   POSITIVE DELTA TPS LAUNCH THRESHOLD

0546        03      3 KDLPLCH          3 UNITS  POSITIVE DELTA LOAD LAUNCH THRESHOLD

0547        05      5 KLCLD2           5 UNITS  POSITIVE DELTA LOAD LAUNCH THRESHOLD FOR LEAVING LAUNCH

MODE SET

0548        50     80 KLCHTM           1 SEC    LAUNCH MODE TIME

0549        03      3 KNDTPLH          1 %TPS         NEGATIVE D-TPS LAUNCH MODE OFF THRESH

054A        40     64 KLHEGF        0.25 FACTOR    LAUNCH EGR DUTY CYCLE GAIN FACTOR

## END OF MALFS

###################################################

#  F115                                           #

#  LAUNCH MODE SPARK ADVANCE VS. RPM AND MAP      #

#   N=E*256/90                                    #

###################################################

LAUNCH MODE SPARK ADVANCE

* -------------------------------------------------------------------

054B        00      0                   0        R MIN; R = NTRPM

054C        90    144                144        Q MIN; Q = LOAD (MAP LOAD)

054D        08      8                  8        R NUM

* --------------------------------------------- SPEED        400 RPM

054E        14     20                  7 DEG       65    KPA-MAP

054F        14     20                  7 DEG       70

0550        14     20                  7 DEG       75

0551        14     20                  7 DEG       80

0552        14     20                  7 DEG       85

0553        14     20                  7 DEG       90

0554        14     20                  7 DEG       95

0555        14     20                  7 DEG      100

* --------------------------------------------- SPEED        600 RPM

0556        1A     26                  9 DEG       65    KPA-MAP

0557        22     34                 12 DEG       70

0558        22     34                 12 DEG       75

0559        1A     26                  9 DEG       80

055A        14     20                  7 DEG       85

055B        14     20                  7 DEG       90

055C        14     20                  7 DEG       95

055D        14     20                  7 DEG      100

* --------------------------------------------- SPEED        800 RPM

055E        1A     26                  9 DEG       65    KPA-MAP

055F        22     34                 12 DEG       70

0560        22     34                 12 DEG       75

0561        1A     26                  9 DEG       80

0562        14     20                  7 DEG       85

0563        14     20                  7 DEG       90

0564        14     20                  7 DEG       95

0565        14     20                  7 DEG      100

* --------------------------------------------- SPEED       1000 RPM

0566        14     20                  7 DEG       65    KPA-MAP

0567        17     23                  8 DEG       70

0568        25     37                 13 DEG       75

0569        22     34                 12 DEG       80

056A        22     34                 12 DEG       85

056B        1F     31                 11 DEG       90

056C        17     23                  8 DEG       95

056D        17     23                  8 DEG      100

* --------------------------------------------- SPEED       1200 RPM

056E        14     20                  7 DEG       65    KPA-MAP

056F        1A     26                  9 DEG       70

0570        22     34                 12 DEG       75

0571        28     40                 14 DEG       80

0572        22     34                 12 DEG       85

0573        22     34                 12 DEG       90

0574        17     23                  8 DEG       95

0575        17     23                  8 DEG      100

* --------------------------------------------- SPEED       1400 RPM

0576        14     20                  7 DEG       65    KPA-MAP

0577        1A     26                  9 DEG       70

0578        22     34                 12 DEG       75

0579        24     36               12.7 DEG       80

057A        28     40                 14 DEG       85

057B        22     34                 12 DEG       90

057C        17     23                  8 DEG       95

057D        17     23                  8 DEG      100

* --------------------------------------------- SPEED       1600 RPM

057E        14     20                  7 DEG       65    KPA-MAP

057F        1A     26                  9 DEG       70

0580        22     34                 12 DEG       75

0581        24     36               12.7 DEG       80

0582        28     40                 14 DEG       85

0583        22     34                 12 DEG       90

0584        17     23                  8 DEG       95

0585        17     23                  8 DEG      100

* --------------------------------------------- SPEED       1800 RPM

0586        17     23                  8 DEG       65    KPA-MAP

0587        1A     26                  9 DEG       70

0588        22     34                 12 DEG       75

0589        24     36               12.7 DEG       80

058A        28     40                 14 DEG       85

058B        22     34                 12 DEG       90

058C        17     23                  8 DEG       95

058D        17     23                  8 DEG      100

* --------------------------------------------- SPEED       2000 RPM

058E        17     23                  8 DEG       65    KPA-MAP

058F        1A     26                  9 DEG       70

0590        22     34                 12 DEG       75

0591        24     36               12.7 DEG       80

0592        28     40                 14 DEG       85

0593        22     34                 12 DEG       90

0594        17     23                  8 DEG       95

0595        17     23                  8 DEG      100

0596      0000      0 KNT1         $0000 MULT   (1ST GEAR)

0598      0000      0 KNT2         $0000 MULT   (2ND GEAR)

059A      0000      0 KNT3         $0000 MULT   (3RD GEAR)

059C      0000      0 KNT4         $0000 MULT   (4TH GEAR)

059E        00      0 KSABLEND         0 %      PERCENT OF SADELTA BLENDED EACH LOOP

Auto Transmission

####################################################################

#  F115STM                                                         #

#  STALL TORQUE MANAGEMENT SPARK ADVANCE VS. RPM AND RATIO COUNTS  #

#  RATIO IS TRANSMISSION TURBINE RPM/ENGINE RPM                    #

#   N=E*256/90                                                     #

####################################################################

* -------------------------------------------------------------------

059F        30     48 F115STM1        48        R MIN; R = NTRPMX

05A0        00      0                  0        Q MIN; Q = RATIO

05A1        09      9                  9        R NUM

* --------------------------------------------- SPEED       1200 RPM

05A2        00      0                  0 DEG      0.0 TS/RPM-RATIO

05A3        00      0                  0 DEG    0.0625

05A4        00      0                  0 DEG    0.125

05A5        00      0                  0 DEG    0.1875

05A6        00      0                  0 DEG     0.25

05A7        00      0                  0 DEG    0.3125

05A8        00      0                  0 DEG    0.375

05A9        00      0                  0 DEG    0.4375

05AA        00      0                  0 DEG      0.5

* --------------------------------------------- SPEED       1600 RPM

05AB        00      0                  0 DEG      0.0 TS/RPM-RATIO

05AC        00      0                  0 DEG    0.0625

05AD        00      0                  0 DEG    0.125

05AE        00      0                  0 DEG    0.1875

05AF        00      0                  0 DEG     0.25

05B0        00      0                  0 DEG    0.3125

05B1        00      0                  0 DEG    0.375

05B2        00      0                  0 DEG    0.4375

05B3        00      0                  0 DEG      0.5

* --------------------------------------------- SPEED       2000 RPM

05B4        00      0                  0 DEG      0.0 TS/RPM-RATIO

05B5        00      0                  0 DEG    0.0625

05B6        00      0                  0 DEG    0.125

05B7        00      0                  0 DEG    0.1875

05B8        00      0                  0 DEG     0.25

05B9        00      0                  0 DEG    0.3125

05BA        00      0                  0 DEG    0.375

05BB        00      0                  0 DEG    0.4375

05BC        00      0                  0 DEG      0.5

* --------------------------------------------- SPEED       2400 RPM

05BD        00      0                  0 DEG      0.0 TS/RPM-RATIO

05BE        00      0                  0 DEG    0.0625

05BF        00      0                  0 DEG    0.125

05C0        00      0                  0 DEG    0.1875

05C1        00      0                  0 DEG     0.25

05C2        00      0                  0 DEG    0.3125

05C3        00      0                  0 DEG    0.375

05C4        00      0                  0 DEG    0.4375

05C5        00      0                  0 DEG      0.5

* --------------------------------------------- SPEED       2800 RPM

05C6        00      0                  0 DEG      0.0 TS/RPM-RATIO

05C7        00      0                  0 DEG    0.0625

05C8        00      0                  0 DEG    0.125

05C9        00      0                  0 DEG    0.1875

05CA        00      0                  0 DEG     0.25

05CB        00      0                  0 DEG    0.3125

05CC        00      0                  0 DEG    0.375

05CD        00      0                  0 DEG    0.4375

05CE        00      0                  0 DEG      0.5

* --------------------------------------------- SPEED       3200 RPM

05CF        00      0                  0 DEG      0.0 TS/RPM-RATIO

05D0        00      0                  0 DEG    0.0625

05D1        00      0                  0 DEG    0.125

05D2        00      0                  0 DEG    0.1875

05D3        00      0                  0 DEG     0.25

05D4        00      0                  0 DEG    0.3125

05D5        00      0                  0 DEG    0.375

05D6        00      0                  0 DEG    0.4375

05D7        00      0                  0 DEG      0.5

* --------------------------------------------- SPEED       3600 RPM

05D8        00      0                  0 DEG      0.0 TS/RPM-RATIO

05D9        00      0                  0 DEG    0.0625

05DA        00      0                  0 DEG    0.125

05DB        00      0                  0 DEG    0.1875

05DC        00      0                  0 DEG     0.25

05DD        00      0                  0 DEG    0.3125

05DE        00      0                  0 DEG    0.375

05DF        00      0                  0 DEG    0.4375

05E0        00      0                  0 DEG      0.5

* --------------------------------------------- SPEED       4000 RPM

05E1        00      0                  0 DEG      0.0 TS/RPM-RATIO

05E2        00      0                  0 DEG    0.0625

05E3        00      0                  0 DEG    0.125

05E4        00      0                  0 DEG    0.1875

05E5        00      0                  0 DEG     0.25

05E6        00      0                  0 DEG    0.3125

05E7        00      0                  0 DEG    0.375

05E8        00      0                  0 DEG    0.4375

05E9        00      0                  0 DEG      0.5

* --------------------------------------------- SPEED       4400 RPM

05EA        00      0                  0 DEG      0.0 TS/RPM-RATIO

05EB        00      0                  0 DEG    0.0625

05EC        00      0                  0 DEG    0.125

05ED        00      0                  0 DEG    0.1875

05EE        00      0                  0 DEG     0.25

05EF        00      0                  0 DEG    0.3125

05F0        00      0                  0 DEG    0.375

05F1        00      0                  0 DEG    0.4375

05F2        00      0                  0 DEG      0.5

####################################################################

#  F115STMB                                                        #

#  STALL TORQUE MANAGEMENT SPARK ADVANCE VS. RPM AND MAP           #

#   N=E*256/90                                                     #

####################################################################

* -------------------------------------------------------------------

05F3        30     48 F115STMB        48        R MIN; R = NTRPMX

05F4        90    144                144        Q MIN; Q = NMAPLD

05F5        08      8                  8        R NUM

* --------------------------------------------- SPEED       1200 RPM

05F6        00      0                  0 DEG       65    KPA-MAP

05F7        00      0                  0 DEG       70

05F8        00      0                  0 DEG       75

05F9        00      0                  0 DEG       80

05FA        00      0                  0 DEG       85

05FB        00      0                  0 DEG       90

05FC        00      0                  0 DEG       95

05FD        00      0                  0 DEG      100

* --------------------------------------------- SPEED       1600 RPM

05FE        00      0                  0 DEG       65    KPA-MAP

05FF        00      0                  0 DEG       70

0600        00      0                  0 DEG       75

0601        00      0                  0 DEG       80

0602        00      0                  0 DEG       85

0603        00      0                  0 DEG       90

0604        00      0                  0 DEG       95

0605        00      0                  0 DEG      100

* --------------------------------------------- SPEED       2000 RPM

0606        00      0                  0 DEG       65    KPA-MAP

0607        00      0                  0 DEG       70

0608        00      0                  0 DEG       75

0609        00      0                  0 DEG       80

060A        00      0                  0 DEG       85

060B        00      0                  0 DEG       90

060C        00      0                  0 DEG       95

060D        00      0                  0 DEG      100

* --------------------------------------------- SPEED       2400 RPM

060E        00      0                  0 DEG       65    KPA-MAP

060F        00      0                  0 DEG       70

0610        00      0                  0 DEG       75

0611        00      0                  0 DEG       80

0612        00      0                  0 DEG       85

0613        00      0                  0 DEG       90

0614        00      0                  0 DEG       95

0615        00      0                  0 DEG      100

* --------------------------------------------- SPEED       2800 RPM

0616        00      0                  0 DEG       65    KPA-MAP

0617        00      0                  0 DEG       70

0618        00      0                  0 DEG       75

0619        00      0                  0 DEG       80

061A        00      0                  0 DEG       85

061B        00      0                  0 DEG       90

061C        00      0                  0 DEG       95

061D        00      0                  0 DEG      100

* --------------------------------------------- SPEED       3200 RPM

061E        00      0                  0 DEG       65    KPA-MAP

061F        00      0                  0 DEG       70

0620        00      0                  0 DEG       75

0621        00      0                  0 DEG       80

0622        00      0                  0 DEG       85

0623        00      0                  0 DEG       90

0624        00      0                  0 DEG       95

0625        00      0                  0 DEG      100

* --------------------------------------------- SPEED       3600 RPM

0626        00      0                  0 DEG       65    KPA-MAP

0627        00      0                  0 DEG       70

0628        00      0                  0 DEG       75

0629        00      0                  0 DEG       80

062A        00      0                  0 DEG       85

062B        00      0                  0 DEG       90

062C        00      0                  0 DEG       95

062D        00      0                  0 DEG      100

* --------------------------------------------- SPEED       4000 RPM

062E        00      0                  0 DEG       65    KPA-MAP

062F        00      0                  0 DEG       70

0630        00      0                  0 DEG       75

0631        00      0                  0 DEG       80

0632        00      0                  0 DEG       85

0633        00      0                  0 DEG       90

0634        00      0                  0 DEG       95

0635        00      0                  0 DEG      100

* --------------------------------------------- SPEED       4400 RPM

0636        00      0                  0 DEG       65    KPA-MAP

0637        00      0                  0 DEG       70

0638        00      0                  0 DEG       75

0639        00      0                  0 DEG       80

063A        00      0                  0 DEG       85

063B        00      0                  0 DEG       90

063C        00      0                  0 DEG       95

063D        00      0                  0 DEG      100

063E        FF    255 KRPM12        6375 RPM    RPM THRESH FOR SPARK RETARD DURING 1-2 SHIFT

063F        FF    255 KTPS12        99.6 %      TPS THRESH FOR SPARK RETARD DURING 1-2 SHIFT

0640        FF    255 KRPM23        6375 RPM    RPM THRESH FOR SPARK RETARD DURING 2-3 SHIFT

0641        FF    255 KTPS23        99.6 %      TPS THRESH FOR SPARK RETARD DURING 2-3 SHIFT

*################################################

*   F12DELTA TABLE: NEGATIVE RPM DELTA ENABLE   #

*                   THRESHOLD FOR SHIFT ENERGY  #

*                   SPARK RETARD, 1-2 UPSHIFT   #

*                            VS. RPM            #

*   TABLE VALUE = RPM DELTA                     #

*################################################

* -------------------------------------------------------------------

0642        00      0                 0 RPM     2800    RPM-SPEED

0643        00      0                  0 RPM     3200

0644        00      0                  0 RPM     3600

0645        00      0                  0 RPM     4000

0646        00      0                  0 RPM     4400

0647        00      0                  0 RPM     4800

0648        00      0                  0 RPM     5200

0649        00      0                  0 RPM     5600

064A        00      0                  0 RPM     6000

064B        00      0                  0 RPM     6400

*################################################

*   F12DELTB TABLE: NEGATIVE RPM DELTA EXIT     #

*                   THRESHOLD FOR SHIFT ENERGY  #

*                   SPARK RETARD, 1-2 UPSHIFT   #

*                            VS. RPM            #

*   TABLE VALUE = RPM DELTA                     #

*################################################

* -------------------------------------------------------------------

064C        00      0                  0 RPM     2800    RPM-SPEED

064D        00      0                  0 RPM     3200

064E        00      0                  0 RPM     3600

064F        00      0                  0 RPM     4000

0650        00      0                  0 RPM     4400

0651        00      0                  0 RPM     4800

0652        00      0                  0 RPM     5200

0653        00      0                  0 RPM     5600

0654        00      0                  0 RPM     6000

0655        00      0                  0 RPM     6400

*################################################

*   F1TIME12 TABLE: PRETIMER THRESHOLD BEFORE   #

*                   ALLOWING SHIFT ENERGY       #

*                   SPARK RETARD, 1-2 UPSHIFT   #

*                            VS. RPM            #

*   TABLE VALUE = SEC * 80                      #

*################################################

* -------------------------------------------------------------------

0656        00      0                  0 SEC     2800    RPM-SPEED

0657        00      0                  0 SEC     3200

0658        00      0                  0 SEC     3600

0659        00      0                  0 SEC     4000

065A        00      0                  0 SEC     4400

065B        00      0                  0 SEC     4800

065C        00      0                  0 SEC     5200

065D        00      0                  0 SEC     5600

065E        00      0                  0 SEC     6000

065F        00      0                  0 SEC     6400

*################################################

*   F2TIME12 TABLE: SHIFT ENERGY SPARK RETARD   #

*                   TIME DURING 1-2 UPSHIFT     #

*                            VS. RPM            #

*   TABLE VALUE = SEC * 80                      #

*################################################

* -------------------------------------------------------------------

0660        00      0                  0 SEC     2800    RPM-SPEED

0661        00      0                  0 SEC     3200

0662        00      0                  0 SEC     3600

0663        00      0                  0 SEC     4000

0664        00      0                  0 SEC     4400

0665        00      0                  0 SEC     4800

0666        00      0                  0 SEC     5200

0667        00      0                  0 SEC     5600

0668        00      0                  0 SEC     6000

0669        00      0                  0 SEC     6400

*################################################

*   F23DELTA TABLE: NEGATIVE RPM DELTA ENABLE   #

*                   THRESHOLD FOR SHIFT ENERGY  #

*                   SPARK RETARD, 2-3 UPSHIFT   #

*                            VS. RPM            #

*   TABLE VALUE = RPM DELTA                     #

*################################################

* -------------------------------------------------------------------

066A        00      0                 0 RPM     2800    RPM-SPEED

066B        00      0                  0 RPM     3200

066C        00      0                  0 RPM     3600

066D        00      0                  0 RPM     4000

066E        00      0                  0 RPM     4400

066F        00      0                  0 RPM     4800

0670        00      0                  0 RPM     5200

0671        00      0                  0 RPM     5600

0672        00      0                  0 RPM     6000

0673        00      0                  0 RPM     6400

*################################################

*   F23DELTB TABLE: NEGATIVE RPM DELTA EXIT     #

*                   THRESHOLD FOR SHIFT ENERGY  #

*                   SPARK RETARD, 2-3 UPSHIFT   #

*                            VS. RPM            #

*   TABLE VALUE = RPM DELTA                     #

*################################################

* -------------------------------------------------------------------

0674        00      0                  0 RPM     2800    RPM-SPEED

0675        00      0                  0 RPM     3200

0676        00      0                  0 RPM     3600

0677        00      0                  0 RPM     4000

0678        00      0                  0 RPM     4400

0679        00      0                  0 RPM     4800

067A        00      0                  0 RPM     5200

067B        00      0                  0 RPM     5600

067C        00      0                  0 RPM     6000

067D        00      0                  0 RPM     6400

*################################################

*   F1TIME23 TABLE: PRETIMER THRESHOLD BEFORE   #

*                   ALLOWING SHIFT ENERGY       #

*                   SPARK RETARD, 2-3 UPSHIFT   #

*                            VS. RPM            #

*   TABLE VALUE = SEC * 80                      #

*################################################

* -------------------------------------------------------------------

067E        00      0                 0 SEC     2800    RPM-SPEED

067F        00      0                  0 SEC     3200

0680        00      0                  0 SEC     3600

0681        00      0                  0 SEC     4000

0682        00      0                  0 SEC     4400

0683        00      0                  0 SEC     4800

0684        00      0                  0 SEC     5200

0685        00      0                  0 SEC     5600

0686        00      0                  0 SEC     6000

0687        00      0                  0 SEC     6400

*################################################

*   F2TIME23 TABLE: SHIFT ENERGY SPARK RETARD   #

*                   TIME DURING 2-3 UPSHIFT     #

*                            VS. RPM            #

*   TABLE VALUE = SEC * 80                      #

*################################################

* -------------------------------------------------------------------

0688        00      0                 0 SEC     2800    RPM-SPEED

0689        00      0                  0 SEC     3200

068A        00      0                  0 SEC     3600

068B        00      0                  0 SEC     4000

068C        00      0                  0 SEC     4400

068D        00      0                  0 SEC     4800

068E        00      0                  0 SEC     5200

068F        00      0                  0 SEC     5600

0690        00      0                  0 SEC     6000

0691        00      0                  0 SEC     6400

* -------------------------------------------------------------------

0692        00      0                 0 DEG     2800    RPM-SPEED

0693        00      0                  0 DEG     3200

0694        00      0                  0 DEG     3600

0695        00      0                  0 DEG     4000

0696        00      0                  0 DEG     4400

0697        00      0                  0 DEG     4800

0698        00      0                  0 DEG     5200

0699        00      0                  0 DEG     5600

069A        00      0                  0 DEG     6000

069B        00      0                  0 DEG     6400

#################################################

#  F241BAR TABLE                                #

# REDUCTION OF 1-2 SHIFT SPARK RETARD VS. BARO  #

# TABLE VALUE = DEG * 256/90                    #

#################################################

* -------------------------------------------------------------------

069C        00      0                  0 DEG       65    KPA-BARO

069D        00      0                  0 DEG       70

069E        00      0                  0 DEG       75

069F        00      0                  0 DEG       80

06A0        00      0                  0 DEG       85

06A1        00      0                  0 DEG       90

06A2        00      0                  0 DEG       95

06A3        00      0                  0 DEG      100

06A4        00      0                  0 DEG      105

* -------------------------------------------------------------------

06A5        00      0                  0 DEG     2800    RPM-SPEED

06A6        00      0                  0 DEG     3200

06A7        00      0                  0 DEG     3600

06A8        00      0                  0 DEG     4000

06A9        00      0                  0 DEG     4400

06AA        00      0                  0 DEG     4800

06AB        00      0                  0 DEG     5200

06AC        00      0                  0 DEG     5600

06AD        00      0                  0 DEG     6000

06AE        00      0                  0 DEG     6400

#################################################

#  F242BAR TABLE                                #

# REDUCTION OF 2-3 SHIFT SPARK RETARD VS. BARO  #

# TABLE VALUE = DEG * 256/90                    #

#################################################

* -------------------------------------------------------------------

06AF        00      0                  0 DEG       65    KPA-BARO

06B0        00      0                  0 DEG       70

06B1        00      0                  0 DEG       75

06B2        00      0                  0 DEG       80

06B3        00      0                  0 DEG       85

06B4        00      0                  0 DEG       90

06B5        00      0                  0 DEG       95

06B6        00      0                  0 DEG      100

06B7        00      0                  0 DEG      105

06B8        00      0 KPHMPH           0 MPH    IF HI MPH, IGNORE P/N SWITCH INPUT FOR ROCK

CYCLE & ICE ABUSE IN CASE OF FAULTY SWITCH

06B9        00      0 KPNTIMER         0 SEC    IF HI RPM, KEEP FUEL OFF FOR KPNTIMER

AFTER A TRANSITION FROM P/N TO  'IN GEAR'

06BA      FFFF  65535 KPNFLOFF     65535 RPM    RPM LIMIT BEFORE SHUTTING OFF FUEL

06BC      FFFE  65534 KPNFLON      65534 RPM    RPM LIMIT BEFORE TURNING FUEL BACK ON

06BE        00      0 KSTMTIME         0 SEC    TIME LIMIT FOR NO TURBINE SPEED PULSES RECEIVED

06BF        D4    212 KAFOPT1  %11010100 N      AIR FUEL OPTION FLAG WORD 1

VATS ENABLED   Bit 3    06BF        DC    212 KAFOPT1  %11011100 N      AIR FUEL OPTION FLAG WORD 1

BIT     DESCRIPTION

NO      BIT0    1 = BPW MOD; 0 = A/F MOD SELECTED

NO       BIT1    1 = CALCULATE EXECUTION TIME IN EACH BRANCH

YES      BIT2    1 = CALCULATE 'BPW AT IDLE' AS FUNCTION OF AIRFLO

VATS ENABLE BIT          NO     BIT3    1 = VATS ENABLED

YES     BIT4    1 = NORMALLY OPEN FAN #2 REQUEST

NO      BIT5    1 = MATDEG FOR F132 TABLE; 0 = INLETTMP

YES     BIT6    1 = MULTIPLY DE DELTA MAP TERM BY NTRPM

YES    BIT7    1 = MANUAL SHIFT LOGIC; 0 = AUTOMATIC TCC LOGIC

06C0        F1    241 KAFOPT2  %11110001 N      AIR FUEL OPTION FLAG WORD 2

BIT     DESCRIPTION

BIT0    1 = MALF CODE CLEAR BY DIAG GND OPT SELECTED

BIT1    1 = CLOSED LOOP CCP OPTION

BIT2    1 = ADDITIVE CLOSED LOOP CCP OPTION

BIT3    1 = F31 TRANSMISSION OPTION SELECTED

BIT4    1 = PREVENT 80HZ IAC OPERATION (EXCEPT IAC RESET)

BIT5    1 = IGNORE 3RD GEAR SWITCH IN MALF 62 DIAGNOSTIC

BIT6    1 = BYPASS EGR LEARNED OPENING POINT LOGIC

BIT7    1 = A/C PRESSURE TRANSDUCER PRESENT

06C1        82    130 KAFOPT3  %10000010 N      AIR FUEL OPTION FLAG WORD 3

BIT     DESCRIPTION

BIT0    1 = OPTION TO USE NMAPLDAL FOR F56 TABLE LOOKUP

BIT1    1 = MUST BE CLOSED LOOP TO ENABLE CCP

BIT2    1 = F CAR BLM CELL 2 ENTRANCE CRITERION SELECTED

BIT3    1 = OPTION TO USE NMAPLDAL FOR F49 TABLE LOOKUP

BIT4    1 = NORMALLY OPEN FAN #1 REQUEST

BIT5    1 = NORMALLY CLOSED CCP VALVE; 0 = NORMALLY OPEN

BIT6    1 = W-CAR A/C OPTION

BIT7    1 = USE FILTERED THROTTLE IN DECEL ENLEANMENT

06C2        E8    232 KAFOPT4  %11101000 N      AIR FUEL OPTION FLAG WORD 4

BIT     DESCRIPTION

BIT0    1 = RESET C/L INTEGRATOR ON BLM CELL CHANGE

BIT1    1 = 4TH GEAR REQUIRED FOR CONV OT SPARK ADVANCE

BIT2    1 = HOT START IDLE SPEED OFFSET DELTA DISABLED

BIT3    1 = DUAL RAM HUD DISPLAY OPTION SELECTED

BIT4    1 = TCC MUST BE LOCKED FOR CONV OT SPARK ADVANCE

BIT5   1 = RESET C/L INTEGRATOR ON ACCEL ENRICH (1ST PASS)

BIT6    1 = RPM DERIVATIVE SPARK CONTROL OPTION SELECTED

BIT7    1 = F1 SPARK TABLE, IDLE CELL, FIRST ROW COMMAND

06C3        10     16 KAFOPT5  %00010000 N      AIR FUEL OPTION FLAG WORD 5

BIT     DESCRIPTION

BIT0    1 = CCP SLEW OPTION;  0 = EGR SLEW OPTION

BIT1    X

BIT2    1 = STALL TORQUE MANAGEMENT OPTION SELECTED

BIT3    1 = 4 SPEED TRANSMISSION IS BEING USED

BIT4    1 = DIGITAL EGR; 0 = EVRV EGR

BIT5    1 = USE VACUUM AS LOAD AXIS FOR F72 TABLE LOOKUP

0 = USE MAP AS LOAD AXIS FOR F72 TABLE LOOKUP

BIT6    X

BIT7    1 = LEAN CRUISE OPTION

06C4      0260    608 KINJXBC5    0.2375 SEC/GM PRODUCT OF INJECTOR FLOW RATE AND FUEL

CALCULATION BASE CONSTANT  FOR SINGLE FIRE

INJECTOR FLOW RATE

06C6      0200    512 KDISFS       2.667 GL/HR  SINGLE INJECTOR FLOW RATE IN GALLONS/HOUR

06C8        5F     95 K3            67.4 % TPS  THROTTLE HIGH - THROTTLE LOW

NTPSLD = K3*(ADTHROT-K4); K4 IS INITIAL CONDITION

06C9        37     55 K4              55 CTS    INITIAL BASE THROTTLE POSITION IN A/D COUNTS

06CA        03      3 KTA0FF        0.01 COEF   LOW THROTTLE POSITION FILTER COEFFICIENT, N.D.

06CB        14     20 KMPLLTPS      26.3 KPA    MAP THRESH BELOW WHICH ADTA0FF CAN BE INC

06CC        66    102 KO2FF0        0.45 VOLTS  O2 FILTERS' INITIALIZATION VALUE,VOLTS * 226

06CD        FF    255 KMPFILT      0.996 COEFF  MAP FILTER COEFFICIENT

06CE        2E     46 KIDFRPML      1150 RPM    ENG SPEED BELOW WHICH CAN BECOME IDLE FUEL

FORMS HYSTERISIS PAIR WITH  KIDFRPMH

06CF        04      4 KIDFTPSL       1.6 % TPS  THROT POS BELOW WHICH CAN BECOME IDLE FUEL

FORMS HYSTERISIS PAIR WITH  KIDFTPSH

06D0        06      6 KIDFMPHH         6 MPH    MPH ABOVE WHICH CAN BECOME IDLE FUEL

FORMS HYSTERISIS PAIR WITH KIDFMPHL

06D1        30     48 KIDFMAPH        28 KPA    MAP ABOVE WHICH CAN BECOME IDLE FUEL

FORMS HYSTERISIS PAIR WITH  KIDFMAPL

06D2        64    100 KIDFMPL       47.3 KPA    MAP BELOW WHICH CAN BECOME IDLE FUEL

FORMS HYSTERISIS PAIR WITH  KIDFMPH

06D3        32     50 KIDFRPMH      1250 RPM    ENG SPEED ABOVE WHICH CAN BECOME SPEED DENSITY

FORMS HYSTERISIS PAIR WITH KIDFRPML

06D4        05      5 KIDFTPSH         2 % TPS  THROT POS ABOVE WHICH CAN BECOME SPEED DENSITY

FORMS HYSTERISIS PAIR WITH KIDFTPSL

06D5        03      3 KIDFMPHL         3 MPH    MPH BELOW WHICH CAN BECOME SPEED DENSITY

FORMS HYSTERISIS PAIR WITH KIDFMPHH

06D6        14     20 KIDFMAPL      17.7 KPA    MAP BELOW WHICH CAN BECOME SPEED DENSITY

FORMS HYSTERISIS PAIR WITH KIDFMAPH

06D7        78    120 KIDFMPH       54.6 KPA    MAP ABOVE WHICH CAN BECOME SPEED DENSITY

FORMS HYSTERISIS PAIR WITH KIDFMPL

06D8        5E     94 KMPBPWFT        45 KPA    MIN MAP THRESH FOR RPMFILT/RPM BPW MULT

06D9        58     88 KRPMBPFT      1100 RPM    MAX RPM THRESH FOR RPMFILT/RPM BPW MULT

06DA        60     96 KTFFTT          32 DEG C  TRANSIENT FUEL FILTER TEMP THRESHOLD (CLT)

06DB        4E     78 KTFFTTM1      18.5 DEG C  TRANSIENT FUEL FILTER TEMP THRESHOLD (MAT)

DECEL ENLEAN

06DC        60     96 KFIDETAC     0.375 COEF   COLD THROT ANGLE FILT COEF (DECEL ENLEAN)

06DD        30     48 KFIDEPMC     0.188 COEF   COLD MAN. PRESS FILT COEF (DECEL ENLEAN)

06DE        04      4 KFIDETAH     0.016 COEF   HOT THROT ANGLE FILT COEF (DECEL ENLEAN)

06DF        05      5 KFIDEPMH      0.02 COEF   HOT MAN. PRESS FILT COEF (DECEL ENLEAN)

#### Close Loop

06E0        93    147 KADSUCTA        70 DEG C  IF COOLANT >= THIS USE HOT  CLOSED LOOP TIMER (KT1A)

06E1        49     73 KCLTCB          15 DEG C  IF COOLANT <= THIS USE COLD CLOSED LOOP TIMER (KT3A)

06E2        5A     90 KT3A           180 SEC    COLD C/L TIMER VALUE, SEC/2 (ALSO USED TO ALLOW EGR)

06E3        37     55 KT2A           110 SEC    WARM C/L TIMER VALUE, SEC/2 (ALSO USED TO ALLOW EGR)

06E4        14     20 KT1A            40 SEC    HOT C/L TIMER VALUE, SEC/2 (ALSO USED TO ALLOW EGR)

06E5        0E     14 KCLTM        -29.5 DEG C  MAT TEMP THRESHOLD FOR C/L DETERMINATION

06E6      00FF    255 KCLTMTIM       255 SEC    TIME THRESH TO OVERRIDE MAT/KCLTM TEST

06E8        6A    106 KCLTC         39.5 DEG C  TEMPERATURE THRESHOLD FOR C/L DETERMINATION

06E9        0F     15 KO2ATIME         3 SEC    O2 SENSOR NOT READY TIMER LIMIT, SEC*5

06EA        32     50 KO2ATIMI        10 SEC    IDLE O2 SENS NOT READY TIME LIMIT, SEC*5

06EB        6B    107 KLCTCLL         40 DEG C  LC STORE ENABLE LOW COOLANT LEVEL,COOLDEG UNITS

06EC        80    128 KLCESTHU      3200 RPM    LC STORE ENABLE ENGINE SPEED UPPER THRESHOLD

##### BLM LIMIT 1

06ED        78    120 KSAMAMIN         120 CTS      LOW LIMIT FOR SAM CELL A (IDLE)

06EE        87    135 KSAMAMAX       135 CTS      HIGH LIMIT FOR SAM CELL A (IDLE)

06EF        74    116 KSAMBMIN        116 CTS      LOW LIMIT FOR SAM CELL B (PART THROT)

06F0        8A    138 KSAMBMAX       138 CTS      HIGH LIMIT FOR SAM CELL B (PART THROT)

06F1        FA    250 KSAMCOFS        -6 CTS        BLMOR = SAMOFFID + KSAMCOFS (TWO'S COMPLEMENT)

06F2        50     80 KSAMTPL            20 DEGC      TEMP TO ALLOW SAM DELAY DECREMENT, LOWER WINDOW

06F3        B8    184 KSAMTPH          98 DEGC      TEMP TO ALLOW SAM DELAY DECREMENT, UPPER WINDOW

06F4        BA    186 KSAMTPHI         99.5 DEGC   TEMP THRESH TO DETERMINE INITIAL SAM LRN DELAY

06F5        90    144 KSTMRSAM        432 SEC       IF STARTUP TMP<KSAMTPHI, INIT SAM LRN DELAY=THIS

06F6        F0    240 KLTMRSAM        720 SEC       IF STRTUP TMP>=KSAMTPHI, INIT SAM LRN DELAY=THIS

06F7        14     20 KSAMTIM             60 SEC        AFTER SAM LRN DELAY, SAM LRN ALLOWED THIS LONG

06F8        96    150 KCEL0TIM          30 SEC          STOP CELL0 UPDATE WHEN TIME OVER

06F9        A9    169 KCLTSML           87 DEGC       IF < THIS TEMP, SKIP SAM UPDATE

06FA        C3    195 KCLTSMH          106 DEGC     IF >= THIS TEMP, SKIP SAM UPDATE

06FB        13     19 KSAMACOF         19 COEFF     SAM A FILTER COEFFICIENT

06FC        08      8 KSAMBCOF          8 COEFF       SAM B FILTER COEFFICIENT

06FD        50     80 KLCRPM              1000 RPM      RPM THRESH ABOV WHICH LEARN IS DISABLD IN IDLE

06FE        FF    255 KBLMRPMH      6375 RPM    BLOCK LEARN IS DISABLED IF RPM >= KBLMRPMH

(NOT USED WHEN IN THE IDLE CELL)

06FF        10     16 KBLMRPML       400 RPM    BLOCK LEARN IS DISABLED IF RPM <= KBLMRPML

(NOT USED WHEN IN THE IDLE CELL)

0700        10     16 KCPBLDEL     0.125 MULT   DELTA SUBTRACTED FROM SAMOFFID TO ASSURE

THAT BLM DOESN'T GO TOO LEAN ON TRANSITION INTO CELL ONE OR WHILE IN PE

0701        0A     10 KCPBLDLA      0.08 MULT   BLMPT IS RESET TO (SAMOFFID-KCPBLDLA) IF PURGEDC IS

CURRENTLY < KCPBLDCL AND NTPSLD < KCPBLTPS AND PURGEDC

HAS PREVIOUSLY BEEN > KCPBLDCH, ALL WHILE IN CELL 1 (ASSURES

NOT TOO LEAN ON PURGE ACTIVE- NOT ACTIVE TRANSITION).

0702        80    128 KCPBLDCH        50 %      BLM1 RESET PURGEDC THRESH (SEE KCPBLDLA ABOVE)

0703        1A     26 KCPBLDCL        10 %      BLM1 RESET PURGEDC THRESH (SEE KCPBLDLA ABOVE)

0704        0D     13 KCPBLTPS         5 %      BLM1 RESET TPS THRESH (SEE KCPBLDLA ABOVE)

0705        FF    255 KLCVAC           0 KPA    VAC LOAD ABOVE WHICH BLOCK LEARN IS DISABLED IN PART

THROTTLE CELL

0706        20     32 KLEMAPAL        22 KPA    MAP BELOW WHICH BLOCK LEARN IS DISABLED

0707      0040     64 KLCBPWIL      0.98 MSEC   BPW > THRESH REQUIRED TO LEARN IN IDLE CELL

LOW HYSTERESIS VALUE, WITH KLCBPWIH

0709      004D     77 KLCBPWIH      1.17 MSEC   BPW > THRESH REQUIRED TO LEARN IN IDLE CELL

HIGH HYSTERESIS VALUE, WITH KLCBPWIL

070B      0047     71 KLCBPWL       1.08 MSEC   BPW > THRESH REQUIRED TO LEARN IN OFF-IDLE CELL

LOW HYSTERESIS VALUE, WITH KLCBPWH

070D      0055     85 KLCBPWH        1.3 MSEC   BPW > THRESH REQUIRED TO LEARN IN OFF-IDLE CELL

HIGH HYSTERESIS VALUE, WITH  KLCBPWL

070F        04      4 KCELIDLL       1.3 MPH    IF SPEED EXCEEDS THIS THRESHOLD, CAN'T BE IDLE

LOW HYSTERESIS VALUE, WITH KCELIDLH

0710        06      6 KCELIDLH         2 MPH    IF SPEED EXCEEDS THIS THRESHOLD, CAN'T BE IDLE

HIGH HYSTERESIS VALUE, WITH KCELIDLL

0711        24     36 KCELMAPL      31.3 KPA    THIS THRESH IS DIVIDING POINT BETWEEN

PART THROTTLE CELL AND OVERRUN CELL LOW HYSTERESIS VALUE, WITH

KCELMAPH

0712        37     55 KCELMAPH      37.2 KPA    HIGH MAP DETERMINES PART THROTTLE CELL

HIGH HYSTERESIS VALUE, WITH KCELMAPL

0713      0080    128 KCELBPWL      1.95 MSEC   THRESH BETWEEN CELL 1 AND CELL 2

LOW HYSTERESIS VALUE, WITH KCELBPWH

0715      0090    144 KCELBPWH       2.2 MSEC   BPW > THRESH DETERMINES CELL 1

HIGH HYSTERESIS VALUE, WITH KCELBPWL

0717        04      4 KLCITTH          4 UNITS  C/L INTEGRATOR WINDOW VALUE

0718        28     40 KRPMOFFH       500 RPM    HIGH HYST VALUE FOR LOW RPM INT RESET

0719        08      8 KRPMOFFL       100 RPM    LOW HYST VALUE FOR LOW RPM INT RESET

071A        85    133 KINTTCTH        60 DEGC   LOW RPM RESET LOGIC COOLANT THRESHOLD

MIN-MAX BLM Limits  (not idle)

071B        9C    156 KBLMMAX       1.22 VALUE  MAXIMUM ALLOWABLE BLM , VALUE*128

071C        64    100 KBLMMIN         0.78 VALUE  MINIMUM ALLOWABLE BLM , VALUE*128

071D        88    136 KO2AMAX        0.6 VOLTS  C/L TO O/L UPPER O2 THRESHOLD, O2 A/D UNITS

071E        44     68 KO2AMIN          0.3 VOLTS  C/L TO O/L LOWER O2 THRESHOLD, O2 A/D UNITS

071F        71    113 KO2MAXID       0.5 VOLTS  C/L TO O/L UPPER O2 THRESH (IDLE)

0720        5A     90 KO2MINID         0.4 VOLTS  C/L TO O/L LOWER O2 THRESH (IDLE)

0721        4C     76 KCLITMI            76 UNITS  CLOSED LOOP MINIMUM INTEGRATOR VALUE

0722        B4    180 KCLITMX          180 UNITS  CLOSED LOOP MAXIMUM INTEGRATOR VALUE

0723        74    116 KAFTCTH          47 DEG C  AIR/FUEL COOLANT THRESHOLD, COOLDEG UNITS

0724        50     80 KAFTCLOW        20 DEG C  LOW THRESHOLD FOR A/F

0725        5D     93 KAFTCHI             30 DEG C  HI THRESHOLD FOR A/F

0726        AA    170 KMAXLEAN        17 RATIO  MAX LEAN A/F

0727        64    100 KREFCOOL        35 DEG C  IF STARTUP COOLANT < KREFCOOL AND

NOT IN RUN FUEL MODE, FIRE INJECTORS ON EVERY REF

0728      FFFF  65535 KCKRSTIM     65535 SEC    MIN ENG RUNTIME TO RESET PULSE COUNTER

072A        FF    255 KCKRSTPS        99.6 % TPS  MIN TPS LIM FOR RESET PLS CTR

072B      3000  12288 KSCAL164      187.5 MSEC   CRANK TBL SCLR FOR MAX CRANK PW

072D      0054     84 KAPLH         1.28 MSEC   MIN BASE PULSE, MSEC* 65.536 HIGH HYSTERESIS VALUE, WITH

KAPLL

072F      0040     64 KAPLL         0.98 MSEC   MIN BASE PULSE, MSEC* 65.536 LOW HYSTERESIS VALUE, WITH

KAPLH

0731      028F    655 KAPMAX          10 MSEC   MAX ASYNCHRONOUS PULSE, MSEC*65.536

0733      0012     18 KAPMIN        0.27 MSEC   MIN ASYNCHRONOUS PULSE, MSEC*65.536

0735        FF    255 KQSYNMPH       255 MPH    QUASI-SYNCH DISABLED AT KQSYNMPH-3 MPH

0736        01      1 KFRPMTIM      0.01 SEC    HIGH RPM FUEL CUTOFF TIME THRESHOLD ENABLE

0737        30     48 KLNCRSML        48 MPH    LO OF HYST PAIR FOR MPH THRESH FOR LEAN CRUISE

LOW HYSTERESIS VALUE, WITH KLNCRSMH

0738        32     50 KLNCRSMH        50 MPH    HI OF HYST PAIR FOR MPH THRESH FOR LEAN CRUISE

LOW HYSTERESIS VALUE, WITH KLNCRSML

0739        9A    154 KAFLNCRS      15.4 RATIO  LEAN CRUISE AIR FUEL RATIO

073A        93    147 KAFSTCN       14.7 RATIO  STOICH. AIR FUEL RATIO

073B        01      1 KCLOPBLN       0.1 RATIO  CLOSED-TO-OPEN BLEND RATE FOR A/F

073C        04      4 KVSIDLE        1.3 MPH    VEHICLE SPEED THRESHOLD FOR IDLE

073D      0100    256 KSCAL53          1 FACTOR SCALE FACTOR FOR F53 TABLE

#### FUEL SHUT OFF

073F        6C    108 KLDFLOFF       108 MPH    SHUTOFF ALL FUEL IF MPH > THIS

0740      00D3    211 KRPFLOFF      6212 RPM    SHUTOFF ALL FUEL IF RPM >= THIS

0742        69    105 KLDFLON        105 MPH    ALLOW FUEL DELIVERY IF MPH < THIS ONCE FUEL HAS BEEN SHUT

OFF

0743      00DB    219 KRPFLON       5985 RPM    ALLOW FUEL DELIVERY IF RPM < THIS ONCE FUEL HAS BEEN

SHUT OFF     (65 KHZ CTS)

0745        FF    255 KCOOLVLT         5 VOLTS  LOW COOLANT VOLTAGE THRESHOLD

0746        FF    255 KCLTMRLM       255 SEC    LOW COOLANT TIME THRESHOLD

0747        20     32 KCLOFFST        24 DEG C  COOLANT OFFSET CALCULATION CONSTANT

0748      0000      0 KMINBPW          0 SEC    FUEL THRESHOLD FOR ACTIVATING THE 7TH INJ

074A        D9    217 KF94MAP        217 COUNTS FIXING FACTOR FOR MULTEC INJECTOR FLOW RATE.

WHEN DIVIDED BY ADMAPF+28, THIS FACTOR PRODUCES A

SUBTRACTED CHANGE TO BPW IN MSEC.  ITS UNITS ARE THEREFORE

'KPA*MSEC'.  TO PRODUCE THE COMPUTER UNITS FOR THIS CALIBRATION

FROM THE MAP AND DELTA BPW VALUES, USE THIS EQUATION:

N = (65.536*DELTA BPW)*(2.711* MAP) WHERE DELTA BPW IS IN MSEC AND

MAP IS IN KPA, OR, N = 2.711*(DELTA BPW)*(MAP) WHERE DELTA BPW IS IN

COUNTS AND MAP IS IN KPA.

####################################################################

# F26 TABLE                                                        #

#     MULTIPLIER (0-1) VS. MAP/BARO                                #

####################################################################

* -------------------------------------------------------------------

074B        1A     26                0.1 MULT     0.0  PRESS-RATIO

074C        1A     26                0.1 MULT   0.031

074D        1A     26                0.1 MULT   0.063

074E        1A     26                0.1 MULT   0.094

074F        1A     26                0.1 MULT   0.125

0750        1A     26                0.1 MULT   0.156

0751        1A     26                0.1 MULT   0.188

0752        1A     26                0.1 MULT   0.219

0753        1A     26                0.1 MULT    0.25

0754        1A     26                0.1 MULT   0.281

0755        1A     26                0.1 MULT   0.313

0756        14     20               0.08 MULT   0.344

0757        12     18               0.07 MULT   0.375

0758        1E     30              0.117 MULT   0.406

0759        0A     10               0.04 MULT   0.438

075A        06      6              0.023 MULT   0.469

075B        04      4              0.016 MULT     0.5

####################################################################

# F27 TABLE                                                        #

#     MULTIPLIER (0-1) VS. BARO                                    #

####################################################################

* -------------------------------------------------------------------

075C        FF    255              0.996 MULT      65    KPA-BARO

075D        C0    192               0.75 MULT      75

075E        80    128                0.5 MULT      85

075F        40     64               0.25 MULT      95

0760        00      0                  0 MULT     105

####################################################################

# F28 TABLE        BASE PULSE CONSTANT VS DESIRED EGR              #

#     CONSTANT = 1461.5 * (EGR FACTOR)(LITERS/CYL)/(INJECTOR RATE) #

#     INJECTOR RATE IS  IN GM PER SEC                              #

####################################################################

* -------------------------------------------------------------------

0761        E0    224              0.153 CONST      0      %-EGR

0762        DE    222              0.152 CONST    1.6

0763        D4    212              0.145 CONST    3.2

0764        CC    204             0.1396 CONST    4.8

0765        C6    198             0.1355 CONST    6.4

0766        C2    194              0.133 CONST    8.0

0767        BE    190               0.13 CONST    9.6

0768        BA    186              0.127 CONST   11.2

0769        BA    186              0.127 CONST   12.8

076A        BA    186              0.127 CONST   14.4

076B        BA    186              0.127 CONST   16.0

076C        BA    186              0.127 CONST   17.6

076D        BA    186              0.127 CONST   19.2

076E        BA    186              0.127 CONST   20.8

076F        BA    186              0.127 CONST   22.4

0770        BA    186              0.127 CONST   24.0

0771        BA    186              0.127 CONST   25.6

#############################################################

#  F29                                                      #

#    BASE PULSE VOLUMETRIC EFFICIENCY VS RPM AND MAP        #

#    TABLE VALUE = PERCENT*2.56                             #

#############################################################

#### BASE VE TABLES

* -------------------------------------------------------------------

0772        00      0                   0        R MIN: R = RPM

0773        00      0                  0        Q MIN: Q = MAP

0774        0D     13                 13        R NUM

* --------------------------------------------- SPEED        600 RPM

0775        04      4                1.6 %         15    KPA-MAP

0776        08      8                  3 %         20

0777        10     16                6.3 %         25

0778        18     24                9.4 %         30

0779        20     32               12.5 %         35

077A        28     40               15.6 %         40

077B        2E     46                 18 %         45

077C        3C     60               23.4 %         50

077D        46     70               27.3 %         60

077E        48     72                 28 %         70

077F        4A     74                 29 %         80

0780        4C     76               29.7 %         90

0781        4E     78               30.5 %        100

* --------------------------------------------- SPEED        800 RPM

0782        06      6                2.3 %         15    KPA-MAP

0783        0C     12                4.7 %         20

0784        16     22                8.6 %         25

0785        1C     28                 11 %         30

0786        24     36                 14 %         35

0787        2C     44                 17 %         40

0788        32     50               19.5 %         45

0789        42     66               25.8 %         50

078A        48     72                 28 %         60

078B        4A     74                 29 %         70

078C        4C     76               29.7 %         80

078D        4E     78               30.5 %         90

078E        52     82                 32 %        100

* --------------------------------------------- SPEED       1200 RPM

078F        0A     10                  4 %         15    KPA-MAP

0790        0E     14                5.5 %         20

0791        12     18                  7 %         25

0792        1E     30               11.7 %         30

0793        28     40               15.6 %         35

0794        30     48               18.8 %         40

0795        3C     60               23.4 %         45

0796        46     70               27.3 %         50

0797        48     72                 28 %         60

0798        4E     78               30.5 %         70

0799        52     82                 32 %         80

079A        56     86               33.6 %         90

079B        5A     90                 35 %        100

* --------------------------------------------- SPEED       1600 RPM

079C        0C     12                4.7 %         15    KPA-MAP

079D        14     20                  8 %         20

079E        16     22                8.6 %         25

079F        2A     42               16.4 %         30

07A0        32     50               19.5 %         35

07A1        46     70               27.3 %         40

07A2        48     72                 28 %         45

07A3        4A     74                 29 %         50

07A4        4C     76               29.7 %         60

07A5        52     82                 32 %         70

07A6        5A     90                 35 %         80

07A7        5C     92                 36 %         90

07A8        60     96               37.5 %        100

* --------------------------------------------- SPEED       2000 RPM

07A9        10     16                6.3 %         15    KPA-MAP

07AA        16     22                8.6 %         20

07AB        28     40               15.6 %         25

07AC        30     48               18.8 %         30

07AD        40     64                 25 %         35

07AE        48     72                 28 %         40

07AF        50     80               31.3 %         45

07B0        56     86               33.6 %         50

07B1        5A     90                 35 %         60

07B2        5C     92                 36 %         70

07B3        5E     94               36.7 %         80

07B4        60     96               37.5 %         90

07B5        66    102                 40 %        100

* --------------------------------------------- SPEED       2400 RPM

07B6        12     18                  7 %         15    KPA-MAP

07B7        26     38                 15 %         20

07B8        2E     46                 18 %         25

07B9        3C     60               23.4 %         30

07BA        42     66               25.8 %         35

07BB        4C     76               29.7 %         40

07BC        58     88               34.4 %         45

07BD        5A     90                 35 %         50

07BE        5C     92                 36 %         60

07BF        5E     94               36.7 %         70

07C0        60     96               37.5 %         80

07C1        62     98               38.3 %         90

07C2        68    104               40.6 %        100

* --------------------------------------------- SPEED       2800 RPM

07C3        16     22                8.6 %         15    KPA-MAP

07C4        28     40               15.6 %         20

07C5        36     54                 21 %         25

07C6        4C     76               29.7 %         30

07C7        50     80               31.3 %         35

07C8        56     86               33.6 %         40

07C9        5A     90                 35 %         45

07CA        5C     92                 36 %         50

07CB        5E     94               36.7 %         60

07CC        60     96               37.5 %         70

07CD        62     98               38.3 %         80

07CE        64    100                 39 %         90

07CF        6A    106               41.4 %        100

* --------------------------------------------- SPEED       3200 RPM

07D0        1E     30               11.7 %         15    KPA-MAP

07D1        32     50               19.5 %         20

07D2        48     72                 28 %         25

07D3        4E     78               30.5 %         30

07D4        52     82                 32 %         35

07D5        5A     90                 35 %         40

07D6        5E     94               36.7 %         45

07D7        60     96               37.5 %         50

07D8        62     98               38.3 %         60

07D9        64    100                 39 %         70

07DA        66    102                 40 %         80

07DB        68    104               40.6 %         90

07DC        6A    106               41.4 %        100

* --------------------------------------------- SPEED       3600 RPM

07DD        26     38                 15 %         15    KPA-MAP

07DE        38     56                 22 %         20

07DF        4C     76               29.7 %         25

07E0        54     84                 33 %         30

07E1        56     86               33.6 %         35

07E2        5C     92                 36 %         40

07E3        60     96               37.5 %         45

07E4        62     98               38.3 %         50

07E5        64    100                 39 %         60

07E6        66    102                 40 %         70

07E7        68    104               40.6 %         80

07E8        6A    106               41.4 %         90

07E9        6A    106               41.4 %        100

* --------------------------------------------- SPEED       4000 RPM

07EA        2E     46                 18 %         15    KPA-MAP

07EB        42     66               25.8 %         20

07EC        4E     78               30.5 %         25

07ED        56     86               33.6 %         30

07EE        5A     90                 35 %         35

07EF        5E     94               36.7 %         40

07F0        62     98               38.3 %         45

07F1        64    100                 39 %         50

07F2        66    102                 40 %         60

07F3        68    104               40.6 %         70

07F4        6A    106               41.4 %         80

07F5        6A    106               41.4 %         90

07F6        6A    106               41.4 %        100

#### Idle VE Table

#*****************************************************

#  EXTENDED F29 TABLE                                *

#    BASE PULSE VOLUMETRIC EFFICIENCY VS RPM & MAP   *

#     THIS TABLE TO BE USED FOR IDLE                 *

#    TABLE VALUE = PERCENT*2.56                      *

#*****************************************************

* -------------------------------------------------------------------

07F7        30     48                 48        R MIN: R = RPM (FTRPM125)

07F8        00      0                  0        Q MIN: Q = MAP (NMAPLD)

07F9        0D     13                 13        R NUM

* --------------------------------------------- SPEED        600 RPM

07FA        18     24                9.4 %         20    KPA-MAP

07FB        18     24                9.4 %         25

07FC        18     24                9.4 %         30

07FD        18     24                9.4 %         35

07FE        18     24                9.4 %         40

07FF        1B     27               10.5 %         45

0800        1E     30               11.7 %         50

0801        22     34               13.3 %         55

0802        26     38                 15 %         60

0803        2E     46                 18 %         70

0804        36     54                 21 %         80

0805        3E     62               24.2 %         90

0806        46     70               27.3 %        100

* --------------------------------------------- SPEED        800 RPM

0807        1A     26                 10 %         20    KPA-MAP

0808        1A     26                 10 %         25

0809        1A     26                 10 %         30

080A        1A     26                 10 %         35

080B        1A     26                 10 %         40

080C        1D     29               11.3 %         45

080D        20     32               12.5 %         50

080E        24     36                 14 %         55

080F        2A     42               16.4 %         60

0810        32     50               19.5 %         70

0811        3A     58               22.7 %         80

0812        42     66               25.8 %         90

0813        4A     74                 29 %        100

* --------------------------------------------- SPEED       1000 RPM

0814        1C     28                 11 %         20    KPA-MAP

0815        1C     28                 11 %         25

0816        1C     28                 11 %         30

0817        1C     28                 11 %         35

0818        1F     31                 12 %         40

0819        22     34               13.3 %         45

081A        26     38                 15 %         50

081B        28     40               15.6 %         55

081C        2E     46                 18 %         60

081D        36     54                 21 %         70

081E        3E     62               24.2 %         80

081F        48     72                 28 %         90

0820        50     80               31.3 %        100

* --------------------------------------------- SPEED       1200 RPM

0821        1E     30               11.7 %         20    KPA-MAP

0822        1E     30               11.7 %         25

0823        1E     30               11.7 %         30

0824        1E     30               11.7 %         35

0825        21     33                 13 %         40

0826        26     38                 15 %         45

0827        28     40               15.6 %         50

0828        2A     42               16.4 %         55

0829        32     50               19.5 %         60

082A        3A     58               22.7 %         70

082B        42     66               25.8 %         80

082C        4C     76               29.7 %         90

082D        54     84                 33 %        100

* --------------------------------------------- SPEED       1400 RPM

082E        20     32               12.5 %         20    KPA-MAP

082F        20     32               12.5 %         25

0830        20     32               12.5 %         30

0831        20     32               12.5 %         35

0832        23     35               13.7 %         40

0833        28     40               15.6 %         45

0834        2A     42               16.4 %         50

0835        2C     44                 17 %         55

0836        34     52               20.3 %         60

0837        3C     60               23.4 %         70

0838        44     68               26.6 %         80

0839        4E     78               30.5 %         90

083A        56     86               33.6 %        100

* --------------------------------------------- SPEED       1600 RPM

083B        22     34               13.3 %         20    KPA-MAP

083C        22     34               13.3 %         25

083D        22     34               13.3 %         30

083E        22     34               13.3 %         35

083F        25     37               14.4 %         40

0840        2A     42               16.4 %         45

0841        2C     44                 17 %         50

0842        2E     46                 18 %         55

0843        36     54                 21 %         60

0844        3E     62               24.2 %         70

0845        46     70               27.3 %         80

0846        51     81               31.6 %         90

0847        58     88               34.4 %        100

################################################

#     F30 TABLE                                #

#    BASE PULSE VOLUMETRIC EFFICIENCY VS RPM   #

#   TABLE VALUE = PERCENT*2.56                 #

################################################

BASE VE TABLE - F30

* -------------------------------------------------------------------

0848        10     16                16        USE 17 VALUE TABLE

0849        3C     60               23.4 %          0    RPM-SPEED

084A        46     70               27.3 %        400

084B        50     80               31.3 %        800

084C        52     82                 32 %       1200

084D        54     84                 33 %       1600

084E        58     88               34.4 %       2000

084F        5A     90                 35 %       2400

0850        5C     92                 36 %       2800

0851        64    100                 39 %       3200

0852        6A    106               41.4 %       3600

0853        6B    107               41.8 %       4000

0854        5A     90                 35 %       4400

0855        4C     76               29.7 %       4800

0856        3E     62               24.2 %       5200

0857        2A     42               16.4 %       5600

0858        1C     28                 11 %       6000

0859        10     16                6.3 %       6400

*##################################################################

*   F34 TABLE                                                     #

*   MODIFIER FOR VE WHEN EGR ON VS. RPM AND VACUUM                #

*##################################################################

* -------------------------------------------------------------------

085A        20     32                32        R MIN         800 RPM

085B        00      0                  0        Q MIN KPA VAC

085C        09      9                  9

*  800 RPM

* --------------------------------------------- SPEED        800 RPM

085D        00      0                  0 %          0    KPA-VAC

085E        00      0                  0 %         10

085F        00      0                  0 %         20

0860        00      0                  0 %         30

0861        00      0                  0 %         40

0862        00      0                  0 %         50

0863        00      0                  0 %         60

0864        00      0                  0 %         70

0865        00      0                  0 %         80

* --------------------------------------------- SPEED       1200 RPM

0866        00      0                  0 %          0    KPA-VAC

0867        00      0                  0 %         10

0868        00      0                  0 %         20

0869        00      0                  0 %         30

086A        00      0                  0 %         40

086B        00      0                  0 %         50

086C        00      0                  0 %         60

086D        00      0                  0 %         70

086E        00      0                  0 %         80

* --------------------------------------------- SPEED       1600 RPM

086F        00      0                  0 %          0    KPA-VAC

0870        00      0                  0 %         10

0871        00      0                  0 %         20

0872        00      0                  0 %         30

0873        00      0                  0 %         40

0874        00      0                  0 %         50

0875        00      0                  0 %         60

0876        00      0                  0 %         70

0877        00      0                  0 %         80

* --------------------------------------------- SPEED       2000 RPM

0878        00      0                  0 %          0    KPA-VAC

0879        00      0                  0 %         10

087A        00      0                  0 %         20

087B        00      0                  0 %         30

087C        00      0                  0 %         40

087D        10     16                 16 %         50

087E        14     20                 20 %         60

087F        1C     28                 28 %         70

0880        20     32                 32 %         80

* --------------------------------------------- SPEED       2400 RPM

0881        26     38                 38 %          0    KPA-VAC

0882        2C     44                 44 %         10

0883        32     50                 50 %         20

0884        38     56                 56 %         30

0885        3C     60                 60 %         40

0886        44     68                 68 %         50

0887        54     84                 84 %         60

0888        64    100                100 %         70

0889        74    116                116 %         80

* --------------------------------------------- SPEED       2800 RPM

088A        00      0                  0 %          0    KPA-VAC

088B        00      0                  0 %         10

088C        10     16                 16 %         20

088D        14     20                 20 %         30

088E        1C     28                 28 %         40

088F        20     32                 32 %         50

0890        26     38                 38 %         60

0891        2C     44                 44 %         70

0892        32     50                 50 %         80

* --------------------------------------------- SPEED       3200 RPM

0893        38     56                 56 %          0    KPA-VAC

0894        3C     60                 60 %         10

0895        44     68                 68 %         20

0896        54     84                 84 %         30

0897        64    100                100 %         40

0898        74    116                116 %         50

0899        00      0                  0 %         60

089A        00      0                  0 %         70

089B        10     16                 16 %         80

* --------------------------------------------- SPEED       3600 RPM

089C        14     20                 20 %          0    KPA-VAC

089D        1C     28                 28 %         10

089E        20     32                 32 %         20

089F        26     38                 38 %         30

08A0        2C     44                 44 %         40

08A1        32     50                 50 %         50

08A2        38     56                 56 %         60

08A3        3C     60                 60 %         70

08A4        44     68                 68 %         80

* --------------------------------------------- SPEED       4000 RPM

08A5        54     84                 84 %          0    KPA-VAC

08A6        64    100                100 %         10

08A7        74    116                116 %         20

08A8        00      0                  0 %         30

08A9        00      0                  0 %         40

08AA        00      0                  0 %         50

08AB        00      0                  0 %         60

08AC        00      0                  0 %         70

08AD        00      0                  0 %         80

##################################################

#     F31I TABLE                                 #

#   BASE PULSE INVERSE INLETTMP TERM VS INLETTMP #

#   TABLE VALUE = 50000./DEG. KELVIN             #

##################################################

* -------------------------------------------------------------------

08AE        DB    219               228 DEG K    -40  DEG C-COOLTEMP

08AF        CD    205                244 DEG K    -28

08B0        C2    194                258 DEG K    -16

08B1        B8    184                272 DEG K     -4

08B2        B3    179                279 DEG K      8

08B3        AD    173                289 DEG K     20

08B4        A9    169                296 DEG K     32

08B5        A6    166                301 DEG K     44

08B6        A4    164                305 DEG K     56

08B7        9E    158                317 DEG K     68

08B8        98    152                329 DEG K     80

08B9        95    149                336 DEG K     92

08BA        93    147                340 DEG K    104

08BB        8F    143                350 DEG K    116

08BC        8D    141                355 DEG K    128

08BD        8B    139                360 DEG K    140

08BE        87    135                370 DEG K    152

#*****************************************************************

#    F49    A/F MULTIPLIER VS. VACUUM OR NMAPLDAL                *

#    TABLE VALUE = MULTIPLIER * 128  (0-2)                       *

#*****************************************************************

* -------------------------------------------------------------------

08BF        88    136                1.06 MULT   80 20    VAC-MAP

08C0        80    128                  1 MULT   70 30

08C1        80    128                  1 MULT   60 40

08C2        80    128                  1 MULT   50 50

08C3        78    120               0.94 MULT   40 60

08C4        70    112              0.875 MULT   30 70

08C5        6C    108               0.84 MULT   20 80

08C6        68    104               0.81 MULT   10 90

08C7        60     96               0.75 MULT   0 100

#*****************************************************************

#    F50    BIASTEMP VS. COOLTSU                                 *

#   N=E*256/192                                                  *

#*****************************************************************

* -------------------------------------------------------------------

08C8        00      0                  0 DEG C    -40  DEG C-COOLTEMP

08C9        00      0                  0 DEG C    -28

08CA        00      0                  0 DEG C    -16

08CB        00      0                  0 DEG C     -4

08CC        00      0                  0 DEG C      8

08CD        00      0                  0 DEG C     20

08CE        00      0                  0 DEG C     32

08CF        00      0                  0 DEG C     44

08D0        00      0                  0 DEG C     56

08D1        00      0                  0 DEG C     68

08D2        00      0                  0 DEG C     80

###########################################################

#     F51SUBT TABLE  (SUBTRACTIVE AIR/FUEL TIMEOUT)       #

# AIR/FUEL TIME OUT DELTA VS COOLANT TEMP. (COOLDEGB)     #

#   TABLE VALUE = AIRFUEL * 10 (DELTA FROM COMPUTED A/F)  #

###########################################################

* -------------------------------------------------------------------

08D3        32     50                 5 RATIO    -28  DEG C-COOLTEMP

08D4        32     50                  5 RATIO    -16

08D5        2F     47                4.7 RATIO     -4

08D6        2B     43                4.3 RATIO      8

08D7        2A     42                4.2 RATIO     20

08D8        25     37                3.7 RATIO     32

08D9        1F     31                3.1 RATIO     44

08DA        1A     26                2.6 RATIO     56

08DB        12     18                1.8 RATIO     68

08DC        08      8                0.8 RATIO     80

08DD        03      3                0.3 RATIO     92

08DE        02      2                0.2 RATIO    104

08DF        0E     14                1.4 RATIO    116

#################################################################

#     F52 TABLE                                                 #

#    A/F TIME OUT REDUCTION INTERVAL VS COOLANT (COOLDEGB)      #

#   TABLE VALUE = INJECTS                                       #

#################################################################

* -------------------------------------------------------------------

08E0        30     48                 48 INJECT   -28  DEG C-COOLTEMP

08E1        30     48                 48 INJECT   -16

08E2        30     48                 48 INJECT    -4

08E3        30     48                 48 INJECT     8

08E4        30     48                 48 INJECT    20

08E5        20     32                 32 INJECT    32

08E6        20     32                 32 INJECT    44

08E7        0D     13                 13 INJECT    56

08E8        0D     13                 13 INJECT    68

08E9        0D     13                 13 INJECT    80

08EA        0D     13                 13 INJECT    92

08EB        1A     26                 26 INJECT   104

08EC        1A     26                 26 INJECT   116

#########################################################

#  F53  TABLE                                           #

# TIME OUT DECAY DELAY VS COOLANT TEMP. (COOLDEGB)      #

# TABLE VALUE = NUMBER OF INJECTS                       #

#########################################################

* -------------------------------------------------------------------

08ED        80    128                128 INJECT   -28  DEG C-TEMP

08EE        80    128                128 INJECT   -16

08EF        60     96                 96 INJECT    -4

08F0        60     96                 96 INJECT     8

08F1        60     96                 96 INJECT    20

08F2        60     96                 96 INJECT    32

08F3        60     96                 96 INJECT    44

08F4        60     96                 96 INJECT    56

08F5        50     80                 80 INJECT    68

08F6        40     64                 64 INJECT    80

08F7        40     64                 64 INJECT    92

08F8        40     64                 64 INJECT   104

08F9        40     64                 64 INJECT   116

################################################

# F55  TABLE                                   #

# TIMEOUT DECAY MULTIPLIER VS. STARTUP COOLANT #

#                                              #

# TABLE VALUE = MULTIPLIER * 256               #

################################################

* -------------------------------------------------------------------

08FA        F8    248              0.97 MULT     -40  DEG C-TEMP

08FB        F8    248               0.97 MULT     -28

08FC        F8    248               0.97 MULT     -16

08FD        F8    248               0.97 MULT      -4

08FE        FA    250              0.977 MULT       8

08FF        FA    250              0.977 MULT      20

0900        F8    248               0.97 MULT      32

0901        F8    248               0.97 MULT      44

0902        F8    248               0.97 MULT      56

0903        F9    249              0.973 MULT      68

0904        FA    250              0.977 MULT      80

0905        F8    248               0.97 MULT      92

0906        F8    248               0.97 MULT     104

0907        FE    254              0.992 MULT     116

###### COLD AIR FUEL RATIO

###########################################################

#     F56 TABLE                                           #

#    COLD ENGINE TEMP. DEPENDENT AIR/FUEL RATIO           #

#    VS COOLANT TEMPERATURE AND (NVACLD OR NMAPLDAL)      #

#   TABLE VALUE = AIR FUEL * 10                           #

###########################################################

* -------------------------------------------------------------------

0908        00      0                  0        R MIN; R = COOLDEG

0909        00      0                  0        Q MIN; Q = NVACLD OR NMAPLDAL

090A        09      9                  9        R NUM

* --------------------------------------------- COOLTEMP     -40 DEG C

090B        74    116               11.6 RATIO  80 20    VAC-MAP

090C        73    115               11.5 RATIO  70 30

090D        72    114               11.4 RATIO  60 40

090E        71    113               11.3 RATIO  50 50

090F        70    112               11.2 RATIO  40 60

0910        6E    110                 11 RATIO  30 70

0911        6D    109               10.9 RATIO  20 80

0912        6B    107               10.7 RATIO  10 90

0913        6A    106               10.6 RATIO  0 100

* --------------------------------------------- COOLTEMP     -28 DEG C

0914        74    116               11.6 RATIO  80 20    VAC-MAP

0915        73    115               11.5 RATIO  70 30

0916        72    114               11.4 RATIO  60 40

0917        71    113               11.3 RATIO  50 50

0918        70    112               11.2 RATIO  40 60

0919        6E    110                 11 RATIO  30 70

091A        6D    109               10.9 RATIO  20 80

091B        6B    107               10.7 RATIO  10 90

091C        6A    106               10.6 RATIO  0 100

* --------------------------------------------- COOLTEMP     -16 DEG C

091D        78    120                 12 RATIO  80 20    VAC-MAP

091E        77    119               11.9 RATIO  70 30

091F        76    118               11.8 RATIO  60 40

0920        75    117               11.7 RATIO  50 50

0921        74    116               11.6 RATIO  40 60

0922        72    114               11.4 RATIO  30 70

0923        70    112               11.2 RATIO  20 80

0924        6E    110                 11 RATIO  10 90

0925        6C    108               10.8 RATIO  0 100

* --------------------------------------------- COOLTEMP      -4 DEG C

0926        7C    124               12.4 RATIO  80 20    VAC-MAP

0927        7B    123               12.3 RATIO  70 30

0928        7A    122               12.2 RATIO  60 40

0929        79    121               12.1 RATIO  50 50

092A        78    120                 12 RATIO  40 60

092B        76    118               11.8 RATIO  30 70

092C        74    116               11.6 RATIO  20 80

092D        72    114               11.4 RATIO  10 90

092E        6F    111               11.1 RATIO  0 100

* --------------------------------------------- COOLTEMP       8 DEG C

092F        83    131               13.1 RATIO  80 20    VAC-MAP

0930        82    130                 13 RATIO  70 30

0931        81    129               12.9 RATIO  60 40

0932        7F    127               12.7 RATIO  50 50

0933        7D    125               12.5 RATIO  40 60

0934        7B    123               12.3 RATIO  30 70

0935        78    120                 12 RATIO  20 80

0936        76    118               11.8 RATIO  10 90

0937        73    115               11.5 RATIO  0 100

* --------------------------------------------- COOLTEMP      20 DEG C

0938        8B    139               13.9 RATIO  80 20    VAC-MAP

0939        8A    138               13.8 RATIO  70 30

093A        88    136               13.6 RATIO  60 40

093B        86    134               13.4 RATIO  50 50

093C        84    132               13.2 RATIO  40 60

093D        81    129               12.9 RATIO  30 70

093E        7D    125               12.5 RATIO  20 80

093F        7A    122               12.2 RATIO  10 90

0940        77    119               11.9 RATIO  0 100

* --------------------------------------------- COOLTEMP      32 DEG C

0941        92    146               14.6 RATIO  80 20    VAC-MAP

0942        91    145               14.5 RATIO  70 30

0943        8F    143               14.3 RATIO  60 40

0944        8C    140                 14 RATIO  50 50

0945        89    137               13.7 RATIO  40 60

0946        86    134               13.4 RATIO  30 70

0947        82    130                 13 RATIO  20 80

0948        7D    125               12.5 RATIO  10 90

0949        7A    122               12.2 RATIO  0 100

* --------------------------------------------- COOLTEMP      44 DEG C

094A        97    151               15.1 RATIO  80 20    VAC-MAP

094B        95    149               14.9 RATIO  70 30

094C        93    147               14.7 RATIO  60 40

094D        92    146               14.6 RATIO  50 50

094E        8F    143               14.3 RATIO  40 60

094F        8D    141               14.1 RATIO  30 70

0950        87    135               13.5 RATIO  20 80

0951        82    130                 13 RATIO  10 90

0952        7F    127               12.7 RATIO  0 100

* --------------------------------------------- COOLTEMP      56 DEG C

0953        94    148               14.8 RATIO  80 20    VAC-MAP

0954        93    147               14.7 RATIO  70 30

0955        90    144               14.4 RATIO  60 40

0956        90    144               14.4 RATIO  50 50

0957        8C    140                 14 RATIO  40 60

0958        8A    138               13.8 RATIO  30 70

0959        88    136               13.6 RATIO  20 80

095A        84    132               13.2 RATIO  10 90

095B        82    130                 13 RATIO  0 100

* --------------------------------------------- COOLTEMP      68 DEG C

095C        96    150                 15 RATIO  80 20    VAC-MAP

095D        96    150                 15 RATIO  70 30

095E        91    145               14.5 RATIO  60 40

095F        91    145               14.5 RATIO  50 50

0960        91    145               14.5 RATIO  40 60

0961        8F    143               14.3 RATIO  30 70

0962        8C    140                 14 RATIO  20 80

0963        8A    138               13.8 RATIO  10 90

0964        86    134               13.4 RATIO  0 100

* --------------------------------------------- COOLTEMP      80 DEG C

0965        96    150                 15 RATIO  80 20    VAC-MAP

0966        96    150                 15 RATIO  70 30

0967        91    145               14.5 RATIO  60 40

0968        91    145               14.5 RATIO  50 50

0969        91    145               14.5 RATIO  40 60

096A        8F    143               14.3 RATIO  30 70

096B        8C    140                 14 RATIO  20 80

096C        8A    138               13.8 RATIO  10 90

096D        86    134               13.4 RATIO  0 100

* --------------------------------------------- COOLTEMP      92 DEG C

096E        99    153               15.3 RATIO  80 20    VAC-MAP

096F        99    153               15.3 RATIO  70 30

0970        98    152               15.2 RATIO  60 40

0971        94    148               14.8 RATIO  50 50

0972        93    147               14.7 RATIO  40 60

0973        92    146               14.6 RATIO  30 70

0974        8C    140                 14 RATIO  20 80

0975        8A    138               13.8 RATIO  10 90

0976        86    134               13.4 RATIO  0 100

* --------------------------------------------- COOLTEMP     104 DEG C

0977        99    153               15.3 RATIO  80 20    VAC-MAP

0978        99    153               15.3 RATIO  70 30

0979        98    152               15.2 RATIO  60 40

097A        94    148               14.8 RATIO  50 50

097B        93    147               14.7 RATIO  40 60

097C        92    146               14.6 RATIO  30 70

097D        8C    140                 14 RATIO  20 80

097E        8A    138               13.8 RATIO  10 90

097F        86    134               13.4 RATIO  0 100

* --------------------------------------------- COOLTEMP     116 DEG C

0980        96    150                 15 RATIO  80 20    VAC-MAP

0981        96    150                 15 RATIO  70 30

0982        91    145               14.5 RATIO  60 40

0983        91    145               14.5 RATIO  50 50

0984        91    145               14.5 RATIO  40 60

0985        8F    143               14.3 RATIO  30 70

0986        8C    140                 14 RATIO  20 80

0987        8A    138               13.8 RATIO  10 90

0988        86    134               13.4 RATIO  0 100

###########################################################

#     F57 TABLE                                           #

#    AIR/FUEL RATIO  VS  COOLANT TEMPERATURE              #

#    FOR COLD ENGINE AND CLOSED THROTTLE                  #

#   TABLE VALUE = AIR FUEL * 10                           #

###########################################################

* -------------------------------------------------------------------

0989        78    120               12 RATIO    -40  DEG C-COOLTEMP

098A        78    120                 12 RATIO    -28

098B        7C    124               12.4 RATIO    -16

098C        7E    126               12.6 RATIO     -4

098D        80    128               12.8 RATIO      8

098E        84    132               13.2 RATIO     20

098F        89    137               13.7 RATIO     32

0990        8C    140                 14 RATIO     44

0991        8D    141               14.1 RATIO     56

0992        8E    142               14.2 RATIO     68

0993        8F    143               14.3 RATIO     80

0994        94    148               14.8 RATIO     92

0995        96    150                 15 RATIO    104

0996        92    146               14.6 RATIO    116

###########################################################

#     F58 TABLE                                           #

#    BLOCK LEARN DELAY TIME INTERVAL                      #

#       VS. DELTA BETWEEN INT AND 128                     #

#   TABLE VALUE = SEC*20                                  #

###########################################################

* -------------------------------------------------------------------

0997        52     82 F58A           4.1 SEC        0    INT-DELTA

0998        4C     76                3.8 SEC        2

0999        3C     60                  3 SEC        4

099A        32     50                2.5 SEC        6

099B        2C     44                2.2 SEC        8

099C        28     40                  2 SEC       10

099D        24     36                1.8 SEC       12

099E        20     32                1.6 SEC       14

099F        1C     28                1.4 SEC       16

09A0        18     24                1.2 SEC       18

09A1        14     20                  1 SEC       20

09A2        14     20                  1 SEC       22

09A3        14     20                  1 SEC       24

09A4        14     20                  1 SEC       26

09A5        14     20                  1 SEC       28

09A6        14     20                  1 SEC       30

09A7        14     20                  1 SEC       32

#######################################################

#     F33 TABLE             FUEL BIAS                 #

#     FUEL INJECTOR OFFSET VS. BATTERY VOLTAGE        #

#     TABLE VALUE = MSEC * 32.768                     #

#######################################################

* -------------------------------------------------------------------

09A8        10     16                16        USE 17 VALUE TABLE

09A9        16     22               0.67 MSEC     0.0  VOLTS-BATTERY

09AA        16     22               0.67 MSEC     1.6

09AB        16     22               0.67 MSEC     3.2

09AC        16     22               0.67 MSEC     4.8

09AD        B2    178               5.43 MSEC     6.4

09AE        4E     78               2.38 MSEC     8.0

09AF        32     50               1.53 MSEC     9.6

09B0        20     32               0.98 MSEC    11.2

09B1        1A     26                0.8 MSEC    12.8

09B2        13     19               0.58 MSEC    14.4

09B3        0D     13                0.4 MSEC    16.0

09B4        09      9               0.27 MSEC    17.6

09B5        07      7                0.2 MSEC    19.2

09B6        05      5               0.15 MSEC    20.8

09B7        03      3                0.1 MSEC    22.4

09B8        01      1               0.03 MSEC    24.0

09B9        00      0                  0 MSEC    25.5

#######################################################

#     F93 TABLE   MULTIPLIER FOR 1ST QSYNCH PULSE     #

#     MULTIPLIER VS. RPM                              #

#     TABLE VALUE = FACTOR * 256                      #

#  NOTE: THIS TABLE DOES NOT USE THE STANDARD LOOKUP  #

#        ROUTINES AND NO INTERPOLATION IS USED.       #

#######################################################

* -------------------------------------------------------------------

09BA        A0    160                0.625 FACT       0    RPM-SPEED

09BB        A0    160              0.625 FACT     400

09BC        A0    160              0.625 FACT     800

09BD        A0    160              0.625 FACT    1200

09BE        A0    160              0.625 FACT    1600

09BF        A0    160              0.625 FACT    2000

09C0        A0    160              0.625 FACT    2400

09C1        A0    160              0.625 FACT    2800

09C2        A0    160              0.625 FACT    3200

09C3        A0    160              0.625 FACT    3600

09C4        A0    160              0.625 FACT    4000

09C5        A0    160              0.625 FACT    4400

09C6        A0    160              0.625 FACT    4800

09C7        A0    160              0.625 FACT    5200

09C8        A0    160              0.625 FACT    5600

09C9        A0    160              0.625 FACT    6000

09CA        A0    160              0.625 FACT    6400

#######################################################

#     F94 TABLE             FUEL BIAS                 #

#     FUEL INJECTOR OFFSET VS. BASE PULSE WIDTH       #

#     TABLE VALUE = MSEC * 65.536                     #

#######################################################

* -------------------------------------------------------------------

09CB        0C     12               0.18 MSEC   0.488   MSEC-BPW

09CC        0A     10               0.15 MSEC   0.732

09CD        08      8               0.12 MSEC   0.977

09CE        06      6               0.09 MSEC   1.221

09CF        03      3               0.05 MSEC   1.465

09D0        02      2               0.03 MSEC   1.709

09D1        01      1               0.01 MSEC   1.953

09D2        00      0                  0 MSEC   2.197

09D3        00      0                  0 MSEC   2.441

09D4        00      0                  0 MSEC   2.686

09D5        00      0                  0 MSEC   2.930

09D6        00      0                  0 MSEC   3.174

09D7        00      0                  0 MSEC   3.418

09D8        00      0                  0 MSEC   3.662

09D9        00      0                  0 MSEC   3.906

#############################

F163 TABLE                #

MULTIPLIER VS RPM         #

TABLE VALUE = MULTIPLIER  #

* -------------------------------------------------------------------

09DA        08      8                 8        USE 9 VALUE TABLE

09DB        01      1              0.004 MULT       0    RPM-SPEED

09DC        0A     10               0.04 MULT     200

09DD        2D     45              0.176 MULT     400

09DE        4F     79               0.31 MULT     600

09DF        8F    143               0.56 MULT     800

09E0        C0    192               0.75 MULT    1000

09E1        D5    213              0.832 MULT    1200

09E2        EA    234              0.914 MULT    1400

09E3        FF    255              0.996 MULT    1600

###########################################################

#  F164 TABLE                                             #

#  CRANK FUEL PW VS COOLANT TEMPERATURE                   #

# TABLE VALUE = (MSEC*65.536*256)/F64SCAL                 #

#  NOTE: THIS IS SQUIRT NOT CYLINDERS WORTH OF FUEL       #

###########################################################

* -------------------------------------------------------------------

09E4        D8    216               158 MSEC     -40  DEG C-TEMP

09E5        C6    198                145 MSEC     -28

09E6        9F    159              116.4 MSEC     -16

09E7        77    119                 87 MSEC      -4

09E8        5F     95               69.6 MSEC       8

09E9        42     66                 48 MSEC      20

09EA        2D     45                 33 MSEC      32

09EB        24     36               26.4 MSEC      44

09EC        1F     31                 23 MSEC      56

09ED        1D     29                 21 MSEC      68

09EE        19     25                 18 MSEC      80

09EF        15     21               15.4 MSEC      92

09F0        15     21               15.4 MSEC     104

09F1        18     24               17.6 MSEC     116

########################################################

#  F165 TABLE                                          #

#  CRANK FUEL PW MULTIPLIER VS REFERENCE PULSES        #

#  TABLE VALUE = MULTIPLIER * 256                      #

########################################################

* -------------------------------------------------------------------

09F2        E0    224             0.875 MULT       0    REF-PULSES

09F3        B0    176              0.688 MULT       8

09F4        70    112              0.438 MULT      16

09F5        70    112              0.438 MULT      24

09F6        70    112              0.438 MULT      32

09F7        70    112              0.438 MULT      40

#########################################

#  F166 TABLE                           #

#  CRANK FUEL PW MULTIPLIER VS TPS      #

#  TABLE VALUE =  MULTIPLIER * 64 (0-4) #

#########################################

* -------------------------------------------------------------------

09F8        08      8                      8        USE 9 VALUE TABLE

09F9        40     64                  1 MULT    00.0      %-THROTPOS

09FA        70    112               1.75 MULT    12.5

09FB        80    128                  2 MULT    25.0

09FC        80    128                  2 MULT    37.5

09FD        80    128                  2 MULT    50.0

09FE        80    128                  2 MULT    62.5

09FF        00      0                  0 MULT    75.0

0A00        00      0                  0 MULT    87.5

0A01        00      0                  0 MULT   100.0

#################################################

#  F113  TABLE                                  #

# CRANK PULSE WIDTH MULTIPLIER VS. BARO         #

# TABLE VALUE = MULIPLIER * 256                 #

#################################################

* -------------------------------------------------------------------

0A02        A6    166              0.65 MULT      65    KPA-BARO

0A03        B2    178              0.695 MULT      70

0A04        C0    192               0.75 MULT      75

0A05        CD    205                0.8 MULT      80

0A06        D9    217              0.848 MULT      85

0A07        E6    230                0.9 MULT      90

0A08        F3    243               0.95 MULT      95

0A09        F9    249              0.973 MULT     100

0A0A        FF    255              0.996 MULT     105

######################################################

F109 TABLE

AIR-FUEL RATIO WITH CATALYTIC CONVERTER

OVERTEMP  VS. AIRFLOW

TABLE VALUE = A/F RATIO * 10

######################################################

* -------------------------------------------------------------------

0A0B        91    145              14.5 RATIO      0 AIRFLO-GM/SEC

0A0C        82    130                 13 RATIO     16

0A0D        82    130                 13 RATIO     32

0A0E        80    128               12.8 RATIO     48

0A0F        7D    125               12.5 RATIO     64

0A10        7B    123               12.3 RATIO     80

0A11        7A    122               12.2 RATIO     96

0A12        78    120                 12 RATIO    112

0A13        78    120                 12 RATIO    128

*************************************************************

*   TABLE F150                                              *

*      MANIFOLD RUNNER TEMP TARGET COEFFICIENT              *

*                     VS AIRFLOW                            *

*************************************************************

* -------------------------------------------------------------------

0A14        00      0                0 COEF       0 AIRFLO-GM/SEC

0A15        10     16              0.063 COEF       4

0A16        20     32              0.125 COEF       8

0A17        40     64               0.25 COEF      12

0A18        64    100               0.39 COEF      16

0A19        68    104              0.406 COEF      20

0A1A        6A    106              0.414 COEF      24

0A1B        6C    108               0.42 COEF      28

0A1C        70    112              0.438 COEF      32

0A1D        74    116              0.453 COEF      36

0A1E        78    120               0.47 COEF      40

0A1F        7C    124              0.484 COEF      44

0A20        80    128                0.5 COEF      48

0A21        86    134              0.523 COEF      52

0A22        8D    141               0.55 COEF      56

0A23        8D    141               0.55 COEF      60

0A24        8D    141               0.55 COEF      64

*************************************************************

*   TABLE F151                                              *

*      TABLE OF LAG FILTER COEFF'S USED TO FILTER INLETTMP  *

*      TOWARD COOLANT TEMP                                  *

*                     VS AIRFLOW                            *

*************************************************************

* -------------------------------------------------------------------

0A25        90    144             0.563 COEF       0 AIRFLO-GM/SEC

0A26        90    144              0.563 COEF       4

0A27        80    128                0.5 COEF       8

0A28        70    112              0.438 COEF      12

0A29        60     96              0.375 COEF      16

0A2A        60     96              0.375 COEF      20

0A2B        60     96              0.375 COEF      24

0A2C        60     96              0.375 COEF      28

0A2D        60     96              0.375 COEF      32

0A2E        60     96              0.375 COEF      36

0A2F        68    104              0.406 COEF      40

0A30        70    112              0.438 COEF      44

0A31        80    128                0.5 COEF      48

0A32        80    128                0.5 COEF      52

0A33        80    128                0.5 COEF      56

0A34        80    128                0.5 COEF      60

0A35        80    128                0.5 COEF      64

****************************************************************

*   TABLE F152                                                 *

*   COOLANT OFFSET MULTIPLIER (0-1) VS COOLANT TEMP (COOLDEG)  *

****************************************************************

* -------------------------------------------------------------------

0A36        00      0                 0 MULT     -40  DEG C-TEMP

0A37        00      0                  0 MULT     -28

0A38        00      0                  0 MULT     -16

0A39        00      0                  0 MULT      -4

0A3A        00      0                  0 MULT       8

0A3B        00      0                  0 MULT      20

0A3C        00      0                  0 MULT      32

0A3D        00      0                  0 MULT      44

0A3E        00      0                  0 MULT      56

0A3F        00      0                  0 MULT      68

0A40        00      0                  0 MULT      80

0A41        01      1              0.004 MULT      92

0A42        02      2              0.008 MULT     104

0A43        04      4              0.016 MULT     116

0A44        06      6              0.023 MULT     128

0A45        08      8               0.03 MULT     140

0A46        08      8               0.03 MULT     152

#######################################

#     F130 TABLE                      #

#   AIRFLOW OFFSET PER % TPS VS. RPM  #

#######################################

* -------------------------------------------------------------------

0A47        20     32                 32 G/S      400    RPM-SPEED

0A48        30     48                 48 G/S      800

0A49        3E     62                 62 G/S     1200

0A4A        4E     78                 78 G/S     1600

0A4B        5D     93                 93 G/S     2000

0A4C        6C    108                108 G/S     2400

0A4D        8E    142                142 G/S     3200

0A4E        B9    185                185 G/S     4000

0A4F        D7    215                215 G/S     4800

#######################################

#     F131 TABLE                      #

#   MULTIPLIER (0-1) VS. MAP/BARO     #

#######################################

* -------------------------------------------------------------------

0A50        D8    216              0.847 MUL     0.31  PRESS-RATIO

0A51        DC    220              0.863 MUL    0.344

0A52        E3    227               0.89 MUL    0.375

0A53        E5    229                0.9 MUL    0.406

0A54        E7    231              0.906 MUL    0.438

0A55        E8    232               0.91 MUL    0.469

0A56        EC    236              0.925 MUL    0.500

0A57        F0    240               0.94 MUL    0.531

0A58        F3    243              0.953 MUL    0.562

0A59        F6    246              0.965 MUL    0.593

0A5A        F9    249              0.976 MUL    0.625

0A5B        FB    251              0.984 MUL    0.656

0A5C        FC    252               0.99 MUL    0.687

0A5D        FD    253              0.992 MUL    0.718

0A5E        FE    254              0.996 MUL    0.750

0A5F        FF    255                  1 MUL    0.781

0A60        FF    255                  1 MUL    0.812

###########################################

#     F132 TABLE                          #

#   MULTIPLIER (0-2) VS. MAT OR INLETTMP  #

###########################################

* -------------------------------------------------------------------

0A61        9A    154               1.2 MUL      -40  DEG C-TEMP

0A62        97    151               1.18 MUL      -28

0A63        93    147               1.15 MUL      -16

0A64        90    144              1.125 MUL       -4

0A65        8C    140               1.09 MUL        8

0A66        87    135              1.055 MUL       20

0A67        84    132               1.03 MUL       32

0A68        84    132               1.03 MUL       44

0A69        83    131               1.02 MUL       56

0A6A        82    130              1.016 MUL       68

0A6B        81    129               1.01 MUL       80

0A6C        80    128                  1 MUL       92

0A6D        79    121              0.945 MUL      104

0A6E        77    119               0.93 MUL      116

0A6F        75    117              0.914 MUL      128

0A70        73    115                0.9 MUL      140

0A71        72    114               0.89 MUL      152

#######################################

#     F133 TABLE                      #

#   MULTIPLIER (0-1) VS. BARO         #

#######################################

* -------------------------------------------------------------------

0A72        B8    184             0.72 MULT      65    KPA-BARO

0A73        CC    204              0.797 MULT      75

0A74        E6    230                0.9 MULT      85

0A75        F4    244              0.953 MULT      95

0A76        FF    255              0.996 MULT     105

###############################################

#     F134 TABLE                              #

# AIRFLOW (GMS/SEC*16) VS. IAC MOTOR POSITION #

###############################################

* -------------------------------------------------------------------

0A77        00      0 F134A            0 G/SEC      0    IAC-STEPS

0A78        06      6                0.4 G/SEC      4

0A79        0C     12               0.75 G/SEC      8

0A7A        12     18                1.1 G/SEC     12

0A7B        18     24                1.5 G/SEC     16

0A7C        1E     30                1.9 G/SEC     20

0A7D        24     36               2.25 G/SEC     24

0A7E        2A     42                2.6 G/SEC     28

0A7F        30     48                  3 G/SEC     32

0A80        36     54                3.4 G/SEC     36

0A81        3C     60               3.75 G/SEC     40

0A82        42     66                4.1 G/SEC     44

0A83        48     72                4.5 G/SEC     48

0A84        66    102                6.4 G/SEC     64

0A85        88    136                8.5 G/SEC     80

0A86        B0    176                 11 G/SEC     96

0A87        E4    228              14.25 G/SEC    112

#################################################

#     F135 TABLE                                #

# AIRFLOW (GMS/SEC*16) VS. IAC MIN MOT POSITION #

#################################################

* -------------------------------------------------------------------

0A88        5B     91              5.7 G/SEC      0    IAC-STEPS

0A89        55     85                5.3 G/SEC      4

0A8A        4F     79               4.94 G/SEC      8

0A8B        49     73               4.56 G/SEC     12

0A8C        43     67                4.2 G/SEC     16

0A8D        3D     61                3.8 G/SEC     20

0A8E        37     55               3.44 G/SEC     24

0A8F        31     49               3.06 G/SEC     28

0A90        2B     43                2.7 G/SEC     32

0A91        25     37                2.3 G/SEC     36

0A92        1F     31               1.94 G/SEC     40

0A93        19     25               1.56 G/SEC     44

0A94        14     20               1.25 G/SEC     48

0A95        0A     10                0.6 G/SEC     52

0A96        10     16                  1 G/SEC     56

0A97        10     16                  1 G/SEC     60

0A98        10     16                  1 G/SEC     64

0A99        24     36 KSUAETMR       3.6 SEC    DELAY BEFORE TIME-OUT A.E. STARTED

0A9A        FF    255 KNUMINJ        255 INJECT NUMBER OF INJECTS BETWEEN TIME-OUT  A.E. DECAYS

0A9B        00      0 KAEBLDTM         0 SEC    A.E. BLEND TIME

0A9C        A0    160 KINAEBLD     0.625 MULT   INITIAL A.E. BLEND MULTIPLIER

0A9D        FF    255 KAEBLDDT     0.996 MULT   MULTIPLIER INCREASE PER 12.5 MSEC

0A9E        05      5 KTPSAE           2 %TPS   DELTA TPS NEEDED TO ENABLE A.E.

0A9F        03      3 KTPSAEEN         1 %TPS   DELTA TPS NEEDED TO REMAIN IN A.E.

0AA0        80    128 KDTPSLIM        50 % TPS  DELTA TPS LIMIT FOR V6 AE CALC

0AA1        1A     26 KAEOPT       0.102 MULT   BPINJ SCALING FACTOR FOR AEPW CALC.

0AA2        80    128 KTPSFACT       0.5 MULT   FACTOR FOR DELTATPS * 32 TERM

0AA3        08      8 KF100RST         3 %TPS   DELTA TPS FOR F100 REENTRY IF AE ENABLED

0AA4        08      8 KF100REN         8 CTS    INITIAL AECNT FOR F100 REENTRY

*#################################

*  F100B TABLE                   #

*  AE FACTOR % BPINJ (0-4)       #

* TABLE VALUE = FACTOR * 64      #

*#################################

* -------------------------------------------------------------------

0AA5        35     53               0.83 FACTOR     1     AE-PULSES

0AA6        45     69               1.08 FACTOR     2

0AA7        45     69               1.08 FACTOR     3

0AA8        40     64                  1 FACTOR     4

0AA9        30     48               0.75 FACTOR     5

0AAA        25     37               0.58 FACTOR     6

0AAB        20     32                0.5 FACTOR     7

0AAC        10     16               0.25 FACTOR     8

*##############################################################

*  F101B                                                      #

*  0-2 MULTIPLIER OF ASYNFACTOR VS. DELTATPS * 2              #

* TABLE VALUE = FACTOR * 128                                  #

*##############################################################

* -------------------------------------------------------------------

0AAD        60     96               0.75 MULT    0.00      %-DELTATPS

0AAE        70    112              0.875 MULT    3.13

0AAF        70    112              0.875 MULT    6.25

0AB0        70    112              0.875 MULT    9.38

0AB1        70    112              0.875 MULT   12.50

0AB2        65    101               0.79 MULT   15.63

0AB3        65    101               0.79 MULT   18.75

0AB4        65    101               0.79 MULT   21.88

0AB5        65    101               0.79 MULT   25.00

0AB6        65    101               0.79 MULT   28.13

0AB7        65    101               0.79 MULT   31.25

0AB8        63     99               0.77 MULT   34.38

0AB9        60     96               0.75 MULT   37.50

0ABA        60     96               0.75 MULT   40.63

0ABB        55     85              0.664 MULT   43.75

0ABC        55     85              0.664 MULT   46.88

0ABD        55     85              0.664 MULT   50.00

*####################################

*  F102A TABLE                      #

*  ASYNC FACTOR     F(COOLANT)      #

* TABLE VALUE = FACTOR * 128        #

*####################################

* -------------------------------------------------------------------

0ABE        50     80             0.625 FACTOR   -40  DEG C-TEMP

0ABF        50     80              0.625 FACTOR   -28

0AC0        50     80              0.625 FACTOR   -16

0AC1        60     96               0.75 FACTOR    -4

0AC2        60     96               0.75 FACTOR     8

0AC3        60     96               0.75 FACTOR    20

0AC4        60     96               0.75 FACTOR    32

0AC5        38     56               0.44 FACTOR    44

0AC6        36     54               0.42 FACTOR    56

0AC7        34     52               0.41 FACTOR    68

0AC8        32     50               0.39 FACTOR    80

0AC9        30     48              0.375 FACTOR    92

0ACA        28     40               0.31 FACTOR   104

0ACB        26     38                0.3 FACTOR   116

0ACC        24     36               0.28 FACTOR   128

*####################################

*  F103  TABLE                      #

*  ASYNC FACTOR     F(BARO)         #

* TABLE VALUE = FACTOR * 256        #

*####################################

* -------------------------------------------------------------------

0ACD        C0    192               0.75 FACTOR    65    KPA-BARO

0ACE        D5    213              0.832 FACTOR    75

0ACF        E0    224              0.875 FACTOR    85

0AD0        F3    243               0.95 FACTOR    95

0AD1        FE    254              0.992 FACTOR   105

*####################################

*  F107  TABLE                      #

*  ASYNC FACTOR     F(MATDEG)       #

* TABLE VALUE = FACTOR * 128        #

*####################################

* -------------------------------------------------------------------

0AD2        08      8                   8        USE 9 VALUE TABLE

0AD3        A0    160               1.25 FACTOR   -40  DEG C-TEMP

0AD4        A0    160               1.25 FACTOR   -16

0AD5        A0    160               1.25 FACTOR     8

0AD6        80    128                  1 FACTOR    32

0AD7        65    101               0.79 FACTOR    56

0AD8        65    101               0.79 FACTOR    80

0AD9        65    101               0.79 FACTOR   104

0ADA        65    101               0.79 FACTOR   128

0ADB        65    101               0.79 FACTOR   152

############################################################

#  F104   TPS FILTER COEFFICIENT VS. COOLANT               #

#         TABLE VALUE = COEFF * 256                        #

############################################################

* -------------------------------------------------------------------

0ADC        0D     13               0.05 COEFF    -40  DEG C-TEMP

0ADD        0D     13               0.05 COEFF    -28

0ADE        0D     13               0.05 COEFF    -16

0ADF        0C     12              0.047 COEFF     -4

0AE0        0C     12              0.047 COEFF      8

0AE1        0C     12              0.047 COEFF     20

0AE2        0D     13               0.05 COEFF     32

0AE3        0F     15               0.06 COEFF     44

0AE4        10     16              0.063 COEFF     56

0AE5        11     17              0.066 COEFF     68

0AE6        11     17              0.066 COEFF     80

0AE7        11     17              0.066 COEFF     92

0AE8        11     17              0.066 COEFF    104

0AE9        11     17              0.066 COEFF    116

0AEA        11     17              0.066 COEFF    128

####################################

#  F105 TABLE                      #

#                                  #

# TIME-OUT REDUCTION FACTOR OF TPS #

#      FILTER COEFFICIENT          #

#             VS.                  #

#     START-UP COOLANT             #

# TABLE VALUE = COEFF * 256        #

####################################

* -------------------------------------------------------------------

0AEB        08      8               0.03 FACTOR   -40  DEG C-TEMP

0AEC        08      8               0.03 FACTOR   -28

0AED        08      8               0.03 FACTOR   -16

0AEE        08      8               0.03 FACTOR    -4

0AEF        08      8               0.03 FACTOR     8

0AF0        08      8               0.03 FACTOR    20

0AF1        08      8               0.03 FACTOR    32

0AF2        08      8               0.03 FACTOR    44

0AF3        08      8               0.03 FACTOR    56

0AF4        08      8               0.03 FACTOR    68

0AF5        08      8               0.03 FACTOR    80

0AF6        08      8               0.03 FACTOR    92

0AF7        08      8               0.03 FACTOR   104

0AF8        08      8               0.03 FACTOR   116

0AF9        08      8               0.03 FACTOR   128

####################################

#  F106 TABLE                      #

# TIME-OUT REDUCTION FACTOR MULT.  #

#  FOR TPS FILTER COEFFICIENT      #

#             VS.                  #

#     START-UP COOLANT             #

# TABLE VALUE = COEFF * 256        #

####################################

* -------------------------------------------------------------------

0AFA        50     80                0.313 MULT     -40  DEG C-TEMP

0AFB        80    128                0.5 MULT     -28

0AFC        80    128                0.5 MULT     -16

0AFD        CC    204              0.797 MULT      -4

0AFE        CC    204              0.797 MULT       8

0AFF        CC    204              0.797 MULT      20

0B00        CC    204              0.797 MULT      32

0B01        99    153              0.598 MULT      44

0B02        80    128                0.5 MULT      56

0B03        80    128                0.5 MULT      68

0B04        80    128                0.5 MULT      80

0B05        80    128                0.5 MULT      92

0B06        80    128                0.5 MULT     104

0B07        80    128                0.5 MULT     116

0B08        80    128                0.5 MULT     128

0B09        0C     12 KCONVTPS       4.7 %      PE TPS OFFSET WHEN C.O.T. PRESENT

0B0A        0C     12 KPEHYS         4.7 %      PE TPS HYSTERESIS

0B0B        A1    161 KMAPWOT       69.7 KPA    IF MAP > THIS ENABLE PE

0B0C        01      1 KMAPHYST      10.7 KPA    MAP HYSTERSIS FOR POWER ENRICHMENT

0B0D        FF    255 KPEHMPH        255 MPH    THRESHOLD TO CK TO CLEAR PEHMPHCT

0B0E        D0    208 KPECLTTM       116 DEG C  THRESHOLD TO CK TO CLEAR PEHMPHCT

0B0F      0190    400 KPEMPHTM        10 SEC    PE MPH TIME DELAY BEFORE DECREASING AFPE

0B11        76    118 KPEMAP4         54 KPA    PE THRESHOLD WHEN 'KPEHMPH' MPH FOR 'KPEMPHTM'

0B12        14     20 KPEAFDLT         2 RATIO  PE AIRFUEL HIGH MPH DELTA

###################################################

#  F61                                            #

#  POWER ENRICHMENT AIRFUEL RATIO VS. NTRPMX      #

#      TABLE VALUE = RATIO * 10                   #

###################################################

* -------------------------------------------------------------------

0B13        6F    111               11.1 RATIO      0    RPM-SPEED

0B14        6F    111               11.1 RATIO    400

0B15        6F    111               11.1 RATIO    800

0B16        6F    111               11.1 RATIO   1200

0B17        71    113               11.3 RATIO   1600

0B18        79    121               12.1 RATIO   2000

0B19        7B    123               12.3 RATIO   2400

0B1A        7C    124               12.4 RATIO   2800

0B1B        75    117               11.7 RATIO   3200

0B1C        77    119               11.9 RATIO   3600

0B1D        7C    124               12.4 RATIO   4000

0B1E        76    118               11.8 RATIO   4400

0B1F        75    117               11.7 RATIO   4800

0B20        75    117               11.7 RATIO   5200

0B21        6B    107               10.7 RATIO   5600

0B22        68    104               10.4 RATIO   6000

0B23        68    104               10.4 RATIO   6400

#########################################################

#  F120                                                 #

#  POWER ENRICHMENT TPS THRESHOLD VS. COOLANT           #

#  TABLE VALUE =  %TPS * 2.56                           #

#########################################################

* -------------------------------------------------------------------

0B24        9A    154                 60 %TPS      44  DEG C-TEMP

0B25        9A    154                 60 %TPS      56

0B26        9A    154                 60 %TPS      68

0B27        9A    154                 60 %TPS      80

0B28        9A    154                 60 %TPS      92

0B29        9A    154                 60 %TPS     104

0B2A        8D    141                 55 %TPS     116

0B2B        66    102                 40 %TPS     128

0B2C        66    102                 40 %TPS     140

0B2D        66    102                 40 %TPS     152

####################################################

#  F122                                            #

# ALT. COMP. FOR PE TPS THRESHOLD VS. BARO         #

TABLE VALUE = E*2.56                           #

####################################################

* -------------------------------------------------------------------

0B2E        26     38               15 %TPS      65    KPA-BARO

0B2F        23     35               13.7 %TPS      70

0B30        20     32               12.5 %TPS      75

0B31        1D     29               11.3 %TPS      80

0B32        1A     26                 10 %TPS      85

0B33        11     17                6.6 %TPS      90

0B34        09      9                3.5 %TPS      95

0B35        00      0                  0 %TPS     100

0B36        00      0                  0 %TPS     105

0B37        04      4 KDETATH        1.6 %      DECEL ENLEANMENT DELTA THROT ANGLE THRES.

0B38        10     16 KDEPMTH          5 KPA    DECEL ENLEANMENT DELTA PRESSURE THRESHOLD

0B39        50     80 KAEDEBTM         1 SEC    AE TO DE BLEND TIME

0B3A        FF    255 KAEDEIBM     0.996 MULT   AE TO DE INITIAL BLEND MULTIPLIER

0B3B        08      8 KAEDEBDT      0.03 MULT   AE TO DE BLEND MULTIPLIER DELTA

0B3C        80    128 KPEDEBTM       1.6 SEC    PE TO DE BLEND TIME

0B3D        60     96 KPEDEIBM     0.375 MULT   PE TO DE INITIAL BLEND MULTIPLIER

0B3E        03      3 KPEDEBDT      0.01 MULT   PE TO DE BLEND MULTIPLIER DELTA

0B3F        F0    240 KDETCC       0.938 MULT   D.E. MULTIPLIER WHEN TCC LOCKED

0B40        80    128 KDECONVT         1 MULT   D.E. MULT WHEN CONV. O.T. PRESENT

##############################################################

# F35                                                        #

#    DELTA ENLEANMENT FACTOR VS. NUMBER OF INJECTS IN DE     #

#    TABLE VALUE = FACTOR * 256                              #

##############################################################

* -------------------------------------------------------------------

0B41        40     64              0.25 FACTOR     0     DE-INJECTS

0B42        34     52              0.203 FACTOR     1

0B43        2A     42              0.164 FACTOR     2

0B44        20     32              0.125 FACTOR     3

0B45        20     32              0.125 FACTOR     4

0B46        20     32              0.125 FACTOR     5

0B47        20     32              0.125 FACTOR     6

0B48        20     32              0.125 FACTOR     7

0B49        20     32              0.125 FACTOR     8

0B4A        20     32              0.125 FACTOR     9

0B4B        20     32              0.125 FACTOR    10

0B4C        20     32              0.125 FACTOR    11

0B4D        20     32              0.125 FACTOR    12

0B4E        20     32              0.125 FACTOR    13

0B4F        20     32              0.125 FACTOR    14

0B50        20     32              0.125 FACTOR    15

0B51        20     32              0.125 FACTOR    16

##############################################################

# F36                                                        #

#    DELTA ENLEANMENT FACTOR VS. DELTA THROTTLE              #

#    TABLE VALUE = FACTOR * 256                              #

##############################################################

* -------------------------------------------------------------------

0B52        50     80              0.313 FACTOR    00      %-TPS

0B53        50     80              0.313 FACTOR 06.25

0B54        50     80              0.313 FACTOR  12.5

0B55        50     80              0.313 FACTOR 18.75

0B56        50     80              0.313 FACTOR    25

0B57        50     80              0.313 FACTOR 31.25

0B58        50     80              0.313 FACTOR  37.5

0B59        50     80              0.313 FACTOR 43.75

0B5A        50     80              0.313 FACTOR    50

0B5B        50     80              0.313 FACTOR 56.25

0B5C        70    112              0.438 FACTOR  62.5

0B5D        70    112              0.438 FACTOR 68.75

0B5E        70    112              0.438 FACTOR    75

0B5F        70    112              0.438 FACTOR 81.25

0B60        70    112              0.438 FACTOR  87.5

0B61        70    112              0.438 FACTOR 93.75

0B62        70    112              0.438 FACTOR   100

##############################################################

# F38                                                        #

#    DECEL ENLEANMENT TERM MULTIPLIER VS. NMPH               #

#    TABLE VALUE = FACTOR * 256                              #

##############################################################

* -------------------------------------------------------------------

0B63        00      0                0 MULT       0    MPH-SPEED

0B64        15     21              0.082 MULT     2.5

0B65        15     21              0.082 MULT     5.0

0B66        15     21              0.082 MULT     7.5

0B67        15     21              0.082 MULT    10.0

0B68        15     21              0.082 MULT    12.5

0B69        15     21              0.082 MULT    15.0

0B6A        15     21              0.082 MULT    17.5

0B6B        15     21              0.082 MULT    20.0

0B6C        15     21              0.082 MULT    30.0

0B6D        15     21              0.082 MULT    40.0

0B6E        15     21              0.082 MULT    50.0

0B6F        15     21              0.082 MULT    60.0

##############################################################

# F39                                                        #

#    DELTA ENLEANMENT TERM MULTIPLIER VS. NMAPLD             #

#    TABLE VALUE = FACTOR * 128                              #

##############################################################

* -------------------------------------------------------------------

0B70        40     64               0.5 MULT      20    KPA-MAP

0B71        38     56               0.44 MULT      25

0B72        35     53              0.414 MULT      30

0B73        32     50               0.39 MULT      35

0B74        30     48              0.375 MULT      40

0B75        30     48              0.375 MULT      45

0B76        30     48              0.375 MULT      50

0B77        30     48              0.375 MULT      55

0B78        30     48              0.375 MULT      60

0B79        30     48              0.375 MULT      65

0B7A        30     48              0.375 MULT      70

0B7B        30     48              0.375 MULT      75

0B7C        30     48              0.375 MULT      80

0B7D        30     48              0.375 MULT      85

0B7E        30     48              0.375 MULT      90

0B7F        30     48              0.375 MULT      95

0B80        30     48              0.375 MULT     100

##############################################################

# F40                                                        #

#    DELTA ENLEANMENT FACTOR VS. DELTA MAP ABOVE KDEPMTH     #

#    TABLE VALUE = FACTOR * 256                              #

##############################################################

* -------------------------------------------------------------------

0B81        30     48              0.188 FACTOR     0    KPA-MAP

0B82        30     48              0.188 FACTOR     5

0B83        30     48              0.188 FACTOR    10

0B84        30     48              0.188 FACTOR    15

0B85        30     48              0.188 FACTOR    20

0B86        30     48              0.188 FACTOR    25

0B87        35     53              0.207 FACTOR    30

0B88        35     53              0.207 FACTOR    35

0B89        35     53              0.207 FACTOR    40

0B8A        35     53              0.207 FACTOR    45

0B8B        40     64               0.25 FACTOR    50

0B8C        40     64               0.25 FACTOR    55

0B8D        40     64               0.25 FACTOR    60

0B8E        40     64               0.25 FACTOR    65

0B8F        40     64               0.25 FACTOR    70

0B90        40     64               0.25 FACTOR    75

0B91        40     64               0.25 FACTOR    80

##############################################################

# F41                                                        #

#    DELTA ENLEANMENT TERM MULTIPLIER VS. COOLANT            #

#    TABLE VALUE = MULTIPLIER * 64                           #

##############################################################

* -------------------------------------------------------------------

0B92        10     16               0.25 MULT     -40  DEG C-TEMP

0B93        10     16               0.25 MULT     -28

0B94        10     16               0.25 MULT     -16

0B95        10     16               0.25 MULT      -4

0B96        10     16               0.25 MULT       8

0B97        20     32                0.5 MULT      20

0B98        20     32                0.5 MULT      32

0B99        20     32                0.5 MULT      44

0B9A        20     32                0.5 MULT      56

0B9B        20     32                0.5 MULT      68

0B9C        20     32                0.5 MULT      80

0B9D        20     32                0.5 MULT      92

0B9E        20     32                0.5 MULT     104

0B9F        20     32                0.5 MULT     116

0BA0        20     32                0.5 MULT     128

0BA1        20     32                0.5 MULT     140

0BA2        20     32                0.5 MULT     152

##############################################################

# F42                                                        #

#    DELTA ENLEANMENT TERM MULTIPLIER VS. RPM                #

#    TABLE VALUE = MULTIPLIER * 256                          #

##############################################################

* -------------------------------------------------------------------

0BA3        FF    255              0.996 MULT    1200    RPM-SPEED

0BA4        FF    255              0.996 MULT    1600

0BA5        FF    255              0.996 MULT    2000

0BA6        FF    255              0.996 MULT    2400

0BA7        FF    255              0.996 MULT    2800

DFCO CALS

0BA8        24     36 KDFCOMLL      23.6 KPA    ENABLE DFCO IF MAP<THIS  (LONG TIMER, LOWER HYST)

0BA9        24     36 KDFCOMLS      23.6 KPA    ENABLE DFCO IF MAP<THIS  (SHORT TIMER, LOWER HYST)

0BAA        60     96 KDFCOMHL      45.8 KPA    STAY IN DFCO IF MAP<THIS (LONG TIMER, UPPER HYST)

0BAB        60     96 KDFCOMHS      45.8 KPA    STAY IN DFCO IF MAP<THIS (SHORT TIMER, UPPER HYST)

RPM * 25  (1600/25=64d  $40)  1300 rpm=52=$34

0BAC        40     64 KDFCOSHL      1600 RPM    ENABLE DFCO IF RPM>THIS  (LONG TIMER, UPPER HYST)

0BAD        40     64 KDFCOSHS      1600 RPM    ENABLE DFCO IF RPM>THIS  (SHORT TIMER, UPPER HYST)

0BAE        2C     44 KDFCOSLL      1100 RPM    STAY IN DFCO IF RPM>THIS (LONG TIMER, LOWER HYST)

0BAF        2C     44 KDFCOSLS      1100 RPM    STAY IN DFCO IF RPM>THIS (SHORT TIMER, LOWER HYST)

0BB0        03      3 KDFCODRM        38 RPM    MAX RPM DECREASE IN 12.5 MSEC ALLOWED

TO REMAIN IN DFCO

0BB1        60     96 KDFCOTO1      19.2 SEC    ENABLE DFCO IF CONDITIONS MET FOR

TIME >= THIS

0BB2        80    128 KDFCOTM1      25.6 SEC    ENABLE DFCO IF CONDITIONS MET FOR

TIME >= THIS (IF CONVERTER  O.T. PRESENT)

0BB3        1E     30 KDFCOTM2         6 SEC    ENABLE DFCO IF CONDITIONS MET FOR

TIME >= THIS (IF MAP LOW AND  RPM HIGH)

0BB4        01      1 KDFCOINC         1 COUNTS NUMBER OF COUNTS DFCO WAIT TIMER

IS INCREMENTED BY IF   CONDITIONS ARE MET

0BB5        02      2 KDFCODEC         2 COUNTS NUMBER OF COUNTS DFCO WAIT TIMER

IS DECREMENTED BY IF CONDITIONS ARE MET

0BB6        84    132 KDFCORPM      3300 RPM    IF RPM > THIS AND MAP < KDFCOMAP,

THEN ENABLE THE SHORT DFCO  TIMER

0BB7        24     36 KDFCOMAP      23.6 KPA    IF MAP < THIS AND RPM > KDFCORPM,

THEN ENABLE THE SHORT DFCO TIMER

0BB8        08      8 KDFCOTPL         3 %      IF DFCO ALREADY ENABLED AND TPS BECOMES

>= THIS DISABLE DFCO

0BB9        05      5 KDFCOTPH         2 %      IF DFCO NOT ENABLED AND TPS >= THIS

DON'T ALLOW DFCO TO BE ENABLED

0BBA        8C    140 KDFCOOL         65 DEG C  IF COOLANT < THIS DISABLE DFCO

0BBB        14     20 KDFCOVL         20 MPH    IF MPH <= THIS, INHIBIT DFCO (SHORT, DFCO ON)

0BBC        19     25 KDFCOVH         25 MPH    IF MPH <= THIS, INHIBIT DFCO (SHORT, DFCO OFF)

0BBD        22     34 KDFCOVSL        34 MPH    IF MPH <= THIS, INHIBIT DFCO (LONG, DFCO ON)

0BBE        26     38 KDFCOVSH        38 MPH    IF MPH <= THIS, INHIBIT DFCO (LONG, DFCO OFF)

0BBF        80    128 KDFCOPIT       1.6 SEC    TIME DEL BEFOR REMOVING DFCO IAC STEPS

0BC0        29     41 KDFCOTPS        16 % TPS  DFCO ENTRY T/F TPS DEFAULT

0BC1        70    112 KSSTPS        43.8 % TPS  DFCO STALL SAVER T/F TPS DEFAULT

0BC2        10     16 KSSTPSTM       0.2 SEC    DFCO STALL SAVER T/F TPS DURATION

0BC3      0040     64 KSSAEPW       0.98 MSEC   DFCO STALL SAVER AE PULSE WIDTH

0BC5        03      3 KNUMPUL          3 PULSES NUMBER OF DFCO STALL SAVER AE PULSES

0BC6        A0    160 KSSTIME          2 SEC    MINIMUM TIME BETWEEN CONSECUTIVE DFCO'S

0BC7        19     25 KDFCOPI1       9.8  %     AFTER DFCO IAC STEPS HAVE BEEN ADDED

AND KDFCOPIT TIME HAS ELAPSED,  STEPS WILL BE REMOVED AT A RATE OF

KDFCOPI1  PERCENT PER 200 MSEC

0BC8        A0    160 KDFCOIN      0.625 MULT   DFCO FUEL RAMP IN FACTOR (A BIGGER #

RAMPS THE FUEL IN FASTER AFTER EXITING DFCO)

0BC9        A0    160 KDFCOINA     0.625 MULT   SAME AS KDFCOIN, BUT USED WHEN DFCO

WAS EXITED BECAUSE RPM BELOW KDFCOSPL

0BCA        F0    240 KDFCOOUT     0.938 MULT   DFCO FUEL RAMP OUT FACTOR (A BIGGER #  RAMPS THE FUEL

OUT SLOWER  AFTER ENTERING DFCO)

0BCB        F0    240 KADO2AF      0.938 COEF   MINOR LOOP O2 SENSOR FILTER CONSTANT (0-1)

0BCC        88    136 KFLMODF       0.53 FACTOR CLFLOW MAPP TERM SCALING FACTOR

0BCD        04      4 KFLESKF          2 KPA    CLFLOW ADDITIVE MAPP OFFSET

0BCE        05      5 KIDLFLT       0.02 COEF   12.5 MSEC O2 SENSOR FILT.COEFF (IDLE)

0BCF        66    102 KSO2RS        0.45 VOLTS  6.25 MSEC O2 FILTER NOMINAL VALUE WHEN

RESETTING INTEGRATOR

0BD0        0A     10 KIDLEINT      0.13  SEC   ADDITIVE CORRECTION TO INTDLY AT IDLE

0BD1        B0    176 KIDLESF      0.688 MULT   EECC CLOSED LOOP GAIN FACTOR*256

0BD2        1C     28 KFO2DLT      0.124 VOLTS  DELTA VALUE TO FORM RICH-LEAN

WINDOW FOR ADO2AF ("FAST O2")

0BD3        08      8 KHGTH        0.035 VOLTS  MIN ERROR TO IMPLEMENT INT CORRECTION

0BD4        64    100 KIDOXTH      0.442 VOLTS  RICH/LEAN O2 THRESHOLD WHEN IN IDLE

0BD5        70    112 KIDLZU       0.496 VOLTS  ZEREFU VALUE WHEN IDLING

0BD6        58     88 KIDLZL        0.39 VOLTS  ZEREFL VALUE WHEN IDLING

0BD7        08      8 KPCNT1           8 COUNTS PROPORTIONAL COUNTS WHEN IN PROPORTIONAL

DURATION #1

0BD8        0C     12 KPCNT2          12 COUNTS PROPORTIONAL COUNTS WHEN IN PROPORTIONAL

DURATION #2

0BD9        08      8 KIDLPD1        0.1 SEC    PROPORTIONAL DURATION #1 WHEN IDLING

0BDA        0C     12 KIDLPD2       0.15 SEC    PROPORTIONAL DURATION #2 WHEN IDLING

0BDB        80    128 KIDLGF         0.5 FACTOR PROPORTIONAL COUNTS GAIN FACTOR WHEN

IDLING

0BDC        E8    232 KPERMOD      0.906 MULT   GAIN FACTOR FOR POSITIVE ERRORS*256

0BDD        9A    154 KO2FLTCD       0.6 COEFF  USED TO FORM O2 FILTER CONSTANT

WHEN COOLANT IS COOL

0BDE        60     96 KCLDGF          72 DEG C  OFF-SET USED TO FORM COOL INT GAIN

#####################################

# F1CLTBLB       ZEREFU             #

# N = E*226                         #

* -------------------------------------------------------------------

0BDF        63     99              0.44 VOLTS      0   FLOW-GM/SEC

0BE0        64    100              0.442 VOLTS      8

0BE1        66    102               0.45 VOLTS     16

0BE2        67    103              0.456 VOLTS     24

0BE3        69    105              0.465 VOLTS     32

0BE4        6A    106               0.47 VOLTS     40

0BE5        6B    107              0.473 VOLTS     48

0BE6        6E    110              0.487 VOLTS     56

0BE7        6F    111               0.49 VOLTS     64

0BE8        73    115               0.51 VOLTS     72

0BE9        75    117              0.518 VOLTS     80

0BEA        77    119              0.526 VOLTS     88

0BEB        78    120               0.53 VOLTS     96

0BEC        78    120               0.53 VOLTS    104

0BED        78    120               0.53 VOLTS    112

0BEE        78    120               0.53 VOLTS    120

0BEF        78    120               0.53 VOLTS    128

#####################################

#  F2CLTBLB      ZEREFL             #

# N = E*226                         #

* -------------------------------------------------------------------

0BF0        56     86              0.38 VOLTS      0   FLOW-GM/SEC

0BF1        58     88               0.39 VOLTS      8

0BF2        5A     90                0.4 VOLTS     16

0BF3        5D     93               0.41 VOLTS     24

0BF4        60     96              0.425 VOLTS     32

0BF5        62     98              0.434 VOLTS     40

0BF6        63     99               0.44 VOLTS     48

0BF7        66    102               0.45 VOLTS     56

0BF8        69    105              0.465 VOLTS     64

0BF9        6E    110              0.487 VOLTS     72

0BFA        70    112              0.496 VOLTS     80

0BFB        72    114              0.504 VOLTS     88

0BFC        73    115               0.51 VOLTS     96

0BFD        73    115               0.51 VOLTS    104

0BFE        73    115               0.51 VOLTS    112

0BFF        73    115               0.51 VOLTS    120

0C00        73    115               0.51 VOLTS    128

#####################################

#  F3CLTBLB      CLOXTHA            #

# N = E*226                         #

* -------------------------------------------------------------------

0C01        60     96               0.425 VOLTS      0   FLOW-GM/SEC

0C02        61     97               0.43 VOLTS      8

0C03        63     99               0.44 VOLTS     16

0C04        64    100              0.442 VOLTS     24

0C05        65    101              0.447 VOLTS     32

0C06        66    102               0.45 VOLTS     40

0C07        67    103              0.456 VOLTS     48

0C08        6A    106               0.47 VOLTS     56

0C09        6C    108               0.48 VOLTS     64

0C0A        70    112              0.496 VOLTS     72

0C0B        72    114              0.504 VOLTS     80

0C0C        74    116              0.513 VOLTS     88

0C0D        75    117              0.518 VOLTS     96

0C0E        75    117              0.518 VOLTS    104

0C0F        75    117              0.518 VOLTS    112

0C10        75    117              0.518 VOLTS    120

0C11        75    117              0.518 VOLTS    128

#####################################

#  F4CLTBLB      INTDLY             #

# N = E*80                          #

* -------------------------------------------------------------------

0C12        2C     44               0.55 SEC        0   FLOW-GM/SEC

0C13        2A     42               0.52 SEC        8

0C14        28     40                0.5 SEC       16

0C15        26     38               0.47 SEC       24

0C16        24     36               0.45 SEC       32

0C17        20     32                0.4 SEC       40

0C18        1C     28               0.35 SEC       48

0C19        18     24                0.3 SEC       56

0C1A        16     22               0.27 SEC       64

0C1B        19     25               0.31 SEC       72

0C1C        0C     12               0.15 SEC       80

0C1D        0B     11               0.14 SEC       88

0C1E        0A     10               0.13 SEC       96

0C1F        0A     10               0.13 SEC      104

0C20        09      9               0.11 SEC      112

0C21        08      8                0.1 SEC      120

0C22        08      8                0.1 SEC      128

#####################################

#  F5CLTBLB      SO2FILT            #

#  N = E*226                        #

* -------------------------------------------------------------------

0C23        08      8              0.035 VOLTS      0   FLOW-GM/SEC

0C24        08      8              0.035 VOLTS      8

0C25        08      8              0.035 VOLTS     16

0C26        08      8              0.035 VOLTS     24

0C27        09      9               0.04 VOLTS     32

0C28        09      9               0.04 VOLTS     40

0C29        0A     10              0.044 VOLTS     48

0C2A        0B     11               0.05 VOLTS     56

0C2B        0C     12              0.053 VOLTS     64

0C2C        0D     13              0.057 VOLTS     72

0C2D        0E     14               0.06 VOLTS     80

0C2E        10     16               0.07 VOLTS     88

0C2F        12     18               0.08 VOLTS     96

0C30        14     20               0.09 VOLTS    104

0C31        16     22              0.097 VOLTS    112

0C32        17     23                0.1 VOLTS    120

0C33        18     24              0.106 VOLTS    128

#####################################

#  F6CLTBLB  PROPORTIONAL ERROR     #

#    GAIN FACTOR VS. CLFLOW         #

#    TABLE VALUE = FACTOR * 256     #

* -------------------------------------------------------------------

0C34        10     16               0.063 MULT       0   FLOW-GM/SEC

0C35        2A     42              0.164 MULT       8

0C36        30     48              0.188 MULT      16

0C37        50     80              0.313 MULT      24

0C38        50     80              0.313 MULT      32

0C39        50     80              0.313 MULT      40

0C3A        50     80              0.313 MULT      48

0C3B        5A     90               0.35 MULT      56

0C3C        32     50              0.195 MULT      64

0C3D        3E     62              0.242 MULT      72

0C3E        4A     74               0.29 MULT      80

0C3F        65    101              0.394 MULT      88

0C40        80    128                0.5 MULT      96

0C41        95    149              0.582 MULT     104

0C42        AA    170              0.664 MULT     112

0C43        B2    178              0.695 MULT     120

0C44        BA    186              0.727 MULT     128

#####################################

#  F8CLTBLA  RICH-LEAN OFFSET       #

# (COOLDEG)                         #

# N = E+128                         #

* -------------------------------------------------------------------

0C45        80    128                  0 COUNTS   -28  DEG C-TEMP

0C46        80    128                  0 COUNTS   -16

0C47        80    128                  0 COUNTS    -4

0C48        80    128                  0 COUNTS     8

0C49        6C    108                -20 COUNTS    20

0C4A        64    100                -28 COUNTS    32

0C4B        64    100                -28 COUNTS    44

0C4C        70    112                -16 COUNTS    56

0C4D        70    112                -16 COUNTS    68

0C4E        78    120                 -8 COUNTS    80

0C4F        80    128                  0 COUNTS    92

0C50        80    128                  0 COUNTS   104

0C51        80    128                  0 COUNTS   116

#####################################

#  F9CLTBL       PCCNT              #

#  PROPORTIONAL COUNTS (ERROR)      #

#  N = E                            #

* -------------------------------------------------------------------

0C52        12     18                18 COUNTS 0 ERROR

0C53        18     24                 24 COUNTS 8

0C54        1A     26                 26 COUNTS 16

0C55        1E     30                 30 COUNTS 24

0C56        20     32                 32 COUNTS 32

0C57        22     34                 34 COUNTS 40

0C58        24     36                 36 COUNTS 48

0C59        28     40                 40 COUNTS 56

0C5A        30     48                 48 COUNTS 64

0C5B        38     56                 56 COUNTS 72

0C5C        40     64                 64 COUNTS 80

0C5D        48     72                 72 COUNTS 88

#####################################

#  F10CLTBL  RICH/LEAN OFF-SET      #

# (CLDEGFLT)                        #

# N = E+128                         #

* -------------------------------------------------------------------

0C5E        80    128                  0 COUNTS   -28  DEG C-TEMP

0C5F        80    128                  0 COUNTS   -16

0C60        80    128                  0 COUNTS    -4

0C61        80    128                  0 COUNTS     8

0C62        6C    108                -20 COUNTS    20

0C63        64    100                -28 COUNTS    32

0C64        64    100                -28 COUNTS    44

0C65        70    112                -16 COUNTS    56

0C66        70    112                -16 COUNTS    68

0C67        78    120                 -8 COUNTS    80

0C68        80    128                  0 COUNTS    92

0C69        80    128                  0 COUNTS   104

0C6A        80    128                  0 COUNTS   116

#####################################

#  F11CLTBL GAIN FACTOR FOR INTDLY  #

#  (ERROR) =T.V./256                #

#  N = E                            #

* -------------------------------------------------------------------

0C6B        E0    224               224 MULT   0 ERROR

0C6C        E0    224                224 MULT   8

0C6D        E0    224                224 MULT   16

0C6E        C0    192                192 MULT   24

0C6F        A0    160                160 MULT   32

0C70        80    128                128 MULT   40

0C71        60     96                 96 MULT   48

0C72        50     80                 80 MULT   56

0C73        50     80                 80 MULT   64

0C74        50     80                 80 MULT   72

0C75        4A     74                 74 MULT   80

0C76        48     72                 72 MULT   88

#####################################

#  F12CLTBL  PROP. DURATION #1 VS.  #

#    CLFLOW  (WHEN NOT IDLING)      #

#    N = E*80                       #

* -------------------------------------------------------------------

0C77        10     16               0.2 SEC        0   FLOW-GM/SEC

0C78        0E     14               0.17 SEC       16

0C79        0E     14               0.17 SEC       32

0C7A        0C     12               0.15 SEC       48

0C7B        0C     12               0.15 SEC       64

0C7C        0A     10               0.13 SEC       80

0C7D        08      8                0.1 SEC       96

0C7E        06      6               0.07 SEC      112

0C7F        03      3               0.04 SEC      128

#####################################

#  F13CLTBL  PROP. DURATION #2 VS.  #

#    CLFLOW  (WHEN NOT IDLING)      #

#    N = E*80                       #

* -------------------------------------------------------------------

0C80        18     24               0.3 SEC        0   FLOW-GM/SEC

0C81        12     18               0.22 SEC       16

0C82        0F     15               0.19 SEC       32

0C83        0C     12               0.15 SEC       48

0C84        09      9               0.11 SEC       64

0C85        08      8                0.1 SEC       80

0C86        07      7               0.09 SEC       96

0C87        06      6               0.07 SEC      112

0C88        05      5               0.06 SEC      128

0C89        20     32 KIDLPGLO       $20 UNIT   LO PROP GAIN (USED IN PID/EECC CROSSTALK)

0C8A        30     48 KIDLPGNM       $30 UNIT   NOMINAL PROP GAIN (USED IN PID/EECC CROSSTALK)

0C8B        30     48 KIDLPGHI       $30 UNIT   HI PROP GAIN (USED IN PID/EECC CROSSTALK)

0C8C        30     48 KIDLPGXH       $30 UNIT   XTRA HI PROP GAIN (USED IN PID/EECC CROSSTALK)

0C8D        18     24 KIDINTLO       $18 UNIT   LO INT GAIN (USED IN PID/EECC CROSSTALK)

0C8E        10     16 KIDINTNM       $10 UNIT   NOMINAL INT GAIN (USED IN PID/EECC CROSSTALK)

0C8F        0C     12 KIDINTHI       $0C UNIT   HI INT GAIN (USED IN PID/EECC CROSSTALK)

0C90        04      4 KIDINTXH       $04 UNIT   XTRA HI INT GAIN (USED IN PID/EECC CROSSTALK)

0C91        10     16 KISHIRPM       $C8 RPM    RPM THRESHOLD

0C92        05      5 KISHIRAT       $4E R/SEC  RPM RATE OF CHANGE THRESHOLD

#####################################

#  F128     IDLE GAIN TABLES        #

#####################################

* -------------------------------------------------------------------

0C93        01      1                $01 UNITS  $0

0C94        00      0                $00 UNITS  $0

0C95        00      0                $00 UNITS  $0

0C96        00      0                $00 UNITS  $0

0C97        01      1                $01 UNITS  $4

0C98        01      1                $01 UNITS  $5

0C99        00      0                $00 UNITS  $0

0C9A        00      0                $00 UNITS  $0

0C9B        01      1                $01 UNITS  $8

0C9C        01      1                $01 UNITS  $9

0C9D        00      0                $00 UNITS  $0

0C9E        00      0                $00 UNITS  $0

0C9F        00      0                $00 UNITS  $0

0CA0        00      0                $00 UNITS  $0

0CA1        00      0                $00 UNITS  $0

0CA2        00      0                $00 UNITS  $0

0CA3        00      0                $00 UNITS  $10

0CA4        00      0                $00 UNITS  $0

0CA5        00      0                $00 UNITS  $12

0CA6        05      5                $05 UNITS  $0

0CA7        01      1                $01 UNITS  $14

0CA8        05      5                $05 UNITS  $15

0CA9        01      1                $01 UNITS  $16

0CAA        01      1                $01 UNITS  $17

0CAB        02      2                $02 UNITS  $18

0CAC        05      5                $05 UNITS  $19

0CAD        01      1                $01 UNITS  $1A

0CAE        00      0                $00 UNITS  $1B

0CAF        00      0                $00 UNITS  $0

0CB0        00      0                $00 UNITS  $0

0CB1        00      0                $00 UNITS  $0

0CB2        00      0                $00 UNITS  $0

0CB3        02      2                $02 UNITS  $20

0CB4        00      0                $00 UNITS  $0

0CB5        02      2                $02 UNITS  $22

0CB6        00      0                $00 UNITS  $0

0CB7        02      2                $02 UNITS  $24

0CB8        02      2                $02 UNITS  $25

0CB9        02      2                $02 UNITS  $26

0CBA        02      2                $02 UNITS  $27

0CBB        02      2                $02 UNITS  $28

0CBC        02      2                $02 UNITS  $29

0CBD        02      2                $02 UNITS  $2A

0CBE        02      2                $02 UNITS  $2B

0CBF        00      0                $00 UNITS  $0

0CC0        00      0                $00 UNITS  $0

0CC1        00      0                $00 UNITS  $0

0CC2        00      0                $00 UNITS  $0

0CC3        00      0                $00 UNITS  16 BYTES OF BLANK SPACE

0CC4        00      0                $00 UNITS

0CC5        00      0                $00 UNITS

0CC6        00      0                $00 UNITS

0CC7        00      0                $00 UNITS

0CC8        00      0                $00 UNITS

0CC9        00      0                $00 UNITS

0CCA        00      0                $00 UNITS

0CCB        00      0                $00 UNITS

0CCC        00      0                $00 UNITS

0CCD        00      0                $00 UNITS

0CCE        00      0                $00 UNITS

0CCF        00      0                $00 UNITS

0CD0        00      0                $00 UNITS

0CD1        00      0                $00 UNITS

0CD2        00      0                $00 UNITS

0CD3        01      1                $01 UNITS  $40

0CD4        00      0                $00 UNITS  $0

0CD5        00      0                $00 UNITS  $0

0CD6        00      0                $00 UNITS  $0

0CD7        01      1                $01 UNITS  $44

0CD8        01      1                $01 UNITS  $45

0CD9        00      0                $00 UNITS  $0

0CDA        00      0                $00 UNITS  $0

0CDB        01      1                $01 UNITS  $48

0CDC        05      5                $05 UNITS  $49

0CDD        00      0                $00 UNITS  $0

0CDE        00      0                $00 UNITS  $0

0CDF        00      0                $00 UNITS  $0

0CE0        00      0                $00 UNITS  $0

0CE1        00      0                $00 UNITS  $0

0CE2        00      0                $00 UNITS  $0

0CE3        01      1                $01 UNITS  $50

0CE4        00      0                $00 UNITS  $0

0CE5        02      2                $02 UNITS  $52

0CE6        00      0                $00 UNITS  $0

0CE7        02      2                $02 UNITS  $54

0CE8        05      5                $05 UNITS  $55

0CE9        02      2                $02 UNITS  $56

0CEA        05      5                $05 UNITS  $57

0CEB        02      2                $02 UNITS  $58

0CEC        03      3                $03 UNITS  $59

0CED        03      3                $03 UNITS  $5A

0CEE        03      3                $03 UNITS  $5B

0CEF        00      0                $00 UNITS  $0

0CF0        00      0                $00 UNITS  $0

0CF1        00      0                $00 UNITS  $0

0CF2        00      0                $00 UNITS  $0

0CF3        03      3                $03        $60

0CF4        00      0                $00        $0

0CF5        03      3                $03        $62

0CF6        00      0                $00        $0

0CF7        02      2                $02        $64

0CF8        03      3                $03        $65

0CF9        03      3                $03        $66

0CFA        03      3                $03        $67

0CFB        03      3                $03        $68

0CFC        03      3                $03        $69

0CFD        07      7                $07        $6A

0CFE        07      7                $07        $6B

0CFF        00      0                $00        $0

0D00        00      0                $00        $0

0D01        00      0                $00        $0

0D02        00      0                $00        $0

************************************************************

*  F87RPMUS UNDERSPEED PROPORTION FEEDBACK VS. RPM ERROR   *

*  TABLE VALUE = UNITS                                     *

************************************************************

* -------------------------------------------------------------------

0D03        00      0                 0 UNITS      0    RPM-SPEED

0D04        00      0                  0 UNITS   12.5

0D05        00      0                  0 UNITS   25.0

0D06        00      0                  0 UNITS   37.5

0D07        01      1                  1 UNITS   50.0

0D08        01      1                  1 UNITS   62.5

0D09        02      2                  2 UNITS   75.0

0D0A        02      2                  2 UNITS   87.5

0D0B        02      2                  2 UNITS  100.0

0D0C        02      2                  2 UNITS  112.5

0D0D        03      3                  3 UNITS  125.0

0D0E        03      3                  3 UNITS  137.5

0D0F        03      3                  3 UNITS  150.0

0D10        04      4                  4 UNITS  162.5

0D11        04      4                  4 UNITS  175.0

0D12        05      5                  5 UNITS  187.5

0D13        06      6                  6 UNITS  200.0

************************************************************

*  F87RPMOS OVERSPEED PROPORTIONAL FEEDBACK VS. RPM ERROR  *

*  TABLE VALUE = UNITS                                     *

************************************************************

* -------------------------------------------------------------------

0D14        00      0                  0 UNITS   12.5    RPM-SPEED

0D15        00      0                  0 UNITS   25.0

0D16        01      1                  1 UNITS   37.5

0D17        01      1                  1 UNITS   50.0

0D18        01      1                  1 UNITS   62.5

0D19        01      1                  1 UNITS   75.0

0D1A        02      2                  2 UNITS   87.5

0D1B        02      2                  2 UNITS  100.0

0D1C        02      2                  2 UNITS  112.5

0D1D        02      2                  2 UNITS  125.0

0D1E        02      2                  2 UNITS  137.5

0D1F        02      2                  2 UNITS  150.0

0D20        03      3                  3 UNITS  162.5

0D21        03      3                  3 UNITS  175.0

0D22        03      3                  3 UNITS  187.5

0D23        03      3                  3 UNITS  200.0

0D24        03      3                  3 UNITS  212.5

*****************************************************************

*  F87RATD SPEED DECREASING DERIVATIVE FEEDBACK VS. RPM ERROR   *

*  TABLE VALUE =                                                *

*****************************************************************

* -------------------------------------------------------------------

0D25        00      0                 0 UNITS      0  RPM/S-RATE

0D26        00      0                  0 UNITS  15.62

0D27        00      0                  0 UNITS  31.25

0D28        00      0                  0 UNITS  46.87

0D29        00      0                  0 UNITS   62.5

0D2A        01      1                  1 UNITS  78.12

0D2B        01      1                  1 UNITS  93.75

0D2C        01      1                  1 UNITS  109.37

0D2D        01      1                  1 UNITS  125.0

0D2E        02      2                  2 UNITS  140.62

0D2F        03      3                  3 UNITS  156.25

0D30        03      3                  3 UNITS  171.87

0D31        04      4                  4 UNITS  187.5

0D32        04      4                  4 UNITS  203.12

0D33        04      4                  4 UNITS  218.75

0D34        04      4                  4 UNITS  234.37

0D35        04      4                  4 UNITS  250.0

*****************************************************************

*  F87RATI SPEED INCREASING DERIVATIVE FEEDBACK VS. RPM ERROR   *

*  TABLE VALUE = CORRECTION FACTOR                              *

*****************************************************************

* -------------------------------------------------------------------

0D36        00      0 F87RATI          0 UNITS      0  RPM/S-RATE

0D37        00      0                  0 UNITS  15.62

0D38        00      0                  0 UNITS  31.25

0D39        00      0                  0 UNITS  46.87

0D3A        01      1                  1 UNITS   62.5

0D3B        01      1                  1 UNITS  78.12

0D3C        01      1                  1 UNITS  93.75

0D3D        02      2                  2 UNITS  109.37

0D3E        02      2                  2 UNITS  125.0

0D3F        03      3                  3 UNITS  140.62

0D40        03      3                  3 UNITS  156.25

0D41        04      4                  4 UNITS  171.87

0D42        04      4                  4 UNITS  187.5

0D43        05      5                  5 UNITS  203.12

0D44        05      5                  5 UNITS  218.75

0D45        05      5                  5 UNITS  234.37

0D46        05      5                  5 UNITS  250.0

0D47        01      1 KISTRDY1      0.05 SEC    DELAY COMMMAND SPEED CHANGE AFTER TRANSITION

TO OR FROM P/N

##### IDLE SPEED STARTUP DELAY

0D48        E0      224 KISSUDL       1400 MSEC   IDLE SPEED START-UP DELAY

0D49        04      4 KISACON         50 RPM    IAC COMMAND SPEED OFFSET WITH A/C ON

0D4A        0A     10 KISMPHLO        10 MPH    MPH THRESH BELO WHICH FILT ISACANF TO ISACAN

0D4B        0F     15 KISMPHHI        15 MPH    MPH THRESH ABOV WHICH FILT ISACANF TO KISACMN

0D4C        80    128 KACFIL1        0.5 CONST  FILT CONST FOR ISACANF WHEN MPH < KISMPHLO

******************************************

*  F88 : HI MPH FILT CONST FOR ISACANF   *

*         VS. MANIFOLD AIR TEMP (DEG C)  *

*  TABLE VALUE = CONST*256               *

******************************************

* -------------------------------------------------------------------

0D4D        14     20               0.08 CONST      8  DEG C-MAT

0D4E        14     20               0.08 CONST     20

0D4F        14     20               0.08 CONST     32

0D50        14     20               0.08 CONST     44

0D51        14     20               0.08 CONST     56

0D52        14     20               0.08 CONST     68

0D53        14     20               0.08 CONST     80

******************************************

*  F89 : ANTICIPATED A/C LOAD STEPS      *

*         VS. A/C PRESSURE (PSI)         *

*  TABLE VALUE = STEPS                   *

******************************************

* -------------------------------------------------------------------

0D54        0A     10             10 STEPS     80    PSI-ACPRESS

0D55        0C     12                 12 STEPS     96

0D56        0E     14                 14 STEPS    112

0D57        10     16                 16 STEPS    128

0D58        12     18                 18 STEPS    144

0D59        14     20                 20 STEPS    160

0D5A        16     22                 22 STEPS    176

0D5B        18     24                 24 STEPS    192

0D5C        1A     26                 26 STEPS    208

0D5D        04      4 KISCLTHI        50 RPM    RPM OFFSET FROM COMMAND FOR HIGH COOLANT

0D5E        8C    140 KISCLTLM        65 DEG C  IF COOLANT <= THIS DISABLE HOT OFFSET

0D5F        14     20 KISMPHLM        20 MPH    IF MPH >= THIS DISABLE HOT OFFSET

0D60      00C8    200 KISCLTTM       200 SEC    IF ERUNTIME >= THIS DISABLE HOT OFFSET

0D62        01      1 KISMAXCH        13 RPM    MAX COMMAND SPEED CHANGE PER 50 MSEC

0D63        50     80 KCMSPDPT      1000 RPM    CHECK MIN COMM. SPD DELTA IF PART THROTTLE

AND COMMAND SPEED <= THIS

0D64        08      8 KPTSPOFS       100 RPM    MIN DELTA COMMAND SPEED FOR PART THROTTLE

0D65        FE    254 KMAXICOM      3175 RPM    MAX COMMAND SPEED CHANGE PER 50 MSEC

0D66        00      0 KHWSPD           0 RPM    COMMAND SPEED FOR HEATED WINDSHIELD MODE

0D67        88    136 KMAXCSPD      1700 RPM    MAX COMMAND SPEED DURING LO BAT COMPENSATION

0D68        01      1 KBATDEL         13 RPM    DELTA COMM SPEED PER 200 MS FOR LO BAT VOLT

0D69        04      4 KBATDLMX        50 RPM    MAX DELTA COMM SPEED FOR LO BAT VOLT

0D6A        64    100 KBATCOML        10 VOLT   LO BAT VOLT THRESH TO INCREASE COMM SPEED

0D6B        80    128 KBATCOMH      12.8 VOLT   LO BAT VOLT THRESH TO INCREASE COMM SPEED

***************************************

*  F17A : ENGINE SPEED AT IDLE        *

*          VS. TEMPERATURE IN DRIVE   *

*  TABLE VALUE = RPM/12.5             *

***************************************

* -------------------------------------------------------------------

0D6C        88    136              1700 RPM      -40  DEG C-TEMP

0D6D        80    128               1600 RPM      -28

0D6E        70    112               1400 RPM      -16

0D6F        6C   108               1350 RPM       -4

0D70        68    104               1300 RPM        8

0D71        64    100               1250 RPM       20

0D72        64    100               1250 RPM       32

0D73        5C     92               1150 RPM       44

0D74        50     80               1000 RPM       56

0D75        48     72                900 RPM       68                800 rpm = $40

0D76        46     70                875 RPM       80                650 rpm = $34

0D77        44     68                850 RPM       92                650 rpm = $34

0D78        44     68                850 RPM      104                650 rpm = $34

0D79        45     69                863 RPM      116                700 rpm = $38

0D7A        46     70                875 RPM      128                875 rpm = $46

0D7B        48     72                900 RPM      140                875 rpm = $46

0D7C        48     72                900 RPM      152

***************************************

*  F19A : ENGINE SPEED AT IDLE        *

*          VS. TEMPERATURE IN NEUTRAL *

*  TABLE VALUE = RPM/12.5             *

***************************************

* -------------------------------------------------------------------

0D7D        88    136              1700 RPM      -40  DEG C-TEMP

0D7E        80    128               1600 RPM      -28

0D7F        70    112               1400 RPM      -16

0D80        6C    108               1350 RPM       -4

0D81        68    104               1300 RPM        8

0D82        64    100               1250 RPM       20

0D83        64    100               1250 RPM       32

0D84        5C     92               1150 RPM       44

0D85        50     80               1000 RPM       56

0D86        48     72                900 RPM       68

0D87        46     70                875 RPM       80

0D88        44     68                850 RPM       92

0D89        44     68                850 RPM      104

0D8A        45     69                863 RPM      116

0D8B        46     70                875 RPM      128

0D8C        48     72                900 RPM      140

0D8D        48     72                900 RPM      152

---------------------------

0D8E        06      6 KISACLD         75 RPM    IDLE SPEED DEADBAND FOR A/C & MIN MOT. LEARN

0D8F        06      6 KACSUOFF         6 STEPS  IAC STEPS ADDED AT START UP IF A/C ON

0D90        05      5 KISTATH          2 % TPS  ISC T.A. THRESHOLD BELOW WHICH ISC IS ENABLED

0D91        04      4 KISADTPS       1.6 ADTPS  ADTHROT-ADTAOFF THRESH BELO WHICH ISC IS ENABL

0D92        04      4 KISVSTH        1.3 MPH    VEHICLE SPEED THRESH FOR THE PID ENABLE

0D93        04      4 KISRETH         50 RPM    RPM ERROR UNDERSPEED THRESHOLD NEEDED

BEFORE ENABLING THE PID REGULATOR,  DURING A TRANSITION OF CONTROL

MODES.

0D94        1A     26 KISNDED        406 RPM/S  RPM RATE (NDOT) TO DISABLE INT WHILE IN

DRIVE

0D95        1A     26 KISNDEN        406 RPM/S  RPM RATE (NDOT) TO DISABLE INT WHILE IN

NEUTRAL

************************************************************

*  F121 # OF INJECTS BETWEEN STEPS DECAYED FROM ISTIMOUT   *

*  TABLE VALUE = INJECTS                                   *

************************************************************

* -------------------------------------------------------------------

0D96        38     56                 56 INJ      -40  DEG C-TEMP

0D97        30     48                 48 INJ      -16

0D98        18     24                 24 INJ        8

0D99        18     24                 24 INJ       32

0D9A        15     21                 21 INJ       56

0D9B        10     16                 16 INJ       80

0D9C        10     16                 16 INJ      104

0D9D        18     24                 24 INJ      128

0D9E        1A     26                 26 INJ      152

************************************************************

*  F125POS  INTEGRAL GAIN RATES VS. POSITIVE RPM ERROR     *

*  TABLE VALUE = UNITS                                     *

************************************************************

* -------------------------------------------------------------------

0D9F        00      0               0 UNITS      0    RPM-SPEED

0DA0        01      1               0.01 UNITS   12.5

0DA1        02      2               0.02 UNITS   25.0

0DA2        03      3               0.04 UNITS   37.5

0DA3        03      3               0.04 UNITS   50.0

0DA4        03      3               0.04 UNITS   62.5

0DA5        03      3               0.04 UNITS   75.0

0DA6        04      4               0.05 UNITS   87.5

0DA7        04      4               0.05 UNITS  100.0

0DA8        04      4               0.05 UNITS  112.5

0DA9        04      4               0.05 UNITS  125.0

0DAA        08      8                0.1 UNITS  137.5

************************************************************

*  F125NEG  INTEGRAL GAIN RATES VS. NEGATIVE RPM ERROR     *

*  TABLE VALUE = UNITS                                     *

************************************************************

* -------------------------------------------------------------------

0DAB        00      0               0 UNITS      0    RPM-SPEED

0DAC        01      1               0.01 UNITS   12.5

0DAD        03      3               0.04 UNITS   25.0

0DAE        03      3               0.04 UNITS   37.5

0DAF        04      4               0.05 UNITS   50.0

0DB0        04      4               0.05 UNITS   62.5

0DB1        04      4               0.05 UNITS   75.0

0DB2        04      4               0.05 UNITS   87.5

0DB3        04      4               0.05 UNITS  100.0

0DB4        04      4               0.05 UNITS  112.5

0DB5        04      4               0.05 UNITS  125.0

0DB6        04      4               0.05 UNITS  137.5

0DB7        A0    160 KISPNGN      0.625 STEPS  ISC PARK/NEUT SCALAR GAIN

(N = E * 256); (NETURAL STEPS)  /(DRIVE STEP)

0DB8        0A     10 KISMPSN         63 MSEC   MIN NO OF 6.25 MSEC LOOPS PER P/N

DECAY STEP WHEN IN OPEN LOOP MODE

0DB9        04      4 KISMPSD         25 MSEC   MIN NO OF 6.25 MSEC LOOPS PER DRIVE

DECAY STEP WHEN IN OPEN LOOP MODE

0DBA        04      4 KISMPSTF        25 MSEC   MIN NO OF 6.25 MSEC LOOPS PER P/N

DECAY STEP WHEN IN THROTTLE FOLLOWER

##########################################

#  F129 TABLE                            #

#    THROTTLE FOLLOWER DECAY RATE        #

#      VS. MPH (WHEN NOT IN P/N)         #

##########################################

* -------------------------------------------------------------------

0DBB        03      3                 19 MSEC       0    MPH-SPEED

0DBC        05      5                 31 MSEC       4

0DBD        07      7                 44 MSEC       8

0DBE        07      7                 44 MSEC      12

0DBF        07      7                 44 MSEC      16

0DC0        07      7                 44 MSEC      20

0DC1        05      5                 31 MSEC      24

0DC2        05      5                 31 MSEC      28

0DC3        05      5                 31 MSEC      32

0DC4        05      5                 31 MSEC      36

0DC5        05      5                 31 MSEC      40

0DC6        05      5                 31 MSEC      44

0DC7        05      5                 31 MSEC      48

0DC8        05      5                 31 MSEC      52

0DC9        05      5                 31 MSEC      56

0DCA        05      5                 31 MSEC      60

0DCB        05      5                 31 MSEC      64

0DCC        40     64 KISCODM        400 MSEC   IDLE SPEED MAX NO OF LOOPS PER DECAY

STEP IN THE OPEN LOOP MODE.

0DCD        00      0 KISCONV          0 MSEC   IDLE SPEED T.F. STEP DECAY OFFSET

WHEN CONVERTER OVERTEMP INDICATED

0DCE        70    112 KISTFGN       2.24         'STP/%TP' IDLE SPEED FOLLOWER SLOPE GAIN

(N = E * 50); STEPS/(PERCENT TPS)

0DCF        FF    255 KISTFPN      0.996 STEPS  IDLE SPEED THROTTLE FOLLOWER P/N SCALER

(N = E * 256); (NEUTRAL STEPS) /(DRIVE STEPS)

0DD0        0A     10 KDFLTPS          4 %TPS   DEFAULT NTPSLD USED IN THROTTLE

FOLLOWER IF THERE IS A MALF 21 OR 22

0DD1        02      2 KTFMVLMT         2 STEPS  IDLE SPEED THROTTLE FOLLOWER MIN STEPS

(N = E); (MINIMUM STEPS)

0DD2        18     24 KISACDS         24 STEPS  IDLE SPEED A/C COMPENSATION DEFAULT STEPS

0DD3        04      4 KACMXLRN         4 STEPS  MAX CHANGE IN A/C LEARNED LOAD STEPS

0DD4        23     35 KISACMX         35 STEPS  IDLE SPEED A/C COMPENSATION MAX NO OF STEPS

0DD5        0A     10 KISACMN         10 STEPS  IDLE SPEED A/C COMPENSATION MIN NO OF STEPS

0DD6        1E     30 KISACLE       1500 MSEC   IDLE SPEED ENGINE SPEED IN DEADBAND

( NO. OF PID CYCLES IN THE DEADBAND BEFORE

ENABLING OR DISABLING A/C LEARNING)

0DD7        20     32 KISMMLE       1600 MSEC   IDLE SPEED ENGINE SPEED IN DEADBAND

( NO. OF PID CYCLES IN THE DEADBAND BEFORE ENABLING OR DISABLING MIN

MOT LEARNING)

0DD8        80    128 KISINTP          1 STEPS  INVERSE THROTTLE FOLLOWER P/N GAIN (DRIVE STEPS)/(NEUTRAL

STEP)= (1/KISTFPN)  KISQUGN AND KISINQU MUST MAINTAIN THE

MATHEMATICAL RELATIONSHIP SPECIFIED IN THE COMMENTS BELOW IN

ORDER FOR THE IAC MOTOR STEPS CALCULATIONS TO WORK AS EXPECTED.

0DD9        2B     43 KISQUGN      0.168        QAUNTIZER GAIN TERM (MUST EQUAL 256/KISINQU)

0DDA        06      6 KISINQU          6        PID COUNTS PER STEP (MUST EQUAL 256/KISQUGN)

0DDB        05      5 KTFDDTPS         2 % TPS  IF NEGATIVE DELTA THROTTLE > THIS, THEN DELAY REMOVAL OF

THE  THROT FOLL STEPS

0DDC        80    128 KTFDDTIM       0.8 SEC    INIT VALUE OF THE THROTTLE FOLLOWER DECAY DELAY TIMER

IAC PKA and MISC

0DDD        28     40 KISOLDY       2000 MSEC   OPEN-LOOP TO CLOSED- LOOP ENABLE DELAY TIME

0DDE        06      6 KISTFDY        300 MSEC   THROTTLE-FOLLOWER TO PID-ENABLE DELAY TIME

0DDF        05      5 KISTPSD        250 MSEC   PSPS CRAMP PID-ENABLE DELAY TIME

0DE0        06      6 KISDNDY        300 MSEC   DRIVE-TO-NEUTRAL SHIFT TO PID-ENABLE DELAY TIME

0DE1        06      6 KISNDDY        300 MSEC   NEUTRAL-TO-DRIVE SHIFT TO PID-ENABLE DELAY TIME

0DE2        01      1 KISMAN           1 FLAG   MANUAL VEHICLE OPTION FLAG

0DE3        10     16 KISMANOF        16 STEPS  THROTTLE FOLLOWER OFFSET FOR A MANUAL VEHICLE, IF THE

VEHICLE IS MOVING.

0DE4        8C    140 KISMXSP        140 STEPS  MAXIMUM STEP MOTOR POSITION POSSIBLE,

WORST CASE VALUE:STEPS FROM  THE ORIFICE

0DE5        50     80 KIACPARK        80 STEPS  PARK START UP POSITION

0DE6        30     48 KMXMNSP         48 STEPS  MAXIMUM KEEP ALIVE MINIMUM MOT POS

0DE7        80    128 KNVMPKAT        56 DEGC   LOW THRESH OF COOL TEMP WINDOW FOR KEEP ALIVE MIN MOT

UPDATING AFTER NVM FAIL

0DE8        A9    169 KMNMPCLD        87 DEGC   LOW THRESH OF COOL TEMP WINDOW FOR  KEEP ALIVE MIN

MOT UPDATING

0DE9        C5    197 KMNMPHOT       108 DEGC   HI THRESH OF COOL TEMP WINDOW FOR KEEP ALIVE MIN MOT

UPDATING

0DEA        02      2 KTPSPKAL       0.8 %TPS   DISABLE PKA UPDATE IF TPS > THIS

0DEB        03      3 KISMNMPF      0.01 CONST  FILT CONST FOR FILT ISMNMP INTO ISMNMPKA

0DEC        08      8 KMNMPKA          8 STEPS  INIT FOR ISMNMPKA WHEN KEEP ALIVE RAM FAIL

0DED        10     16 KMMPKAHI        16 STEPS  ISMMPKAC-ISMNMPKA DELTA THRESH TO ADJ ISMNMPKA

0DEE        0A     10 KMMPKALO        10 STEPS  ISMNMPKA-ISMMPKAC DELTA THRESH TO ADJ ISMNMPKA

0DEF        02      2 KPKADLT          2 STEPS  MAX ADJUST TO ISMNMPKA BECAUSE OF A/C LEARN

0DF0        08      8 KLIMIACA         8 STEPS  ISMMPKAC (ALT ADJUSTED) MUST BE THIS CLOSE TO ISMNMP FOR

NVM FAIL ISMNMPKA MODE TO BE DISABLED

0DF1        06      6 KLIMIACN         6 STEPS  ISMNMPKA (ALT ADJUSTED) MUST BE THIS CLOSE TO ISMNMP FOR

NVM FAIL  ISMNMPKA MODE TO BE DISABLED

0DF2        AF    175 KPKATIM         35 SEC    TIME IN CELL 0 BEFORE ALLOWING 'QUICK LEARN'

###########################################################

#  F15  TABLE                                             #

#  'ARTIFICIAL' PKA ADJUSTMENT FOR NVM FAIL 'QUICK LEARN' #

#                  VS                                     #

#               COOLANT                                   #

# TABLE VALUE = STEPS                                     #

###########################################################

* -------------------------------------------------------------------

0DF3        1E     30                 30 STEPS    -28  DEG C-TEMP

0DF4        18     24                 24 STEPS    -16

0DF5        16     22                 22 STEPS     -4

0DF6        14     20                 20 STEPS      8

0DF7        12     18                 18 STEPS     20

0DF8        12     18                 18 STEPS     32

0DF9        0E     14                 14 STEPS     44

0DFA        1E     30                 30 STEPS     56

0DFB        16     22                 22 STEPS     68

0DFC        0D     13                 13 STEPS     80

0DFD        07      7                  7 STEPS     92

0DFE        07      7                  7 STEPS    104

0DFF        08      8                  8 STEPS    116

0E00      0002      2 KIACRSRT         2 SEC    ENABLE IAC RESET IF ENGINE RUN TIME EXCEEDS THIS VALUE

0E02        64    100 KIACBATL        10 VOLTS  MIN BAT VOLT FOR MOVING IAC MOTOR RELIABLY

0E03        17     23 KIACRPM       0.92 RPM    RPM BELO WHICH TRY TO MOVE IAC MOT

EVEN IF THE BAT VOLT IS LOW

0E04        06      6 KISPSAN          6 STEPS  POWER STEERING ANTICIPATE CORRECTION

0E05        08      8 KISPAAN          8 STEPS  POWER STEERING ANTICIPATE STEPS AC ON

0E06        08      8 KPKALIMU         8 STEPS  MAX ALLOWED ISMNMPKA POS DELTA FROM ENG STARTUP

0E07        10     16 KPKALIMD        16 STEPS  MAX ALLOWED ISMNMPKA NEG DELTA FROM ENG STARTUP

##########################################################

#     F16B TABLE                                         #

#  WARM PARK POSITION (STEPS) VS COOLANT TEMPERATURE     #

#  TABLE VALUE = STEPS                                   #

##########################################################

* -------------------------------------------------------------------

0E08        08      8                   8        USE 9 VALUE TABLE

0E09        3C     60                 60 STEPS    -40  DEG C-TEMP

0E0A        32     50                 50 STEPS    -16

0E0B        28     40                 40 STEPS      8

0E0C        1E     30                 30 STEPS     32

0E0D        18     24                 24 STEPS     56

0E0E        0C     12                 12 STEPS     80

0E0F        04      4                  4 STEPS    104

0E10        04      4                  4 STEPS    128

0E11        04      4                  4 STEPS    152

###########################################################

#  F110 TABLE                                             #

#  MAXIMUM THROTTLE FOLLOWER STEPS MULTIPLIER             #

#                  VS                                     #

#               COOLANT                                   #

# TABLE VALUE = MULTIPLIER * 256                          #

###########################################################

* -------------------------------------------------------------------

0E12        36     54                0.21 MULT     -28  DEG C-TEMP

0E13        36     54               0.21 MULT     -16

0E14        36     54               0.21 MULT      -4

0E15        3C     60              0.234 MULT       8

0E16        3C     60              0.234 MULT      20

0E17        3C     60              0.234 MULT      32

0E18        90    144              0.563 MULT      44

0E19        90    144              0.563 MULT      56

0E1A        A0    160              0.625 MULT      68

0E1B        B0    176              0.688 MULT      80

0E1C        FC    252              0.984 MULT      92

0E1D        FC    252              0.984 MULT     104

0E1E        FC    252              0.984 MULT     116

#############################################

#  F111 TABLE                               #

#  MAXIMUM THROTTLE FOLLOWER STEPS          #

#                 VS                        #

#           VEHICLE SPEED                   #

# TABLE VALUE = STEPS                       #

#############################################

* -------------------------------------------------------------------

0E1F        12     18                18 STEPS      0    MPH-SPEED

0E20        27     39                 39 STEPS     10

0E21        35     53                 53 STEPS     20

0E22        40     64                 64 STEPS     30

0E23        42     66                 66 STEPS     40

0E24        45     69                 69 STEPS     50

0E25        50     80                 80 STEPS     60

####################################

#  F112 TABLE                      #

#     WARM PARKDOWN BIAS           #

#             VS.                  #

#            BARO                  #

####################################

* -------------------------------------------------------------------

0E26        10     16                 16 STEPS     65    KPA-BARO

0E27        0C     12                 12 STEPS     75

0E28        06      6                  6 STEPS     85

0E29        03      3                  3 STEPS     95

0E2A        00      0                  0 STEPS    105

IAC MOTOR STEPS OFFSET AT STARTUP

************************************************

*  F116 : IAC MOTOR STEPS OFFSET AT START UP   *

*          VS. TEMPERATURE                     *

*  TABLE VALUE = IAC MOTOR STEPS               *

************************************************

* -------------------------------------------------------------------

0E2B        32     50                  50 STEPS    -40  DEG C-TEMP

0E2C        32     50                 50 STEPS    -28

0E2D        2D     45                 45 STEPS    -16

0E2E        2D     45                 45 STEPS     -4

0E2F        28     40                 40 STEPS      8

0E30        25     37                 37 STEPS     20

0E31        25     37                 37 STEPS     32

0E32        24     36                 36 STEPS     44

0E33        23     35                 35 STEPS     56

0E34        23     35                 35 STEPS     68

0E35        22     34                 34 STEPS     80

0E36        21     33                 33 STEPS     92

0E37        1E     30                 30 STEPS    104

0E38        19     25                 25 STEPS    116

0E39        1E     30                 30 STEPS    128

0E3A        1E     30                 30 STEPS    140

0E3B        1E     30                 30 STEPS    152

#############################################

#  F117 TABLE                                                                #

#  MAXIMUM THROTTLE FOLLOWER STEPS            #

#         VS   RPM   TABLE VALUE = STEPS                   #

#############################################

* -------------------------------------------------------------------

0E3C        40     64                64 STEPS      0    RPM-SPEED

0E3D        40     64                 64 STEPS    400

0E3E        40     64                 64 STEPS    800

0E3F        40     64                 64 STEPS   1200

0E40        40     64                 64 STEPS   1600

0E41        60     96                 96 STEPS   2000

0E42        60     96                 96 STEPS   2400

0E43        60     96                 96 STEPS   2800

0E44        60     96                 96 STEPS   3200

0E45        60     96                 96 STEPS   3600

0E46        60     96                 96 STEPS   4000

0E47        60     96 KEGR1GML     0.375 FACT   1ST GEAR MULTIPLIER (0 TO 1) TO EGR

0E48        80    128 KEGR2GML       0.5 FACT   2ND GEAR MULTIPLIER (0 TO 1) TO EGR

0E49        30     48 KDGEGRPM      1200 RPM    RPM BELOW WHICH DIGITAL EGR  WILL BE FORCED TO ZERO

0E4A        02      2 KEGRBAND         2 CTS    HYSTERESIS COUNTS FOR DIG EGR F72 TABLE VALUE;

PROVIDES HYSTERSIS BETWEEN DIG EGR OUTPUT CHANGES

0E4B        00      0 KZEROFLT         0 COEFF  EGR POS.LEARNED ZERO VALUE FILT. COEF

0E4C        A0    160 K32AZCT         80 DEGC   IF COOLANT <= THIS BYPASS AUTO ZERO

0E4D        6C    108 KAIRFLOW      3.38 UNITS  AIR FLOW MULTIPLIER

0E4E        34     52 KEGRINIT       133 %      EGR POSITION AND ZERO POSITION INIT VALUE

0E4F        04      4 KEGRDOFS       0.4 %      OFFSET VALUE FOR EGRDESA

*#################################################################

*   F73 TABLE                                                    #

*   EGRDESA LAG FILTER COEFFICIENT VS. AIRFLOW                   #

*   TABLE VALUE = COEFFICIENT * 256                              #

*#################################################################

* -------------------------------------------------------------------

0E50        06      6               0.023 COEF       0 AIRFLO-GM/SEC

0E51        0A     10               0.04 COEF       8

0E52        12     18               0.07 COEF      16

0E53        16     22              0.086 COEF      24

0E54        1A     26                0.1 COEF      32

0E55        1E     30              0.117 COEF      40

0E56        22     34              0.133 COEF      48

0E57        26     38               0.15 COEF      56

0E58        26     38               0.15 COEF      64

*#################################################################

*   F75 TABLE                                                    #

*   EGR FLOW PRESSURE COMPENSATION VS PRESSURE                   #

*   TABLE VALUE = CONST / F(KPA)                                 #

*#################################################################

* -------------------------------------------------------------------

0E59        59     89                  89 UNITS  0   KPA

0E5A        59     89                 89 UNITS  0

0E5B        5E     94                 94 UNITS  1.8

0E5C        6E    110                110 UNITS  7.7

0E5D        7D    125                125 UNITS  13.6

0E5E        8D    141                141 UNITS  19.5

0E5F        9D    157                157 UNITS  25.4

0E60        AD    173                173 UNITS  31.3

0E61        BC    188                188 UNITS  37.2

0E62        CC    204                204 UNITS  43.1

0E63        DC    220                220 UNITS  49.0

0E64        EC    236                236 UNITS  54.9

0E65        FB    251                251 UNITS  60.8

0E66        FF    255                255 UNITS  66.7

0E67        FF    255                255 UNITS  72.6

0E68        FF    255                255 UNITS  78.5

0E69        FF    255                255 UNITS  84.4

EGR TABLE

*##################################################################

*   F72 TABLE                                                     #

*                                                                 #

*   FOR PWM EGR OPTION:       EGR DUTY CYCLE VS. VAC OR MAP & RPM #

*                             TABLE VALUE = % * 2.56              #

*   FOR DIGITAL EGR OPTION: SOLENOID COMBINATION VS. VAC/MAP& RPM #

*                           TABLE VALUE = (SOL COMB) * 32         #

*##################################################################

* -------------------------------------------------------------------

0E6A        20     32                 32        R MIN         800 RPM

0E6B        00      0                  0        Q MIN KPA VAC

0E6C        0C     12                 12

* 800 RPM

* --------------------------------------------- SPEED        800 RPM

0E6D        00      0                  0 %      0    40 KPA VA-KPA MP

0E6E        00      0                  0 %      2.5  45

0E6F        00      0                  0 %      5.0  50

0E70        00      0                  0 %      7.5  55

0E71        00      0                  0 %      10.0 60

0E72        00      0                  0 %      12.5 65

0E73        00      0                  0 %      15.0 70

0E74        00      0                  0 %      17.5 75

0E75        00      0                  0 %      20.0 80

0E76        00      0                  0 %      30.0 85

0E77        00      0                  0 %      40.0 90

0E78        00      0                  0 %      50.0 95

* --------------------------------------------- SPEED       1200 RPM

0E79        00      0                  0 %      0    40 KPA VA-KPA MP

0E7A        00      0                  0 %      2.5  45

0E7B        00      0                  0 %      5.0  50

0E7C        10     16                0.5 %      7.5  55

0E7D        20     32                  1 %      10.0 60

0E7E        30     48                1.5 %      12.5 65

0E7F        40     64                  2 %      15.0 70

0E80        50     80                2.5 %      17.5 75

0E81        50     80                2.5 %      20.0 80

0E82        40     64                  2 %      30.0 85

0E83        40     64                  2 %      40.0 90

0E84        40     64                  2 %      50.0 95

* --------------------------------------------- SPEED       1600 RPM

0E85        00      0                  0 %      0    40 KPA VA-KPA MP

0E86        00      0                  0 %      2.5  45

0E87        10     16                0.5 %      5.0  50

0E88        20     32                  1 %      7.5  55

0E89        30     48                1.5 %      10.0 60

0E8A        40     64                  2 %      12.5 65

0E8B        50     80                2.5 %      15.0 70

0E8C        50     80                2.5 %      17.5 75

0E8D        50     80                2.5 %      20.0 80

0E8E        50     80                2.5 %      30.0 85

0E8F        40     64                  2 %      40.0 90

0E90        40     64                  2 %      50.0 95

* --------------------------------------------- SPEED       2000 RPM

0E91        00      0                  0 %      0    40 KPA VA-KPA MP

0E92        00      0                  0 %      2.5  45

0E93        10     16                0.5 %      5.0  50

0E94        30     48                1.5 %      7.5  55

0E95        50     80                2.5 %      10.0 60

0E96        60     96                  3 %      12.5 65

0E97        70    112                3.5 %      15.0 70

0E98        70    112                3.5 %      17.5 75

0E99        70    112                3.5 %      20.0 80

0E9A        50     80                2.5 %      30.0 85

0E9B        40     64                  2 %      40.0 90

0E9C        40     64                  2 %      50.0 95

* --------------------------------------------- SPEED       2400 RPM

0E9D        00      0                  0 %      0    40 KPA VA-KPA MP

0E9E        00      0                  0 %      2.5  45

0E9F        30     48                1.5 %      5.0  50

0EA0        40     64                  2 %      7.5  55

0EA1        60     96                  3 %      10.0 60

0EA2        70    112                3.5 %      12.5 65

0EA3        70    112                3.5 %      15.0 70

0EA4        70    112                3.5 %      17.5 75

0EA5        60     96                  3 %      20.0 80

0EA6        50     80                2.5 %      30.0 85

0EA7        40     64                  2 %      40.0 90

0EA8        40     64                  2 %      50.0 95

* --------------------------------------------- SPEED       2800 RPM

0EA9        10     16                0.5 %      0    40 KPA VA-KPA MP

0EAA        30     48                1.5 %      2.5  45

0EAB        30     48                1.5 %      5.0  50

0EAC        40     64                  2 %      7.5  55

0EAD        60     96                  3 %      10.0 60

0EAE        70    112                3.5 %      12.5 65

0EAF        70    112                3.5 %      15.0 70

0EB0        70    112                3.5 %      17.5 75

0EB1        70    112                3.5 %      20.0 80

0EB2        60     96                  3 %      30.0 85

0EB3        40     64                  2 %      40.0 90

0EB4        40     64                  2 %      50.0 95

* --------------------------------------------- SPEED       3200 RPM

0EB5        10     16                0.5 %      0    40 KPA VA-KPA MP

0EB6        20     32                  1 %      2.5  45

0EB7        30     48                1.5 %      5.0  50

0EB8        50     80                2.5 %      7.5  55

0EB9        60     96                  3 %      10.0 60

0EBA        60     96                  3 %      12.5 65

0EBB        60     96                  3 %      15.0 70

0EBC        60     96                  3 %      17.5 75

0EBD        60     96                  3 %      20.0 80

0EBE        50     80                2.5 %      30.0 85

0EBF        40     64                  2 %      40.0 90

0EC0        40     64                  2 %      50.0 95

*#################################################################

*   F74DIG TABLE                                                 #

*   EGR FLOW VS SOLENOID POSITION FOR DIGITAL EGR                #

*   TABLE VALUE = (GRAMS/SEC)*8    (NO INTERPOLATION)            #

*#################################################################

* -------------------------------------------------------------------

0EC1        00      0                 0 GMS/S  0

0EC2        07      7                0.9 GMS/S  1

0EC3        0E     14                1.8 GMS/S  2

0EC4        16     22                2.8 GMS/S  3

0EC5        19     25                3.1 GMS/S  4

0EC6        21     33                4.1 GMS/S  5

0EC7        27     39                4.9 GMS/S  6

0EC8        2F     47                5.9 GMS/S  7

##############################################

# F76  TABLE                                 #

# EGR DUTY CYCLE MULTIPLIER (0-2) VS COOLANT #

# TABLE VALUE = MULTIPLIER*128               #

##############################################

* -------------------------------------------------------------------

0EC9        33     51                0.4 GAIN      32  DEG C-TEMP

0ECA        5A     90                0.7 GAIN      44

0ECB        73    115                0.9 GAIN      56

0ECC        80    128                  1 GAIN      68

0ECD        80    128                  1 GAIN      80

0ECE        80    128                  1 GAIN      92

##############################################

# F77  TABLE                                 #

# EGR DUTY CYCLE MULTIPLIER (0-2) VS EGRDC   #

# TABLE VALUE = MULTIPLIER*128               #

##############################################

* -------------------------------------------------------------------

0ECF        00      0 F77A             0 MULT       0      %-EGRDC

0ED0        00      0                  0 MULT    6.25

0ED1        00      0                  0 MULT   12.50

0ED2        00      0                  0 MULT   18.75

0ED3        00      0                  0 MULT   25.00

0ED4        00      0                  0 MULT   31.25

0ED5        00      0                  0 MULT   37.50

0ED6        00      0                  0 MULT   43.75

0ED7        00      0                  0 MULT   50.00

0ED8        00      0                  0 MULT   56.25

0ED9        00      0                   0 MULT   62.50

0EDA        00      0                  0 MULT   68.75

0EDB        00      0                  0 MULT   75.00

0EDC        00      0                  0 MULT   81.25

0EDD        00      0                  0 MULT   87.50

0EDE        00      0                  0 MULT   93.75

0EDF        00      0                  0 MULT   100.0

##############################################

# F79  TABLE                                 #

# EXHAUST SYSTEM BACKPRESSURE VS AIRFLOW1    #

# TABLE VALUE = (KPA*2.71)-28.06             #

##############################################

* -------------------------------------------------------------------

0EE0        00      0               10.3 KPA        0   FLOW-GM/SEC

0EE1        03      3               11.5 KPA        8

0EE2        06      6               12.6 KPA       16

0EE3        0B     11               14.4 KPA       24

0EE4        11     17               16.6 KPA       32

0EE5        13     19               17.4 KPA       40

0EE6        14     20               17.7 KPA       48

0EE7        16     22               18.5 KPA       56

0EE8        11     17               16.6 KPA       64

0EE9        10     16               16.3 KPA       72

0EEA        0F     15                 16 KPA       80

0EEB        0F     15                 16 KPA       88

0EEC        0E     14               15.5 KPA       96

0EED        0E     14               15.5 KPA      104

0EEE        0E     14               15.5 KPA      112

0EEF        0E     14               15.5 KPA      120

0EF0        0E     14               15.5 KPA      128

##############################################

# F80  TABLE   USED TO COMP VE FOR EGR       #

# MULTIPLIER (0-2) VS MAP WHEN TCC ENGAGED   #

# TABLE VALUE = MULTIPLIER*128               #

##############################################

* -------------------------------------------------------------------

0EF1        80    128                  1 MULT      40    KPA-MAP

0EF2        80    128                  1 MULT      45

0EF3        80    128                  1 MULT      50

0EF4        80    128                  1 MULT      55

0EF5        80    128                  1 MULT      60

0EF6        80    128                  1 MULT      65

0EF7        80    128                  1 MULT      70

0EF8        80    128                  1 MULT      75

0EF9        80    128                  1 MULT      80

0EFA        80    128                  1 MULT      85

0EFB        80    128                  1 MULT      90

0EFC        80    128                  1 MULT      95

0EFD        00      0 KEGRPSFT         0 MULT   LAG FILTER CONSTANT FOR RAW A/D FILTERING

0EFE        00      0 KEGRLIMA         0 MULT   MULTIPLIER (0-1) USED TO COMP VE FOR EGR

EGR COOLANT

0EFF        5D     93 KEGRTENA        30 DEGC   IF COOLANT <= THIS, DISABLE EGR

0F00        E0    224 KEGRDCDL      87.5 %      MINIMUM EGR DUTY CYCLE DELTA

0F01        20     32 KEGRDCDH      12.5 %      MAXIMUM EGR DUTY CYCLE DELTA

0F02        38     56 KEGRMIN         22 %      MINIMUM EGR DUTY CYCLE

0F03        08      8 KEGRON           3 %      EGR DUTY CYCLE FOR EGR VALVE TO BE OPEN

0F04        04      4 KEGRDLT        1.6 %      LEARNED POSITIVE DELTA FOR EGR DUTY CYCLE

0F05        40     64 KEGRNDC         25 %      LEARNED NEGATIVE DELTA FOR EGR DUTY CYCLE

0F06        06      6 KEGRTIME       0.6 SEC    LEARN ENABLE TIME CONSTANT

0F07        40     64 KEGRMAPL        34 KPA    IF MAP <= THIS, DISABLE EGR

0F08        04      4 KF4TPS2        1.6 %      F4 TBL CLOSED THROT THRESH, LOWER HYST PAIR

0F09        FF    255 KEGRVAC1         0 KPA    VAC THRESHOLD, EGR ON

0F0A        4A     74 KEGRMAPH      37.6 KPA    IF MAP <= THIS, DISABLE EGR

0F0B        06      6 KF4TPS1        2.3 %      F4 TBL CLOSED THROT THRESH, UPPER HYST PAIR

0F0C        E8    232 KEGRVAC2       7.5 KPA    VAC THRESHOLD, EGR OFF

0F0D        B0    176 KFILTVE      0.688 COEF   FILTER COEFICIENT APPLIED TO VEEGRFAC

**************************

* F9MSGT TABLE           *

* MESSAGE SCHEDULE TABLE *

* TABLE VALUE = ADDRESS  *

**************************

* -------------------------------------------------------------------

0F0E      8F2E  36654            $8F2E ADDRS  TIME SLOT 0

0F10      0000      0                  0 ADDRS  !         1

0F12      8F41  36673              $8F41 ADDRS  !         2

0F14      0000      0                  0 ADDRS  !         3

0F16      8F2E  36654              $8F2E ADDRS  !         4

0F18      0000      0                  0 ADDRS  !         5

0F1A      8F62  36706              $8F62 ADDRS  !         6

0F1C      0000      0                  0 ADDRS  !         7

0F1E      8F2E  36654              $8F2E ADDRS  !         8

0F20      0000      0                  0 ADDRS  !         9

0F22      0000      0                  0 ADDRS  !         A

0F24      0000      0                  0 ADDRS  !         B

0F26      8F2E  36654              $8F2E ADDRS  !         C

0F28      0000      0                  0 ADDRS  !         D

0F2A      0000      0                  0 ADDRS  !         E

0F2C      0000      0                  0 ADDRS  !         F

***************************************************************

* ALDL TRANSMIT TABLES FOR MODES ZERO (NORMAL MODE) AND SEVEN *

*                                                             *

***************************************************************

* -------------------------------------------------------------------

0F2E      8F41  36673 F9MSG1       $8F41 ADDRS  NEXT MESSAGE ENTRY ADDRESS

0F30        0A     10                $0A        MESSAGE ID

0F31        80    128                $80 FLAG   USE ROM TABLE

0F32        03      3                $03 BYTES  NUMBER OF OUTPUT DATA BYTES

0F33      0119    281              $0119 ADDRS  ADDRESS OF OUTPUT MESSAGE BUFFER

0F35      0000      0                  0 ADDRS  ADDRESS OF INPUT MESSAGE BUFFER

0F37      0033     51              $0033 ADDRS  FLAGWORD (SEE RAMEQU MEMBER FOR DESCRIPTION)

0F39      FFFF  65535              $FFFF        N = RPM/256

0F3B      018F    399              $018F        N = RPM

0F3D      0000      0                  0

0F3F      0000      0                  0

* -------------------------------------------------------------------

0F41      8F62  36706 F9MSG2       $8F62 ADDRS  NEXT MESSAGE ENTRY ADDRESS

0F43        05      5                $05        MESSAGE ID

0F44        80    128                $80 FLAG   USE ROM TABLE

0F45        0A     10                $0A BYTES  NUMBER OF OUTPUT DATA BYTES

0F46      0119    281              $0119 ADDRS  ADDRESS OF OUTPUT MESSAGE BUFFER

0F48      0000      0                  0 ADDRS  ADDRESS OF INPUT MESSAGE BUFFER

0F4A      0033     51              $0033 ADDRS  FLAGWORD (SEE RAMEQU MEMBER FOR DESCRIPTION)

0F4C      0032     50              $0032        FLAGWORD (SEE RAMEQU MEMBER FOR DESCRIPTION)

0F4E      0069    105              $0069        N = MPH

0F50      800E  32782              $800E        N = RPM/100

0F52      006C    108              $006C        N = VOLTS * 10

0F54      FFFF  65535              $FFFF        N=MSEC*65.536/256

0F56      0022     34              $0022        N=MSEC*65.536

0F58      86C6  34502              $86C6        N=(GAL/HR)*32*KNUMCYL /256

0F5A      86C7  34503              $86C7        N=(GAL/HR)*32*KNUMCYL

0F5C      00E8    232              $00E8        N=(DEGC+40)*256/192

0F5E      0000      0                  0

0F60      0000      0                  0

######################################################

* F9MSG3                                             #

* MESSAGE TO BE TRANSMITTED IN BROADCAST REMOTE MODE #

* TABLE VALUE = ADDRESS                              #

######################################################

* -------------------------------------------------------------------

0F62      0000      0 F9MSG3           0 ADDRS  NEXT MESSAGE ENTRY ADDRESS

0F64        F0    240                $F0        MESSAGE ID

0F65        80    128                $80 FLAG   USE ROM TABLE

0F66        01      1                  1 BYTES  NUMBER OF OUTPUT DATA BYTES

0F67      0119    281              $0119 ADDRS  ADDRESS OF OUTPUT MESSAGE BUFFER

0F69      0000      0                  0 ADDRS  ADDRESS OF INPUT MESSAGE BUFFER

0F6B      8F6D  36717              $8F6D ADDRS  1

0F6D        F4    244 KALDLID        $F4 MSG ID ECM ALDL MESSAGE ID

***********************************

*     SERIAL DATA CALIBRATION     *

***********************************

0F6E      FFFF  65535 KMD4TIM     819.19 SEC    ALLOW MODE 4 FOR THIS LONG

0F70      0000      0 KMD4LCK          0 SEC    PREVENT MODE 4 FOR THIS LONG

********************************

*     SERIAL DATA MESSAGES     *

********************************

********************************************************

* SDRF4TBL                                             *

* MESSAGE RECEIVED IN RESPONSE TO ALDL POLLING MESSAGE *

* TABLE VALUE = ADDRESS                                *

********************************************************

0F72      0000      0                  0        NEXT MESSAGE ENTRY ADDRESS

0F74        F4    244                $F4        MESSAGE ID

0F75        00      0                  0 FLAG   USE NO BUFFER

0F76        01      1                $01 BYTES  NUMBER OF OUTPUT DATA BYTES

0F77      0119    281              $0119 ADDRS  ADDRESS OF OUTPUT MESSAGE BUFFER

0F79      0119    281              $0119 ADDRS  ADDRESS OF INPUT MESSAGE BUFFER

0F7B      8F91  36753              $8F91        SERIAL DATA RECEIVE MSG, MODE 0

0F7D      8F9A  36762              $8F9A        SERIAL DATA RECEIVE MSG, MODE 1

0F7F      9087  36999              $9087        SERIAL DATA RECEIVE MSG, MODE 2

0F81      9090  37008              $9090        SERIAL DATA RECEIVE MSG, MODE 3

0F83      9099  37017              $9099        SERIAL DATA RECEIVE MSG, MODE 4

0F85      90A2  37026              $90A2        SERIAL DATA RECEIVE MSG, MODE 5

0F87      90AD  37037              $90AD        SERIAL DATA RECEIVE MSG, MODE 6

0F89      90B6  37046              $90B6        SERIAL DATA RECEIVE MSG, MODE 7

0F8B      90BF  37055              $90BF        SERIAL DATA RECEIVE MSG, MODE 8

0F8D      90C8  37064              $90C8        SERIAL DATA RECEIVE MSG, MODE 9

0F8F      90D1  37073              $90D1        SERIAL DATA RECEIVE MSG, MODE 10

*********************************************************

* SDRMF4M0                                              *

* SERIAL DATA RECEIVE MESSAGE, MESSAGE ID = $F4, MODE 0 *

* TABLE VALUE = ADDRESS                                 *

*********************************************************

0F91      0000      0 SDRMF4M0         0        NEXT MESSAGE ENTRY ADDRESS

0F93        F4    244                $F4        MESSAGE ID

0F94        00      0                  0 FLAG   USE NO BUFFER

0F95        01      1                $01 BYTES  NUMBER OF OUTPUT DATA BYTES

0F96      0119    281              $0119 ADDRS  ADDRESS OF OUTPUT MESSAGE BUFFER

0F98      0119    281              $0119 ADDRS  ADDRESS OF INPUT MESSAGE BUFFER

*********************************************************

* SDRMF4M1                                              *

* SERIAL DATA RECEIVE MESSAGE, MESSAGE ID = $F4, MODE 1 *

* TABLE VALUE = ADDRESS                                 *

0F9A      9000  36864 SDRMF4M1     $9000

* NOTE - THE 'MODE1TB0' TABLE AND TABLES SDRMF4M2 THROUGH SDRMF4MA

*        ARE LOCATED IN THE MEMBER DATASD2.

*************************************************

**  RPM DERIVATIVE SPARK CONTROL CALIBRATIONS  **

*************************************************

0F9C        2B     43 KRDSRPML      1075 RPM    BYPASS R.D.S.C. IF NTRPMX < THIS

0F9D        C8    200 KRDSRPMH      5000 RPM    BYPASS R.D.S.C. IF NTRPMX >= THIS

0F9E        C8    200 KRDSNVHI       200 RATIO  BYPASS R.D.S.C. IF NVRATIO > THIS

0F9F        A5    165 KRDSNV1        165 RATIO  USE 1ST GEAR PARAMETERS IF NVRATIO > THIS

0FA0        5F     95 KRDSNV2         95 RATIO  USE 2ND GEAR PARAMETERS IF NVRATIO > THIS

0FA1        01      1 KRDSNV3          1 RATIO  USE 3RD GEAR PARAMETERS IF NVRATIO > THIS

0FA2        08      8 KRDSTPSL         3 % TPS  BYPASS R.D.S.C. IF TPS < THIS AND LAUNCH MODE

0FA3        09      9 KRDSTPSH       3.5 % TPS  BYPASS R.D.S.C. IF TPS >= KRDSTPSL AND < THIS

0FA4        04      4 KRDSDTPS       1.6 % TPS  BYPASS RDSC IF PART THROTTLE AND D-TPS < THIS

0FA5        FF    255 KRDSTIMR      3.19 SEC    ALLOW PART THROTTLE R.D.S.C. FOR THIS LONG

###########################

##  1ST GEAR PARAMETERS  ##

###########################

0FA6        05      5 KDSONS1          5 MULT   DRPM MULTIPLIER, 1ST GEAR, TPS > KRDSTPSH

0FA7        25     37 KDSONC1         13 DEG    MAX RDSC SPARK: 1ST GEAR, TPS > KRDSTP, DRPM +

0FA8        15     21 KDSOPC1        7.4 DEG    MAX RDSC SPARK: 1ST GEAR, TPS > KRDSTP, DRPM -

0FA9        10     16 KDSCNS1         16 MULT   DRPM MULTIPLIER: 1ST GEAR, TPS < KRDSTPSL, NO A/C

0FAA        25     37 KDSCNC1         13 DEG    MAX RDSC SPK: 1ST, TPS < KRDSTPSL, NO A/C, DRPM +

0FAB        10     16 KDSCPC1        5.6 DEG    MAX RDSC SPK: 1ST, TPS < KRDSTPSL, NO A/C, DRPM -

0FAC        10     16 KDSCNSA1        16 MULT   DRPM MULTIPLIER: 1ST GEAR, TPS < KRDSTPSL, A/C

0FAD        25     37 KDSCNCA1        13 DEG    MAX RDSC SPK: 1ST, TPS < KRDSTPSL, A/C, DRPM +

0FAE        10     16 KDSCPCA1       5.6 DEG    MAX RDSC SPK: 1ST, TPS < KRDSTPSL, A/C, DRPM -

###########################

##  2ND GEAR PARAMETERS  ##

###########################

0FAF        07      7 KDSONS2          7 MULT   DRPM MULTIPLIER: 2ND GEAR, TPS > KRDSTPSH

0FB0        15     21 KDSONC2        7.4 DEG    MAX RDSC SPK: 2ND GEAR, TPS > KRDSTP, DRPM +

0FB1        10     16 KDSOPC2        5.6 DEG    MAX RDSC SPK: 2ND GEAR, TPS > KRDSTP, DRPM -

0FB2        10     16 KDSCNS2         16 MULT   DRPM MULTIPLIER: 2ND GEAR, TPS < KRDSTPSL, NO A/C

0FB3        15     21 KDSCNC2        7.4 DEG    MAX RDSC SPK: 2ND, TPS < KRDSTPSL, NO A/C, DRPM +

0FB4        20     32 KDSCPC2       11.3 DEG    MAX RDSC SPK: 2ND, TPS < KRDSTPSL, NO A/C, DRPM -

0FB5        10     16 KDSCNSA2        16 MULT   DRPM MULTIPLIER: 2ND GEAR, TPS < KRDSTPSL, A/C

0FB6        15     21 KDSCNCA2       7.4 DEG    MAX RDSC SPK: 2ND, TPS < KRDSTPSL, A/C, DRPM +

0FB7        25     37 KDSCPCA2        13 DEG    MAX RDSC SPK: 2ND, TPS < KRDSTPSL, A/C, DRPM -

###########################

##  3RD GEAR PARAMETERS  ##

###########################

0FB8        00      0 KDSONS3          0 MULT   DRPM MULTIPLIER: 3RD GEAR, TPS > KRDSTPSH

0FB9        00      0 KDSONC3          0 DEG    MAX RDSC SPK: 3RD GEAR, TPS > KRDSTPSH, DRPM +

0FBA        00      0 KDSOPC3          0 DEG    MAX RDSC SPK: 3RD GEAR, TPS > KRDSTPSH, DRPM -

0FBB        00      0 KDSCNS3          0 MULT   DRPM MULTIPLIER: 3RD GEAR, TPS < KRDSTPSL, NO A/C

0FBC        00      0 KDSCNC3          0 DEG    MAX RDSC SPK: 3RD, TPS < KRDSTPSL, NO A/C, DRPM +

0FBD        00      0 KDSCPC3          0 DEG    MAX RDSC SPK: 3RD, TPS < KRDSTPSL, NO A/C, DRPM -

0FBE        00      0 KDSCNSA3         0 MULT   DRPM MULTIPLIER: 3RD GEAR, TPS < KRDSTPSL, A/C

0FBF        00      0 KDSCNCA3         0 DEG    MAX RDSC SPK: 3RD, TPS < KRDSTPSL, A/C, DRPM +

0FC0        00      0 KDSCPCA3         0 DEG    MAX RDSC SPK: 3RD, TPS < KRDSTPSL, A/C, DRPM -

0FC1        06      6 KRDSMIN          2 DEG    MINIMUM RDSC SPARK CHANGE ALLOWED

0FF8        30     48 KCUSTID1        48        CUSTOMER ID BYTE 1

0FF9        9F    159 KCUSTID2       159        CUSTOMER ID BYTE 2

0FFA        16     22 KCUSTID3        22        CUSTOMER ID BYTE 3

0FFB        00      0 KCUSTID4         0        CUSTOMER ID BYTE 4

0FFC        60     96 KCUSTID5        96        CUSTOMER ID BYTE 5

0FFD      0016     22 KHUDRAM         22 ADDR   ADDRESS CONTENTS DISPLAYED AT "RAM CONTENTS"

LOCATION ON THE 'B' SIDE IF

THE DUAL RAM

HUD DISPLAY OPTION SELECTED

0FFF        FF    255 KHUDLEDS       255 ADDR   ADDRESS CONTENTS TO BE DISPLAYED ON HUD LEDS

($FF IS DEFAULT)

**************************************************************

* MODE1TB0   MODE 1 TABLE #0                                 *

* SERIAL DATA RECEIVE MESSAGE, MESSAGE ID = $F4, MODE 1      *

* TABLE VALUE = ADDRESS                                      *

*                                                            *

* TO MAKE SURE DOUBLE BYTE VARIABLES HAVE MATCHING UPPER     *

* AND LOWER BYTES, SET FIRST BYTE TO $FFFF AND SECOND BYTE   *

* TO VARIABLE ADDRESS.                                       *

*                                                            *

**************************************************************

Serial Data MODE 1 CAL TBL

* -------------------------------------------------------------------

1000      0000      0 MODE1TB0         0             NEXT MESSAGE ENTRY ADDRESS

1002        F4    244                $F4                      MESSAGE ID

1003        80    128                $80 FLAG            USE PROM TABLE

1004        40     64                 64 BYTES            NUMBER OF OUTPUT BYTES(1ST IS MODE #)

1005      0119    281              $0119 ADDRS         ADDRESS OF OUTPUT MESSAGE BUFFER

1007      0119    281              $0119 ADDRS         ADDRESS OF INPUT MESSAGE BUFFER

1st word of data

cal add   pointer                         address pointer      #      conversion / desc

1009      8000  32768              $8000 ADDRS       LOCATION 1   PROM ID

100B      8001  32769              $8001 ADDRS  !        2

100D      0005      5              $0005 ADDRS  !       3     MALF FLAG WORD 1

100F      0006      6              $0006 ADDRS  !        4     MALF FLAG WORD 2

1011      0007      7              $0007 ADDRS  !        5     MALF FLAG WORD 3

1013      0008      8              $0008 ADDRS  !        6     MALF FLAG WORD 4

1015      00E8    232              $00E8 ADDRS  !      7      N = (DEGC+40)*(256/192)

1017      00A4    164              $00A4 ADDRS  !      8      N = (DEGC+40)*(256/192)

1019      006D    109              $006D ADDRS  !      9      N = A/D CONVERSION

101B      006E    110              $006E ADDRS  !      10      N = %TPS * 2.56

101D      0062     98              $0062 ADDRS  !        11  N = RPM/ 25

101F      FFFF  65535              $FFFF ADDRS  !    12  N = REFPER*KNUMCYL/256

1021      0050     80              $0050 ADDRS  !         13  (WHERE REFPER = SEC * 65536)

1023      FFFF  65535              $FFFF ADDRS  !     14  N=65536* 120/(RPM*KNUMCYL)

1025      00F0    240              $00F0 ADDRS  !        15

1027      0065    101              $0065 ADDRS  !        16  N = SEC * 80

1029      0069    105              $0069 ADDRS  !        17  N = MPH

102B      004D     77              $004D ADDRS  !        18  N = PID MODE WORD 5

102D      0063     99              $0063 ADDRS  !         19  N = VOLTS * 226

102F      0103    259              $0103 ADDRS  !         20  N = COUNTS

1031      008E    142              $008E ADDRS  !         21  N = MSEC * 65.536

1033      00CE    206              $00CE ADDRS  !        22  N = MULT * 128

1035      00CA    202              $00CA ADDRS  !         23  N = CELL

1037      0087    135              $0087 ADDRS  !           24  N = UNITS

1039      0017     23              $0017 ADDRS  !           25  N = IAC MOTOR STEPS

103B      00C8    200              $00C8 ADDRS  !         26  N = STEPS (2'S COMPLEMENT)

103D      00C9    201              $00C9 ADDRS  !        27  N = RPM/ 12.5

103F      001E     30              $001E ADDRS  !          28  N = 2.71 *KPA - 28.06

1041      0054     84              $0054 ADDRS  !           29  N = 2.71 *KPA - 28.06

1043      004F     79              $004F ADDRS  !           30  N = A/D CONVERSION

1045      00E5    229              $00E5 ADDRS  !          31  N = VOLTS * 10

1047      00FB    251              $00FB ADDRS  !         32  N = DIG. EGR SOLONOID COMBO

1049      0148    328              $0148 ADDRS  !          33  N = A/C PRESS TRANS A/D CNTS

104B      006C    108              $006C ADDRS  !        34  N = VOLTS * 10

104D      0019     25              $0019 ADDRS  !          35  N = IAC MIN MOTOR POSITION

104F      0185    389              $0185 ADDRS  !          36  N = BLM CELL 0 TIMER FOR SAM

1051      00AD    173              $00AD ADDRS  !         37  N = GRAMS PER SECOND

1053      FFFF  65535              $FFFF ADDRS  !       38  N = SPARK ADV * 256/90

1055      00B9    185              $00B9 ADDRS  !          39

1057      00B6    182              $00B6 ADDRS  !          40  N = SPARK ADV * 256/90

1059      0088    136              $0088 ADDRS  !           41  N = A/F RATIO * 10

105B      FFFF  65535              $FFFF ADDRS  !       42  N = SECONDS ON TIME * 65536

105D      01AE    430              $01AE ADDRS  !         43

105F      01D1    465              $01D1 ADDRS  !          44  N = VOLTS

1061      00A8    168              $00A8 ADDRS  !          45  N = % CCP * 2.56

1063      00AE    174              $00AE ADDRS  !          46  N = SPK RETARD * 256/45

1065      00E1    225              $00E1 ADDRS  !          47  N = RPM / MPH

1067      FFFF  65535              $FFFF ADDRS  !        48  N = SECONDS ENG RUN TIME

1069      0015     21              $0015 ADDRS  !            49

106B      0181    385              $0181 ADDRS  !          50  N = (DEGC-300)/3

106D      00AF    175              $00AF ADDRS  !        51  N = KNOCK COUNTS

106F      00C0    192              $00C0 ADDRS  !         52  SC1 OUTPUTS

1071      00BF    191              $00BF ADDRS  !         53  SC1 INPUTS

1073      002B     43              $002B ADDRS  !          54  MODE WORD 1

1075      002C     44              $002C ADDRS  !          55  MODE WORD 2

1077      002D     45              $002D ADDRS  !          56  MODE WORD 3

1079      002E     46              $002E ADDRS  !          57  MODE WORD 4

107B      0036     54              $0036 ADDRS  !          58  LCC AND CCP MODE WORD

107D      0027     39              $0027 ADDRS  !          59  FMD BYTE 1 MODE WORD

107F      0001      1              $0001 ADDRS  !           60  NON- VOLITILE MODE WORD

1081      0038     56              $0038 ADDRS  !          61  ALCL MODE WORD

1083      0039     57              $0039 ADDRS  !          62  CLCC MODE WORD

1085      0043     67              $0043 ADDRS  !          63  AIR/FUEL MODE WORD 1

*********************************************************

* SDRMF4M2                                              *

* SERIAL DATA RECEIVE MESSAGE, MESSAGE ID = $F4, MODE 2 *

* TABLE VALUE = ADDRESS                                 *

*********************************************************

1087      0000      0 SDRMF4M2         0        NEXT MESSAGE ENTRY ADDRESS

1089        F4    244                $F4        MESSAGE ID

108A        40     64                $40 FLAG   USE RAM BUFFER

108B        41     65                 65 BYTES  NUMBER OF OUTPUT DATA BYTES

108C      0119    281              $0119 ADDRS  ADDRESS OF OUTPUT MESSAGE BUFFER

108E      0119    281              $0119 ADDRS  ADDRESS OF INPUT MESSAGE BUFFER

*********************************************************

* SDRMF4M3                                              *

* SERIAL DATA RECEIVE MESSAGE, MESSAGE ID = $F4, MODE 3 *

* TABLE VALUE = ADDRESS                                 *

*********************************************************

1090      0000      0 SDRMF4M3         0        NEXT MESSAGE ENTRY ADDRESS

1092        F4    244                $F4        MESSAGE ID

1093        40     64                $40 FLAG   USE RAM BUFFER

1094        09      9                  9 BYTES  NUMBER OF OUTPUT DATA BYTES

1095      0119    281              $0119 ADDRS  ADDRESS OF OUTPUT MESSAGE BUFFER

1097      0119    281              $0119 ADDRS  ADDRESS OF INPUT MESSAGE BUFFER

*********************************************************

* SDRMF4M4                                              *

* SERIAL DATA RECEIVE MESSAGE, MESSAGE ID = $F4, MODE 4 *

* TABLE VALUE = ADDRESS                                 *

*********************************************************

1099      0000      0 SDRMF4M4         0        NEXT MESSAGE ENTRY ADDRESS

109B        F4    244                $F4        MESSAGE ID

109C        00      0                  0 FLAG   USE NO BUFFER

109D        01      1                $01 BYTES  NUMBER OF OUTPUT DATA BYTES

109E      0119    281              $0119 ADDRS  ADDRESS OF OUTPUT MESSAGE BUFFER

10A0      0119    281              $0119 ADDRS  ADDRESS OF INPUT MESSAGE BUFFER

*********************************************************

* SDRMF4M5                                              *

* SERIAL DATA RECEIVE MESSAGE, MESSAGE ID = $F4, MODE 5 *

* TABLE VALUE = ADDRESS                                 *

10A2      0000      0 SDRMF4M5         0        NEXT MESSAGE ENTRY ADDRESS

10A4        F4    244                $F4        MESSAGE ID

10A5        80    128                $80 FLAG   USE PROM TABLE

10A6        02      2                  2 BYTES  NUMBER OF OUTPUT BYTES(1ST IS MODE #)

10A7      0119    281              $0119 ADDRS  ADDRESS OF OUTPUT MESSAGE BUFFER

10A9      0119    281              $0119 ADDRS  ADDRESS OF INPUT MESSAGE BUFFER

10AB      0112    274              $0112        MODE 5 RESPONSE BYTE

*********************************************************

* SDRMF4M6                                              *

* SERIAL DATA RECEIVE MESSAGE, MESSAGE ID = $F4, MODE 6 *

* TABLE VALUE = ADDRESS                                 *

*********************************************************

10AD      0000      0 SDRMF4M6         0        NEXT MESSAGE ENTRY ADDRESS

10AF        F4    244                $F4        MESSAGE ID

10B0        20     32                $20 FLAG   USE NO BUFFER

10B1        00      0                $00 BYTES  NUMBER OF OUTPUT DATA BYTES

10B2      0119    281              $0119 ADDRS  ADDRESS OF OUTPUT MESSAGE BUFFER

10B4      0119    281              $0119 ADDRS  ADDRESS OF INPUT MESSAGE BUFFER

*********************************************************

* SDRMF4M7                                              *

* SERIAL DATA RECEIVE MESSAGE, MESSAGE ID = $F4, MODE 7 *

* TABLE VALUE = ADDRESS                                 *

*********************************************************

10B6      0000      0 SDRMF4M7         0        NEXT MESSAGE ENTRY ADDRESS

10B8        F4    244                $F4        MESSAGE ID

10B9        20     32                $20 FLAG   USE NO BUFFER

10BA        00      0                $00 BYTES  NUMBER OF OUTPUT DATA BYTES

10BB      0119    281              $0119 ADDRS  ADDRESS OF OUTPUT MESSAGE BUFFER

10BD      0119    281              $0119 ADDRS  ADDRESS OF INPUT MESSAGE BUFFER

****************************************************************

* SDRMF4M8                                                     *

* SERIAL DATA RECEIVE MESSAGE, MESSAGE ID = $F4, MODE 8        *

* TABLE VALUE = ADDRESS                                        *

****************************************************************

10BF      0000      0 SDRMF4M8         0        NEXT MESSAGE ENTRY ADDRESS

10C1        F4    244                $F4        MESSAGE ID

10C2        00      0                  0 FLAG   USE NO BUFFER

10C3        01      1                $01 BYTES  NUMBER OF OUTPUT DATA BYTES

10C4      0119    281              $0119 ADDRS  ADDRESS OF OUTPUT MESSAGE BUFFER

10C6      0119    281              $0119 ADDRS  ADDRESS OF INPUT MESSAGE BUFFER

****************************************************************

* SDRMF4M9                                                     *

* SERIAL DATA RECEIVE MESSAGE, MESSAGE ID = $F4, MODE 9        *

* TABLE VALUE = ADDRESS                                        *

****************************************************************

10C8      0000      0 SDRMF4M9         0        NEXT MESSAGE ENTRY ADDRESS

10CA        F4    244                $F4        MESSAGE ID

10CB        00      0                  0 FLAG   USE NO BUFFER

10CC        01      1                $01 BYTES  NUMBER OF OUTPUT DATA BYTES

10CD      0119    281              $0119 ADDRS  ADDRESS OF OUTPUT MESSAGE BUFFER

10CF      0119    281              $0119 ADDRS  ADDRESS OF INPUT MESSAGE BUFFER

****************************************************************

* SDRMF4MA                                                     *

* SERIAL DATA RECEIVE MESSAGE, MESSAGE ID = $F4, MODE 10 (A)   *

* TABLE VALUE = ADDRESS                                        *

****************************************************************

10D1      0000      0 SDRMF4MA         0        NEXT MESSAGE ENTRY ADDRESS

10D3        F4    244                $F4        MESSAGE ID

10D4        00      0                  0 FLAG   USE NO BUFFER

10D5        01      1                $01 BYTES  NUMBER OF OUTPUT DATA BYTES

10D6      0119    281              $0119 ADDRS  ADDRESS OF OUTPUT MESSAGE BUFFER

10D8      0119    281              $0119 ADDRS  ADDRESS OF INPUT MESSAGE BUFFER

F31 SHIFT POINT TABLES AND RELATED ITEMS

***************************************************

* F31 TRANSMISSION RELATED CALIBRATION PARAMETERS *

***************************************************

************************

************************

** F31 TCC PARAMETERS **

************************

************************

10DA        00      0 KTCCENBL %00000000 N      TCC CONTINGENCY ENABLE OPTION WORD

*               DESCRIPTION

*            BIT7                 X = NOT ASSIGNED

10DB        00      0 KENGHOT        -40 DEGC   COOLANT TEMP ABOVE WHICH TCC WILL BE ALWAYS ON

(W/5.25 DEGC HYSTERESIS)

10DC        00      0 KTCCETMP       -40 DEGC   IF COAST REL ENBL, TCC FORCED OFF BELOW THIS

(W/5.25 DEGC HYSTERESIS)

10DD        00      0 KTHRVS           0 MPH    IF COAST REL ENBL, COAST REL HI/LO MPH THRESHOLD

(W/2 MPH HYSTERESIS)

*

* NOTE: THE FOLLOWING 4 PARAMETERS MUST BE KEPT TOGETHER IN ORDER

10DE        00      0 KMTHR1LO         0 %      IF COAST REL ENBL, LO MPH TPS REL THRESH, LO HYST

10DF        00      0 KMTHR2LO         0 %      IF COAST REL ENBL, HI MPH TPS REL THRESH, LO HYST

10E0        00      0 KMTHR1HI         0 %      IF COAST REL ENBL, LO MPH TPS REL THRESH, HI HYST

10E1        00      0 KMTHR2HI         0 %      IF COAST REL ENBL, HI MPH TPS REL THRESH, HI HYST

* NOTE: THE PRECEDING 4 PARAMETERS MUST BE KEPT TOGETHER IN ORDER

*

10E2        00      0 KBUSY1           0 SEC    IF BUSY ENBL AND TIME BTW CST REL<THIS ADD KBUSYINC

TO TCC LOCK DELAY TIMER

10E3        00      0 KBUSYINC         0 SEC    IF BUSY ENBL AND TIME BTW CST REL<KBUSY1 ADD THIS

TO TCC LOCK DELAY TIMER

10E4        00      0 KBUSY2           0 SEC    IF BUSY ENBL DEC LOCK DLY BY KBUSYDEC EVERY KBUSY2

LOOPS

10E5        00      0 KBUSYDEC         0 SEC    IF BUSY ENBL DEC LOCK DLY BY THIS EVERY KBUSY2

LOOPS

10E6        00      0 KMINBUSY         0 SEC    IF BUSY ENBL, MIN LOCK DELAY TIME

10E7        00      0 KNDELTHR         0 %      IF NEG DEL THR ENBL, ADD KNDELTIM TO LOCK DLY IF

NEG TPS DELTA > THIS

10E8        00      0 KNDELTIM         0 SEC    IF NEG DEL THR ENBL, ADD THIS TO LOCK DLY IF

NEG TPS DELTA > KNDELTHR

10E9        00      0 KPDELTHR         0 %      IF POS DEL THR ENBL, ADD KPDELTIM TO LOCK DLY IF

POS TPS DELTA > THIS

10EA        00      0 KPDELTIM         0 SEC    IF POS DEL THR ENBL, ADD THIS TO LOCK DLY IF

POS TPS DELTA > KPDELTHR

10EB        00      0 KDSTIME          0 SEC    IF DOWNSHIFT CONTINGENCY ENBL, KEEP TCC RELEASED

AFTER DOWNSHIFT FOR KDSTIME

SECONDS

10EC        00      0 KUSTIME          0 SEC    IF UPSHIFT CONTINGENCY ENBL, KEEP TCC RELEASED

AFTER UPSHIFT FOR KUSTIME

SECONDS

10ED        00      0 KTCCOFF          0 SEC    TCC OFF (RELEASE) TIME BEFORE DOWNSHIFT ALLOWED

*

* NOTE: THE FOLLOWING 4 PARAMETERS MUST BE KEPT TOGETHER IN ORDER

10EE        00      0 KAC3L            0 MPH    IF A/C ON & IN 3RD GEAR, UNLOCK IF MPH <= THIS

10EF        00      0 KAC4L            0 MPH    IF A/C ON & IN 4TH GEAR, UNLOCK IF MPH <= THIS

10F0        00      0 KAC3HYS          0 MPH    HYSTERESIS FOR KAC3L

10F1        00      0 KAC4HYS          0 MPH    HYSTERESIS FOR KAC4L

* NOTE: THE PRECEDING 4 PARAMETERS MUST BE KEPT TOGETHER IN ORDER

*

################################################

#     F1AFACT TABLE                            #

#    F31 THROTTLE COMPENSATION FACTOR VS BARO  #

#   TABLE VALUE = FACTOR*128                   #

################################################

* -------------------------------------------------------------------

10F2        00      0 F1AFACT          0 FACTOR     0    KPA-VAC

10F3        00      0                  0 FACTOR    10

10F4        00      0                  0 FACTOR    20

10F5        00      0                  0 FACTOR    30

10F6        00      0                  0 FACTOR    40

10F7        00      0                  0 FACTOR    50

10F8        00      0                  0 FACTOR    60

10F9        00      0                  0 FACTOR    70

10FA        00      0                  0 FACTOR    80

******************************************

* TCC SHIFT PATTERN APPLY/RELEASE TABLES *

******************************************

*

* NOTE:  THE FOLLOWING 8 TABLES MUST BE KEPT TOGETHER IN ORDER

*

##########################################

# F0APLY3  TABLE (3RD/NORM APPLY SPEED)  #

# FILTMPH V. LOAD LIMIT (NTPSLD)         #

# TABLE VALUE = MPH                      #

* -------------------------------------------------------------------

10FB        00      0 F0APLY3          0 MPH       00      %-TPS

10FC        00      0                  0 MPH    06.25

10FD        00      0                  0 MPH     12.5

10FE        00      0                  0 MPH    18.75

10FF        00      0                  0 MPH       25

1100        00      0                  0 MPH    31.25

1101        00      0                  0 MPH     37.5

1102        00      0                  0 MPH    43.75

1103        00      0                  0 MPH       50

1104        00      0                  0 MPH    56.25

1105        00      0                  0 MPH     62.5

1106        00      0                  0 MPH    68.75

1107        00      0                  0 MPH       75

1108        00      0                  0 MPH    81.25

1109        00      0                  0 MPH     87.5

110A        00      0                  0 MPH    93.75

110B        00      0                  0 MPH      100

########################################

# F0PAPLY3 TABLE (3RD/PERF APPLY SPEED)#

# FILTMPH V. LOAD LIMIT (NTPSLD)       #

# TABLE VALUE = MPH                    #

* -------------------------------------------------------------------

110C        00      0 F0PAPLY3         0 MPH       00      %-TPS

110D        00      0                  0 MPH    06.25

110E        00      0                  0 MPH     12.5

110F        00      0                  0 MPH    18.75

1110        00      0                  0 MPH       25

1111        00      0                  0 MPH    31.25

1112        00      0                  0 MPH     37.5

1113        00      0                  0 MPH    43.75

1114        00      0                  0 MPH       50

1115        00      0                  0 MPH    56.25

1116        00      0                  0 MPH     62.5

1117        00      0                  0 MPH    68.75

1118        00      0                  0 MPH       75

1119        00      0                  0 MPH    81.25

111A        00      0                  0 MPH     87.5

111B        00      0                  0 MPH    93.75

111C        00      0                  0 MPH      100

##########################################

# F0APLY4  TABLE (4TH/NORM APPLY SPEED)  #

# FILTMPH V. LOAD LIMIT (NTPSLD)         #

# TABLE VALUE = MPH                      #

* -------------------------------------------------------------------

111D        00      0 F0APLY4          0 MPH       00      %-TPS

111E        00      0                  0 MPH    06.25

111F        00      0                  0 MPH     12.5

1120        00      0                  0 MPH    18.75

1121        00      0                  0 MPH       25

1122        00      0                  0 MPH    31.25

1123        00      0                  0 MPH     37.5

1124        00      0                  0 MPH    43.75

1125        00      0                  0 MPH       50

1126        00      0                  0 MPH    56.25

1127        00      0                  0 MPH     62.5

1128        00      0                  0 MPH    68.75

1129        00      0                  0 MPH       75

112A        00      0                  0 MPH    81.25

112B        00      0                  0 MPH     87.5

112C        00      0                  0 MPH    93.75

112D        00      0                  0 MPH      100

#########################################

# F0PAPLY4 TABLE (4TH/PERF APPLY SPEED) #

# FILTMPH V. LOAD LIMIT (NTPSLD)        #

# TABLE VALUE = MPH                     #

* -------------------------------------------------------------------

112E        00      0 F0PAPLY4         0 MPH       00      %-TPS

112F        00      0                  0 MPH    06.25

1130        00      0                  0 MPH     12.5

1131        00      0                  0 MPH    18.75

1132        00      0                  0 MPH       25

1133        00      0                  0 MPH    31.25

1134        00      0                  0 MPH     37.5

1135        00      0                  0 MPH    43.75

1136        00      0                  0 MPH       50

1137        00      0                  0 MPH    56.25

1138        00      0                  0 MPH     62.5

1139        00      0                  0 MPH    68.75

113A        00      0                  0 MPH       75

113B        00      0                  0 MPH    81.25

113C        00      0                  0 MPH     87.5

113D        00      0                  0 MPH    93.75

113E        00      0                  0 MPH      100

############################################

# F0CREL3  TABLE  (3RD/NORM RELEASE SPEED) #

# FILTMPH V. LOAD LIMIT (NTPSLD)           #

# TABLE VALUE = MPH                        #

* -------------------------------------------------------------------

113F        00      0 F0CREL3          0 MPH       00      %-TPS

1140        00      0                  0 MPH    06.25

1141        00      0                  0 MPH     12.5

1142        00      0                  0 MPH    18.75

1143        00      0                  0 MPH       25

1144        00      0                  0 MPH    31.25

1145        00      0                  0 MPH     37.5

1146        00      0                  0 MPH    43.75

1147        00      0                  0 MPH       50

1148        00      0                  0 MPH    56.25

1149        00      0                  0 MPH     62.5

114A        00      0                  0 MPH    68.75

114B        00      0                  0 MPH       75

114C        00      0                  0 MPH    81.25

114D        00      0                  0 MPH     87.5

114E        00      0                  0 MPH    93.75

114F        00      0                  0 MPH      100

############################################

# F0PCREL3 TABLE  (3RD/PERF RELEASE SPEED) #

# FILTMPH V. LOAD LIMIT (NTPSLD)           #

# TABLE VALUE = MPH                        #

* -------------------------------------------------------------------

1150        00      0 F0PCREL3         0 MPH       00      %-TPS

1151        00      0                  0 MPH    06.25

1152        00      0                  0 MPH     12.5

1153        00      0                  0 MPH    18.75

1154        00      0                  0 MPH       25

1155        00      0                  0 MPH    31.25

1156        00      0                  0 MPH     37.5

1157        00      0                  0 MPH    43.75

1158        00      0                  0 MPH       50

1159        00      0                  0 MPH    56.25

115A        00      0                  0 MPH     62.5

115B        00      0                  0 MPH    68.75

115C        00      0                  0 MPH       75

115D        00      0                  0 MPH    81.25

115E        00      0                  0 MPH     87.5

115F        00      0                  0 MPH    93.75

1160        00      0                  0 MPH      100

############################################

# F0CREL4  TABLE (4TH/NORM RELEASE SPEED)  #

# FILTMPH V. LOAD LIMIT (NTPSLD)           #

# TABLE VALUE = MPH                        #

* -------------------------------------------------------------------

1161        00      0 F0CREL4          0 MPH       00      %-TPS

1162        00      0                  0 MPH    06.25

1163        00      0                  0 MPH     12.5

1164        00      0                  0 MPH    18.75

1165        00      0                  0 MPH       25

1166        00      0                  0 MPH    31.25

1167        00      0                  0 MPH     37.5

1168        00      0                  0 MPH    43.75

1169        00      0                  0 MPH       50

116A        00      0                  0 MPH    56.25

116B        00      0                  0 MPH     62.5

116C        00      0                  0 MPH    68.75

116D        00      0                  0 MPH       75

116E        00      0                  0 MPH    81.25

116F        00      0                  0 MPH     87.5

1170        00      0                  0 MPH    93.75

1171        00      0                  0 MPH      100

############################################

# F0PCREL4 TABLE (4TH/PERF RELEASE SPEED)  #

# FILTMPH V. LOAD LIMIT (NTPSLD)           #

# TABLE VALUE = MPH                        #

* -------------------------------------------------------------------

1172        00      0 F0PCREL4         0 MPH       00      %-TPS

1173        00      0                  0 MPH    06.25

1174        00      0                  0 MPH     12.5

1175        00      0                  0 MPH    18.75

1176        00      0                  0 MPH       25

1177        00      0                  0 MPH    31.25

1178        00      0                  0 MPH     37.5

1179        00      0                  0 MPH    43.75

117A        00      0                  0 MPH       50

117B        00      0                  0 MPH    56.25

117C        00      0                  0 MPH     62.5

117D        00      0                  0 MPH    68.75

117E        00      0                  0 MPH       75

117F        00      0                  0 MPH    81.25

1180        00      0                  0 MPH     87.5

1181        00      0                  0 MPH    93.75

1182        00      0                  0 MPH      100

*

* NOTE:  THE PRECEDING 8 TABLES MUST BE KEPT TOGETHER IN ORDER

*

**************************

**************************

** F31 SHIFT PARAMETERS **

**************************

**************************

*

* NOTE: THE FOLLOWING 4 PARAMETERS MUST BE KEPT TOGETHER IN ORDER

*

1183        00      0 KSOLDEF  %00000000 STATE  BIT 0 = SOL A, BIT 1 = SOL B, 1ST GEAR

1184        00      0 KSOLDEF2 %00000000 STATE  BIT 0 = SOL A, BIT 1 = SOL B, 2ND GEAR

1185        00      0 KSOLDEF3 %00000000 STATE  BIT 0 = SOL A, BIT 1 = SOL B, 3RD GEAR

1186        00      0 KSOLDEF4 %00000000 STATE  BIT 0 = SOL A, BIT 1 = SOL B, 4TH GEAR

*

* NOTE: THE PRECEDING 4 PARAMETERS MUST BE KEPT TOGETHER IN ORDER

*

**************************************

* LOW RANGE SHIFT PATTERN PARAMETERS *

**************************************

1187        00      0 KLO12            0 MPH    IF MPH > CAL, PRNDL LOW, SHIFT 1 TO 2

1188        00      0 KLO21            0 MPH    IF MPH < CAL, PRNDL LOW, SHIFT 2 TO 1

*************************************

* KICKDOWN SHIFT PATTERN PARAMETERS *

*************************************

1189        00      0 KDETNTLO         0 %TPS   IF IN KICKDOWN, TPS < CAL, DISABLE KICKDOWN

118A        00      0 KDETNTHI         0 %TPS   IF TPS > CAL, ENABLE KICKDOWN MODE

*

* NOTE: THE FOLLOWING 3 PARAMETERS MUST BE KEPT TOGETHER IN ORDER

118B        00      0 KICK21           0 MPH    KICKDOWN 2-1 DOWNSHIFT SPEED

118C        00      0 KICK32           0 MPH    KICKDOWN 3-2 DOWNSHIFT SPEED

118D        00      0 KICK43           0 MPH    KICKDOWN 4-3 DOWNSHIFT SPEED

* NOTE: THE PRECEDING 3 PARAMETERS MUST BE KEPT TOGETHER IN ORDER

*

*

* NOTE: THE FOLLOWING 3 PARAMETERS MUST BE KEPT TOGETHER IN ORDER

118E        00      0 KIC12ENA         0 MPH    KICKDOWN 1-2 SHIFT ENABLE SPEED

118F        00      0 KIC23ENA         0 MPH    KICKDOWN 2-3 SHIFT ENABLE SPEED

1190        00      0 KIC34ENA         0 MPH    KICKDOWN 3-4 SHIFT ENABLE SPEED

* NOTE: THE PRECEDING 3 PARAMETERS MUST BE KEPT TOGETHER IN ORDER

*

*

* NOTE: THE FOLLOWING 3 PARAMETERS MUST BE KEPT TOGETHER IN ORDER

1191      0000      0 KIC12RPM         0 RPM    KICKDOWN 1-2 SHIFT ENGINE RPM SHIFT POINT

1193      0000      0 KIC23RPM         0 RPM    KICKDOWN 2-3 SHIFT ENGINE RPM SHIFT POINT

1195      0000      0 KIC34RPM         0 RPM    KICKDOWN 3-4 SHIFT ENGINE RPM SHIFT POINT

* NOTE: THE PRECEDING 3 PARAMETERS MUST BE KEPT TOGETHER IN ORDER

*

************************************

* PERFORMANCE SHIFT PATTERN TABLES *

************************************

*

* NOTE: THE FOLLOWING 3 TABLES MUST BE KEPT TOGETHER IN ORDER

*

***********************************************

* F1P21 TABLE -- PERFORMANCE 2-1 SHIFT LINE   *

*      VEH SPEED VS. % THROTTLE               *

*      TABLE VALUE = MPH                      *

***********************************************

* -------------------------------------------------------------------

1197        00      0 F1P21            0 MPH       00      %-TPS

1198        00      0                  0 MPH    06.25

1199        00      0                  0 MPH     12.5

119A        00      0                  0 MPH    18.75

119B        00      0                  0 MPH       25

119C        00      0                  0 MPH    31.25

119D        00      0                  0 MPH     37.5

119E        00      0                  0 MPH    43.75

119F        00      0                  0 MPH       50

11A0        00      0                  0 MPH    56.25

11A1        00      0                  0 MPH     62.5

11A2        00      0                  0 MPH    68.75

11A3        00      0                  0 MPH       75

11A4        00      0                  0 MPH    81.25

11A5        00      0                  0 MPH     87.5

11A6        00      0                  0 MPH    93.75

11A7        00      0                  0 MPH      100

***********************************************

* F1P32 TABLE -- PERFORMANCE 3-2 SHIFT LINE   *

*      VEH SPEED VS. % THROTTLE               *

*      TABLE VALUE = MPH                      *

***********************************************

* -------------------------------------------------------------------

11A8        00      0 F1P32            0 MPH       00      %-TPS

11A9        00      0                  0 MPH    06.25

11AA        00      0                  0 MPH     12.5

11AB        00      0                  0 MPH    18.75

11AC        00      0                  0 MPH       25

11AD        00      0                  0 MPH    31.25

11AE        00      0                  0 MPH     37.5

11AF        00      0                  0 MPH    43.75

11B0        00      0                  0 MPH       50

11B1        00      0                  0 MPH    56.25

11B2        00      0                  0 MPH     62.5

11B3        00      0                  0 MPH    68.75

11B4        00      0                  0 MPH       75

11B5        00      0                  0 MPH    81.25

11B6        00      0                  0 MPH     87.5

11B7        00      0                  0 MPH    93.75

11B8        00      0                  0 MPH      100

***********************************************

* F1P43 TABLE -- PERFORMANCE 4-3 SHIFT LINE   *

*      VEH SPEED VS. % THROTTLE               *

*      TABLE VALUE = MPH                      *

***********************************************

* -------------------------------------------------------------------

11B9        00      0 F1P43            0 MPH       00      %-TPS

11BA        00      0                  0 MPH    06.25

11BB        00      0                  0 MPH     12.5

11BC        00      0                  0 MPH    18.75

11BD        00      0                  0 MPH       25

11BE        00      0                  0 MPH    31.25

11BF        00      0                  0 MPH     37.5

11C0        00      0                  0 MPH    43.75

11C1        00      0                  0 MPH       50

11C2        00      0                  0 MPH    56.25

11C3        00      0                  0 MPH     62.5

11C4        00      0                  0 MPH    68.75

11C5        00      0                  0 MPH       75

11C6        00      0                  0 MPH    81.25

11C7        00      0                  0 MPH     87.5

11C8        00      0                  0 MPH    93.75

11C9        00      0                  0 MPH      100

*

* NOTE: THE PRECEDING 3 TABLES MUST BE KEPT TOGETHER IN ORDER

*

*

* NOTE: THE FOLLOWING 3 TABLES MUST BE KEPT TOGETHER IN ORDER

*

***********************************************

* F1P12 TABLE -- PERFORMANCE 1-2 SHIFT LINE   *

*      VEH SPEED VS. % THROTTLE               *

*      TABLE VALUE = MPH                      *

***********************************************

* -------------------------------------------------------------------

11CA        00      0 F1P12            0 MPH       00      %-TPS

11CB        00      0                  0 MPH    06.25

11CC        00      0                  0 MPH     12.5

11CD        00      0                  0 MPH    18.75

11CE        00      0                  0 MPH       25

11CF        00      0                  0 MPH    31.25

11D0        00      0                  0 MPH     37.5

11D1        00      0                  0 MPH    43.75

11D2        00      0                  0 MPH       50

11D3        00      0                  0 MPH    56.25

11D4        00      0                  0 MPH     62.5

11D5        00      0                  0 MPH    68.75

11D6        00      0                  0 MPH       75

11D7        00      0                  0 MPH    81.25

11D8        00      0                  0 MPH     87.5

11D9        00      0                  0 MPH    93.75

11DA        00      0                  0 MPH      100

***********************************************

* F1P23 TABLE -- PERFORMANCE 2-3 SHIFT LINE   *

*      VEH SPEED VS. % THROTTLE               *

*      TABLE VALUE = MPH                      *

***********************************************

* -------------------------------------------------------------------

11DB        00      0 F1P23            0 MPH       00      %-TPS

11DC        00      0                  0 MPH    06.25

11DD        00      0                  0 MPH     12.5

11DE        00      0                  0 MPH    18.75

11DF        00      0                  0 MPH       25

11E0        00      0                  0 MPH    31.25

11E1        00      0                  0 MPH     37.5

11E2        00      0                  0 MPH    43.75

11E3        00      0                  0 MPH       50

11E4        00      0                  0 MPH    56.25

11E5        00      0                  0 MPH     62.5

11E6        00      0                  0 MPH    68.75

11E7        00      0                  0 MPH       75

11E8        00      0                  0 MPH    81.25

11E9        00      0                  0 MPH     87.5

11EA        00      0                  0 MPH    93.75

11EB        00      0                  0 MPH      100

***********************************************

* F1P34 TABLE -- PERFORMANCE 3-4 SHIFT LINE   *

*     VEH SPEED VS. % THROTTLE                *

*     TABLE VALUE = MPH                       *

***********************************************

* -------------------------------------------------------------------

11EC        00      0 F1P34            0 MPH       00      %-TPS

11ED        00      0                  0 MPH    06.25

11EE        00      0                  0 MPH     12.5

11EF        00      0                  0 MPH    18.75

11F0        00      0                  0 MPH       25

11F1        00      0                  0 MPH    31.25

11F2        00      0                  0 MPH     37.5

11F3        00      0                  0 MPH    43.75

11F4        00      0                  0 MPH       50

11F5        00      0                  0 MPH    56.25

11F6        00      0                  0 MPH     62.5

11F7        00      0                  0 MPH    68.75

11F8        00      0                  0 MPH       75

11F9        00      0                  0 MPH    81.25

11FA        00      0                  0 MPH     87.5

11FB        00      0                  0 MPH    93.75

11FC        00      0                  0 MPH      100

*

* NOTE: THE PRECEDING 3 TABLES MUST BE KEPT TOGETHER IN ORDER

*

*******************************

* NORMAL SHIFT PATTERN TABLES *

*******************************

*

* NOTE: THE FOLLOWING 3 TABLES MUST BE KEPT TOGETHER IN ORDER

*

***********************************************

* F1N21 TABLE -- NORMAL 2-1 SHIFT LINE        *

*      VEH SPEED VS. % THROTTLE               *

*      TABLE VALUE = MPH                      *

***********************************************

* -------------------------------------------------------------------

11FD        00      0 F1N21            0 MPH       00      %-TPS

11FE        00      0                  0 MPH    06.25

11FF        00      0                  0 MPH     12.5

1200        00      0                  0 MPH    18.75

1201        00      0                  0 MPH       25

1202        00      0                  0 MPH    31.25

1203        00      0                  0 MPH     37.5

1204        00      0                  0 MPH    43.75

1205        00      0                  0 MPH       50

1206        00      0                  0 MPH    56.25

1207        00      0                  0 MPH     62.5

1208        00      0                  0 MPH    68.75

1209        00      0                  0 MPH       75

120A        00      0                  0 MPH    81.25

120B        00      0                  0 MPH     87.5

120C        00      0                  0 MPH    93.75

120D        00      0                  0 MPH      100

***********************************************

* F1N32 TABLE -- NORMAL 3-2 SHIFT LINE        *

*      VEH SPEED VS. % THROTTLE               *

*      TABLE VALUE = MPH                      *

***********************************************

* -------------------------------------------------------------------

120E        00      0 F1N32            0 MPH       00      %-TPS

120F        00      0                  0 MPH    06.25

1210        00      0                  0 MPH     12.5

1211        00      0                  0 MPH    18.75

1212        00      0                  0 MPH       25

1213        00      0                  0 MPH    31.25

1214        00      0                  0 MPH     37.5

1215        00      0                  0 MPH    43.75

1216        00      0                  0 MPH       50

1217        00      0                  0 MPH    56.25

1218        00      0                  0 MPH     62.5

1219        00      0                  0 MPH    68.75

121A        00      0                  0 MPH       75

121B        00      0                  0 MPH    81.25

121C        00      0                  0 MPH     87.5

121D        00      0                  0 MPH    93.75

121E        00      0                  0 MPH      100

***********************************************

* F1N43 TABLE -- NORMAL 4-3 SHIFT LINE        *

*      VEH SPEED VS. % THROTTLE               *

*      TABLE VALUE = MPH                      *

***********************************************

* -------------------------------------------------------------------

121F        00      0 F1N43            0 MPH       00      %-TPS

1220        00      0                  0 MPH    06.25

1221        00      0                  0 MPH     12.5

1222        00      0                  0 MPH    18.75

1223        00      0                  0 MPH       25

1224        00      0                  0 MPH    31.25

1225        00      0                  0 MPH     37.5

1226        00      0                  0 MPH    43.75

1227        00      0                  0 MPH       50

1228        00      0                  0 MPH    56.25

1229        00      0                  0 MPH     62.5

122A        00      0                  0 MPH    68.75

122B        00      0                  0 MPH       75

122C        00      0                  0 MPH    81.25

122D        00      0                  0 MPH     87.5

122E        00      0                  0 MPH    93.75

122F        00      0                  0 MPH      100

*

* NOTE: THE PRECEDING 3 TABLES MUST BE KEPT TOGETHER IN ORDER

*

*

* NOTE: THE FOLLOWING 3 TABLES MUST BE KEPT TOGETHER IN ORDER

*

***********************************************

* F1N12 TABLE -- NORMAL 1-2 SHIFT LINE        *

*      VEH SPEED VS. % THROTTLE               *

*      TABLE VALUE = MPH                      *

***********************************************

* -------------------------------------------------------------------

1230        00      0 F1N12            0 MPH       00      %-TPS

1231        00      0                  0 MPH    06.25

1232        00      0                  0 MPH     12.5

1233        00      0                  0 MPH    18.75

1234        00      0                  0 MPH       25

1235        00      0                  0 MPH    31.25

1236        00      0                  0 MPH     37.5

1237        00      0                  0 MPH    43.75

1238        00      0                  0 MPH       50

1239        00      0                  0 MPH    56.25

123A        00      0                  0 MPH     62.5

123B        00      0                  0 MPH    68.75

123C        00      0                  0 MPH       75

123D        00      0                  0 MPH    81.25

123E        00      0                  0 MPH     87.5

123F        00      0                  0 MPH    93.75

1240        00      0                  0 MPH      100

***********************************************

* F1N23 TABLE -- NORMAL 2-3 SHIFT LINE        *

*      VEH SPEED VS. % THROTTLE               *

*      TABLE VALUE = MPH                      *

***********************************************

* -------------------------------------------------------------------

1241        00      0 F1N23            0 MPH       00      %-TPS

1242        00      0                  0 MPH    06.25

1243        00      0                  0 MPH     12.5

1244        00      0                  0 MPH    18.75

1245        00      0                  0 MPH       25

1246        00      0                  0 MPH    31.25

1247        00      0                  0 MPH     37.5

1248        00      0                  0 MPH    43.75

1249        00      0                  0 MPH       50

124A        00      0                  0 MPH    56.25

124B        00      0                  0 MPH     62.5

124C        00      0                  0 MPH    68.75

124D        00      0                  0 MPH       75

124E        00      0                  0 MPH    81.25

124F        00      0                  0 MPH     87.5

1250        00      0                  0 MPH    93.75

1251        00      0                  0 MPH      100

***********************************************

* F1N34 TABLE -- NORMAL 3-4 SHIFT LINE        *

*     VEH SPEED VS. % THROTTLE                *

*     TABLE VALUE = MPH                       *

***********************************************

* -------------------------------------------------------------------

1252        00      0 F1N34            0 MPH       00      %-TPS

1253        00      0                  0 MPH    06.25

1254        00      0                  0 MPH     12.5

1255        00      0                  0 MPH    18.75

1256        00      0                  0 MPH       25

1257        00      0                  0 MPH    31.25

1258        00      0                  0 MPH     37.5

1259        00      0                  0 MPH    43.75

125A        00      0                  0 MPH       50

125B        00      0                  0 MPH    56.25

125C        00      0                  0 MPH     62.5

125D        00      0                  0 MPH    68.75

125E        00      0                  0 MPH       75

125F        00      0                  0 MPH    81.25

1260        00      0                  0 MPH     87.5

1261        00      0                  0 MPH    93.75

1262        00      0                  0 MPH      100

*

* NOTE: THE PRECEDING 3 TABLES MUST BE KEPT TOGETHER IN ORDER

*