LM-1 Interface.zip

Enclosed in this file you will find all pertinent information that I used to design my own interface between my 1994 Chev. C1500 350 Pickup PCM (16196395) and my Innovate Motorsports LM-1 Wideband Oxygen Sensor Controller/Data logger.  In all, this cost me about $25 in parts and a couple of hours time.  I also use it on my 1995 S-10 4.3 truck which has the same PCM.  I just programmed the LM-1 to compensate for the difference between the V-6 and V-8 RPM signals.  While this information applies to my application, I don’t see any reason why this cannot be easily adapted to any GM ECM/PCM with little effort.  I use this interface to data log RPM, MPH, TPS, MAP, and CTS from the PCM as well as AFR from the LM-1 but with some imagination, the possibilities are endless as to what can be interfaced.  The interface incorporates 2 frequency to voltage converters for interfacing the RPM and MPH signals from the ESC and VSS reluctors.  I elected to use the reluctor signals because they were centered around 0V, were unprocessed sine wave signals and are universal to all GM computer controlled vehicles.  By varying a single potentiometer in each converter, the interface can be adapted to a 4 cyl. or 6 cyl., engine or to different Vehicle Speed Sensors with different PPM (Pulse Per Mile) outputs as long as you know the PPM value of the VSS, tire circumference (or better yet, rotations per mile which most tire manufacturers can supply) and rear end ratio.  Just do the math to figure the potentiometer values or tweak the potentiometers to match your tach and speedo or your ALDL data logging program.  Refer to the enclosed LM2907 data sheet for component value formulas.  If you can solder and read a schematic, this is a fairly simple project.  I purchased most of the components from www.digikey.com  although except for the IC’s, all of the other parts are common and readily available.  I then assembled them on a Radio Shack project PCB then put it in a Radio Shack project box.  See picture below.  I tapped into all of the signals at the PCM and ran them to a connector which mates to the interface box then ran the output of the interface to the LM-1.  I did not include a parts list.  If I get around to it later, I will add one, until then, you’re on your own for that.  If you need to modify the schematic, go to www.expresspcb.com and download the schematic drawing application. This program is very user friendly.  There is also a very handy PCB designer included in this package with which PCB’s can be designed then ordered online very reasonably.  Feel free to e-mail me if you have any questions at di_dallas@yahoo.com .  Use at your own risk!!!!!!!  I am not responsible if you do something stupid.

Readme.doc – this file.

LM-1 Interface Schematic.pdf – The schematic of the interface in .pdf format.

LM-1 Interface.sch - The schematic of the interface in raw modifiable format.

LM-1_Manual_3.0.pdf – The LM-1 manual.  Use this for reference and LM-1 pinouts.

16196395 PCM Manual.pdf – Handy documentation on my PCM.  Use for reference.

LM2907.pdf – Data sheet for the LM2907 Frequency to Voltage converter IC.  Use for reference.

Sample.bmp – This is a screen shot of the interface at work.  I had CTS turned off.